----------------------------------------------------------------------

----------------------------------------------------------------------

DOCUMENT:

BIOS INTERRUPT 1A SOUND SUPPORT

using DMA and TIMER hardware

Version 1.21

March 23, 1992

Programming by Ryan Hanlon and Nick Skrepetos

Documentation by Ryan Hanlon

Copyright (c) 1992,  Covox, Inc.  All Rights Reserved

----------------------------------------------------------------------

PROGRAM NAME : int1atsr.exe  <-- name stands for interrupt 1A tsr

----------------------------------------------------------------------

PROGRAM DESCRIPTION :

'int1atsr' contains a set of simulated BIOS routines to be used for the

input and output of 8-bit PCM sound data.  With the appropriate values

placed into the AH register the BIOS interrupt 1A can be invoked resulting

in a buffer being queued for recording or playback.  (See the section

titled INTERRUPT 1Ah DESCRIPTION for more information)

There are two types of hardware i/o control in the program int1atsr:

DMA and timer interrupt.  Both types are determined by the options

given on the command line (see section titled PROGRAM USAGE).  The DMA

mode has the superior sound quality of the two and will record at a rate

of 20kHz and playback at a rate of 40kHz. Whereas the timer interrupt

mode will not give good results above 8kHz.  Note that the quality of both

modes depends on what type of computer the application is running on.

Both Tandy mode and PC mode are supported and must also be selected

using the options shown below in the section titled PROGRAM USAGE.  The

main difference between the two modes is the sampling rate value.  For

PC mode the actual kHz value is used whereas in Tandy mode a sampling

rate of 3 (fastest) to 4095 (slowest) must be placed into the appropriate

register. Another difference between Tandy mode and PC mode is the volume

control value when outputing PCM data. The range for Tandy machines is

0 to 7 where 0 is no sound and for PC machines 0 to 15 where 0 is no

sound.  Note that software volume control is only supported on the

Sound Master and will be ignored when used with the Voice Master.

----------------------------------------------------------------------

PROGRAM USAGE :

SYNTAX : int1atsr [OPTIONS]

OPTIONS:

/Pxxxx - The port to use for i/o, xxxx can be one of the following:

VM0, VM1, VM2, VM3 for the internal Voice Master,  or

LPT1, LPT2, LPT3 port for the external VM System II, or

DMA for dma mode using the internal Voice Master.

The default is VM0.

/Mx    - Select 'T' for Tandy sound mode and 'P' for PC sound mode.

The default is 'P'

/Q     - Uninstall int1atsr program.

/S     - Install/Uninstall quietly (no screen output).

The following apply only to Voice Master and Sound Master II.

/Jx    - Select DMA port setting (0-3).  -1 (default) for sequential search.

/Cx    - Select DMA channel (1 or 3).  -1 (default) checks 1 then 3.

/Ix    - Select IRQ (2,3,4,5 or 7).  -1 (default) for sequential search.

The following examples will place the address of the interrupt 1A sound

support routine into the BIOS and the leave the program resident.

--------

EXAMPLE:

Initialize and direct input and output to the external port on

the Voice Master II.

C:\>int1atsr /pLPT1

--------

EXAMPLE:

Prepare for DMA input and output on the internal Voice Master.

C:\>int1atsr /pDMA

--------

EXAMPLE:

Prepare for DMA output on the Voice Master board. IRQ 7 and DMA

channel 3 have been selected.  If there is no Voice Master board

at these settings then an error message will be returned.

C:\>int1atsr /pDMA /i7 /c3

----------------------------------------------------------------------

INTERRUPT 1Ah DESCRIPTION:

The interrupt function is placed in AH by a calling program. The values

and functions of AH are as follows.

INT 1A

AH = 81h : get sound status.

This interrupt call will set the carry flag if i/o

is being performed, or clear the carry flag if no

i/o is being performed. The number of buffers left in

the queue is placed in AH upon return.

On return :

(AL) = 0C4h

(AH) = number of buffers remaining in queue

(CF) = 1  if Busy

(CF) = 0  if Not Busy

AH = 82h : record sound.

This interrupt call is used to queue an empty buffer

for PCM recording of sound.  If the maximum number of

buffers have been queued then the carry flag is set.

The carry flag is cleared if the queue request was

successful.

On entry:

(ES:BX) = Address of buffer

(CX)    = Length of buffer at ES:BX

(DX)    = sampling rate

On return :

(AH) = 00

(CF) = 1  if Busy

(CF) = 0  if Not Busy

AH = 83h : output sound.

This interrupt call is used to queue a buffer of

PCM data for playback.  If the maximum number of

buffers have been queued then the carry flag is set.

The carry flag is cleared if the queue request was

successful.

On entry:

(ES:BX) = Address of buffer

(CX)    = Length of buffer at ES:BX

(DX)    = sampling rate

(AL)    = Volume

On return :

(AH) = 00

(CF) = 1  if Busy

(CF) = 0  if Not Busy

AH = 84h : stop sound input and output.

This interrupt call will stop all sound i/o.

Note on returned value of carry flag (CF):

The queue that is used for i/o has a maximum of MAX_BUFFER_REQUESTS (155)

entries. After that number has been reached, a call for i/o will set the

carry flag.  The calling program should monitor the carry flag and loop

until a spot is freed in the queue and the carry flag is cleared. (See

programming example below)

----------------------------------------------------------------------

INTERRUPT 1A AH=83h PROGRAMMING EXAMPLE :

// This example mixes MicroSoft C 6.0 code with in-line assembly.

// Before this program was run the following command was given at

// the DOS prompt.

//

//           C:\> int1atsr /pDMAV

//

// Now after 'int1atsr' has been installed the following code segment

// will send 200 buffers to be queued for output. Since the maximum

// number of buffers that can be queued is 155, the carry flag needs

// to be checked after each request to queue a buffer.  If the carry

// flag is returned as 1 the in-line assembly jumps back to QUEUE_REQUEST

// and the same request is sent again.  When the carry flag is returned

// as 0 the request was sucessful.

// Call int 1A to perform i/o

//

for(i=0; i<200; i++)

{

// Allocate memory for buffer[i] to be used for output of PCM data.

//

buffer[i] = ( char far * )_fmalloc( BUFFER_SIZE );

// Read PCM data into buffer[i] from a file. Notice that the buffer

// size is 2K but could have been as large as FFFFh.

//

_dos_read(file_handle, (void far *)buffer[i], 2048, &bytes_read);

// Put segment and offset of buffer into variables that can be

// recognized by in-line assembly.

//

buffer_OFF = FP_OFF(buffer[i]);

buffer_SEG = FP_SEG(buffer[i]);

_asm  QUEUE_REQUEST:

_asm     mov   bx, buffer_SEG      ; LOAD es:bx WITH SEG OFFSET OF BUFFER.

_asm     mov   es, bx              ;

_asm     mov   bx, buffer_OFF      ;

_asm     mov   cx, 2048            ; SET BUFFER LENGTH

_asm     mov   dx, 8000             ; SET SAMPLE RATE AT 8000kHz.

_asm     mov   al, 4               ; SET VOLUME.

_asm     mov   ah, 83h             ; PERFORM INTERRUPT FOR I/O

_asm     int   01Ah                ;

_asm     jc    QUEUE_REQUEST       ; IF CF = 1 THEN REQUEST WAS UNSUCESSFUL

}