BLUE

Portable Synth Engine

External Structure Description

Ver: 0.005

Written by Howard M. Brown, chief systems architect

Oct. 20,1994

Edited by Fred Lapitino, product marketing manager

Sept. 4, 1996

Changes between 0.003 and 0.004

- Sample pointer go from 3 bytes to longs

- Amp adjust for the samples became signed

- Added a flags byte to the samples header for the dethumper

- added a count of global sources to the program structure

- added exclusive zones to the LSP and moved the crossfade control to the amp CCD

- changed the controller identifier allocation

- more clearly defined the controller list

- changed the rate calculation for LFOs

- moved the start value in the envelope descriptor to it’s proper place.

- added curve selection bit in the env configuration

- add two bit descriptions for the layer flags

- Hz detune became a controller

Changes from 0.004 to 0.005

- added new SYSEX messages

- fixed maxTranspose to be a word not a long as in previous versions.

Introduction

This document is meant as an overview of the external data structures of the Blue    portable synth engine that will control an MA-1 device as featured in the MASS[ies] reference design. One of the primary goals of this engine is to be portable to many different platforms.    Given this goal, this engine concentrates on low memory requirements at the expense of computation. The design decisions stemming from this will be pointed out as the design is discussed.

This document assumes some familiarity with the concepts of synthesis and sample playback technology. Knowledge of the Kurzweil K2000 will be helpful.

A note: All of the data structures are defined and will be created using x86 endian and alignment rules.

Overview

Synthesis

The basic architecture of the synthesizer in the MA-1 chip goes back to the earliest days of sound synthesis. Basically, an MA-1 voice is made up of a sample based interpolating oscillator, followed by a multipurpose frequency based processor and an amplifier. This synthesizer presents several controllable parameters.

(1) Pointers into sample memory that allow the determination of the sample wave shape. The sample pointers are fixed and chosen based on the note that is to be played.

(2) Pitch

(3) 2 modulatable parameters for controlling the programmable filter engine.

(4) Amplitude.

These values are controlled by modulators. There are several types of modulators. There are basic modulators such as velocity, note number, and MIDI controllers. There are also more complicated modulators that evolve over time on their own or in response to other controllers. This type of modulator includes such blocks as low frequency oscillator, envelopes, function blocks, etc.

Blue Programs

Instruments in a wavetable synthesizer can be composed of one or more layers. A layer is a description of the samples, modulators and their interconnection. Each layer may or may not sound based on several criteria including note number and note velocity.

Thus there are two basic parts to a Blue program: the global program parameters and the per-layer information.

The program is basically a set of flags that are associated with the entire instrument and a set of global modulators that all layers have access to. These modulators are useful if the modulation of all of the voices of a particular instrument need to be synchronized. For example, global vibrato.

The layers are composed of several parts:

(1) Layer selection criteria    - Information used to determine if this layer should be started.

(2) Sample selection criteria

(3) A list of layer specific modulators.

(4) A list of the parameter control descriptors and how each parameter (pitch, filter cutoff, and amplitude) are effected by the modulators.

Major Data Structures

There are several major data structures that are central to the engine. These are described below.

Objects

The three primary objects in the Blue system (samples, keymaps, and programs) are kept as objects in a database in memory. Each object has a header described below:

/* blueobj.h - contains the blue object data structure. */

#define KEYMAPTYPE 	1

#define SBLKTYPE		2

#define PROGTYPE		3

#define TABLETYPES	4

typedef struct blueObjStr

{

ubyte objType; /* the type of the object */

ubyte pad;

uword objId;      /* the number of the object */

uword objSize;	/* the size of the object not including this header */

char    objName[NAMELEN]; /* Name length character name      */

char    data[];

}BlueObj;

Sample Headers

The sample headers describe the individual samples. The information in the sample headers are as follows:

Start of Sample - The MA-1 address that contains the start of the sample.

End of Sample - The MA-1 address of the end of the sample.

Loop of sample - The designated loop point within the sample.

maximum transpose - This parameter is the maximum number of semitone and bents (binary cents) of upward transposition. A bent is 1/256 of a semitone.    This is centered around note number 64. Thus if a sound can reach note number 76, this value will be 12.

The proper pitch is calculated by multiplying the maximum playback rate (0xffff) time the proper semitone transposition and bent transposition. Thus to play the above sound back at note number 72 we calculate (72 - 64) to center the pitch about note number 64 then 8 -12 for a -4 semitone transposition from the top playback rate.

Amplitude adjustment -    an adjustment value to the sample amplitude. Used to smooth variations in recording level.    (1 byte) units are dB.

Sample Flags - A group of sample related flags. These are defined as :

#define DETHUMP 0x01 - used to remove the first 2ms of a sample if it thumps at the very beginning

#define LINEAR    0x02    - indicates that the sample is linear as opposed to K8 compressed.

#define S816 0x04 - if this bit is set then the sample is 16 bit. Otherwise it is 8 bit.

Natural Envelope - a series of envelope segments used to modify the amplitude back to the original amplitude envelope. (variable). There are an indeterminate number of attack segments which are {sample count, envelope slope} pairs. These are followed by a single decay segment and a single release segment, each of which are a slope with a zero sample count field. Unless disabled, the natural envelope always effects the amplitude.

C structure =

struct natEnvelElem

{

word slope ;

word sampleCount;

}

#define DETHUMP 0x01

#define LINEAR    0x02

#define S816        0x04

struct SampleHdr

{

ulong startOfSample;

ulong    endOfSample;

ulong loopOfSample;

byte ampAdj;

ubyte sampleFlags;

uword maxTranspose;

natEnvElem natEnv[];

}

Keymaps

Keymaps are a list of key - to - sample header translations. The keymap itself is composed of a keymap range (defined by low-key, High key) and an array of keymap elements.    The keymap entries are described by a high key(1 byte)- sample number pair (2 bytes). These are in order from lowest to highest key. Each range is assumed to be contiguous from the high key of the previous sample. The software is responsible for searching the list for the first entry whose high key is >= to the to be played. A sample number of 0xFFFF indicates a null Sample. There is also a further amp adjust in units of half dB.

C struct

struct KeymapElem

{

ubyte highKey;

byte ampAdj;

uword sampleNumber;

}

struct Keymap

{

ubyte lowKey;

ubyte highKey;

KeymapElem keymapEntry[];

}

Vardata format

Several structures in the Blue programs are described using the Vardata format. This format consists of a 2 byte header at the beginning of each structure that uses the Vardata format. The first byte is a combination type and ID byte. The upper 4 bits of the first byte indicate the type of the object. The lower four bits are currently useful for the modulators since they define the destination of the output of the modulator.    The lower 4 bits also defines the algorithm for the filter CCD’s.

The ID is unique within the layer or within the globals section and is used to describe the destination of the output of a modulator. The current list of Vardata objects are:

typedef struct

{

ubyte typeAndId;

ubyte len;

} descHdr;

#define LFODESC		1

#define ENVDESC		2

#define CTRLMAPDESC	3  // not currently used.

#define FUNDESC		4

#define MFILTCCD                  12    /* filter CCDs for Mabel - 15 may be recycled later */

#define PITCHCCD                  13

#define AMPCCD                        14

#define FILTCCD                        15  // not currently used. Used only by Homer systems.

Programs

Programs are composed of a global parameters section, a list of global modulators, and a list of layers. The global parameter section consists of just the global flags, a pad byte, and a count of global sources. This structure also includes a count of layers and a list of layer offsets which indicate the offsets from the program data pointer for each layer. Lastly, the layer offsets are followed by a list of global sources in the Vardata format.    It is intended that this then be followed by all of the variable length layers.

C structure -

struct Prog

{

byte globalFlags;

byte    pad;

byte layerCount;

byte globalCount;

word layerOffsets[];

}

Layers

Layers description blocks (just called layers from here on out) describe the software based control sources used to control the voices.    Layers consist of three parts: Layer Specific Parameters (LSP), Modulators, and Coefficient Control Descriptors (CCD). The LSP is always present and is the first thing in the layer descriptor. This is followed by a variable number of Modulators, and finally the layer is completed by 3 Coefficient Control Descriptors, themselves of variable length, one each for pitch, filter cutoff, and amplitude.

Layer Specific Parameters

The Layer specific parameters include:

Activation parameters that determine if the Layer should be made active. There are two sets: Key range and Velocity Range. If the Key number and Attack velocity all fit within the ranges described by these parameters, then a voice is started using these parameters.

General Flags - These are general flags for the Layer, these could include: Ignore release, xfade sense,    etc.

Root Selection parameters: These include a Timbre Shift amount and keymap number. This allows the engine to determine which root to use.

Layer Delay - Indicates the amount of time from when the note on is processed until the voice is started.

Pan information - This byte indicates how this voice should be panned. There are 16 base positions. It also indicates Algorithmic placement such as autoPan, reverse AutoPan, + MIDI.

Output Indicator - This byte indicates the physical output pair for this voice.

Modulator Count - This byte indicates the number of modulators that are contained in this layer.

exclZone    - This byte indicates what exclusive zone this layer belongs to. If the exclZone is non-zero then all other currently running notes on that channel with the same exclusive zone are stopped before this note is started. This is primarily useful for percussion. For instance a closed high hat should stop an open high hat from sounding.

#define IGNORENOTEOFF 0x08 - if set, ignore note offs and allow to decay naturally.

#define XFADESENSE 0x10 - if set, then interpret the Xfade control inverted (0x7f00 - controller val)

#define NATENVOFF 0x20 - if set, then don’t process the natural envelope

#define NOISEON 0x40 - use the white noise generator rather than the sample.

C struct

struct lspStruct

{

uword layerFlags;

ubyte highKey;

ubyte lowKey;

ubyte highVel;

ubyte lowVel;

byte timbreShift;

ubyte pan;

ubyte exclZone;

ubyte pad;

uword keyMapNum;

uword layerDelay;

ubyte outputPair;

ubyte modCount;

descHdr varData[];

}LSP;

Modulators

The number of modulators per voice is variable. There are currently 3 types of modulators including: LFO, Envelope, and FUNs. All modulators produce an output that varies from -1.0 to +1.0 (in 1.15 format).

All modulators are evaluated at each update in the order they appear in the program . Thus, any editor that is produced needs to order the modulator evaluation such that no modulator will have an undefined input in the first step. The editor will also need to flag any layers that have circular descriptions.

One of the most prevalent structures in the modulators ( and CCD’s) is the controller descriptor. The controller descriptor is an unsigned byte that has two basic types: the numbers between 128-255 point to one of the control source outputs. The numbers between 0 and 127 are constant values that are returned as numbers between -1.0 and 1.0 . Thus there is a limit to the number of control sources available of 127. These are further broken into fixed raw channel sources (such as mod wheel), global modulator outputs, local raw sources (e.g. keyNum) and local modulators.    The list of these has yet to be defined. These are represented by CTRL typedef below.

C struct

typedef CTRL byte;

The Output of a modulator will go into a modulator output table (either the global modulator table or the note modulator table) . The index into this table is the associated ID from the vardata header.

Thus a controller source identifier points to the table and the index within the table. In order to patch a modulator output to another input, the modulator output ID and the controller input ID must correspond.

Currently the value of the CTRL byte    assignments are:

global controllers (e.g. Modwheel) 		0x80 - 0x9f

global switchs (e.g. sustain)			0xa0 - 0xaf

global modulators(e.g. a global LFO)		0xb0 - 0xbf

global sources (e.g. channel count)		0xc0 - 0xcf

local controlls (e.g. note Velocity)		0xd0 - 0xef

local modulators (e.g. local LFO)		0xf0 - 0xff

The specific controllers that are available are:

0x80 - ox8f contain the first 16 MIDI continuous controllers except that 0x80 is actually the value of the pitchbend. 0x90 through 0x9f are available for expansion.

0xa0 through 0xa3 contain the MIDI switches (Sustain, Sostenuto, ???, Soft). 0xa4 through oxaf are available for expansion.

0xb0 through 0xbf contain the output values of the 16 global modulators.

0xc0 through 0xc6 contain global raw controllers which are:

0xc0 - Mono Press", - MIDI mono pressure.

0xc1 - Current Key" - The number of the most recent key start event << 8

0xc2    - Current Vel", - The velocity of the most recent key start event << 8

0xc3 - Chan Count", - The number of voices this channel has allocated expressed

as a fraction between 0 and 1 of all available voices.

0xc4 - Chan State" - A switch that indicates whether this channel has any allocated

voices.

0xc5 - Hi Key", - The highest key of a currently active note on this channel << 8

0xc6 - Lo Key", - The lowest key of a currently active note on this channel << 8

0xc7 through 0xcf are available for expansion

0xd0 through 0xda are the local raw sources which include:

0xd0 - Note Num - the note number of the current note << 8;

0xd1 - Att Vel - The attack velocity of the of the current note << 8

0xd2 - Att Bi-Vel A attack velocity of the current note << 9 - 0x8000

0xd3 - Note State

0xd4 - Key State A switch indicating whether the key is down or not

0xd5 - Loop State - indicates that the note is in it’s loop.

0xd6 - Playback rate - indicates the fraction of max playback rate this note is

playing

0xd7 - Inv Att Vel - The attack velocity of this note expressed as (0x7fff - (vel <<

8));

0xd8 - Release Vel - the release velocity of this note. << 8

0xd9 - Att State -a controller that goes to max value for the initial update period.

0xda - Rel State - a controller indicates that the note has gone into release

0xdb through 0xef are available for expansion

0xf0 through 0xff contain the output values of the 16 local modulators.

LFO

The low frequency oscillator is a multi waveform oscillator whose rate varies between about .02 Hz and 20 Hz. The shapes available are: Sine, Triangle, Square, and Ramp. Also a Phase control is provided to indicate the starting phase of the oscillator. Further the Rate is described by a minimum rate, a maximum rate, and a rate control. This rate control points to one of the control outputs in the Note Control Block (see below).

C struct

{

descHdr lfoHdr;

ubyte shapeAndPhase; // this byte contains the shape index and the phase index

ubyte minRate;

ubyte maxRate;

CTRL rateCtrl;

}

The shape and phase byte are divided into 2 parts. The top 4 bits define the starting phase of the LFO divided into 16 separate phases. The shape ID is contained in the next 4 bits. There are currently only 4 defined. see prog.h

#define SINELFO		1

#define SQUARELFO	2

#define RAMPLFO		3

#define TRILFO		4

The frequency for the LFO is found by multiplying 25Hz (max rate) by    (rate byte)/256. The rate byte can be viewed as a 8 bit fractional value of 25 Hz. For instance a rate of 2 translates to a LFO rate of    0.2 Hz.

Envelopes

The Blue Envelopes are multiSegment envelope generators whose segment rates can be controlled by external controller values. The number of segments is variable per envelope with a maximum of 8 attack segments and 4 decay and 4 release segments each. each of these envelope sections can have it’s rate controlled by a general controller. There are always 3 segments in each envelope, one attack , one decay and one release.

The first entry in the envelope structure is the envelope configuration. This first byte is divided as follows:

bit 0,1,2 - indicates the number of attack segments 1 - 7

bit 3,4      - indicates the number of decay segments 1 - 3

bit 5,6      - indicates the number of release segments 1 -3

bit 7 - if set, make attack segments linear in amplitude not log as is natural

The second byte of the envelope is the Start Value. This provides the initial value for the envelope. Following the start value is the trig control. The envelope does not start until the trig control goes above .5.    The    fourth, fifth, and sixth bytes are control indexes that point to the selected rate controllers, one for attack, one for decay, and one for release. Currently, the rate controllers are only evaluated at segment change time.

After these fixed values come the segment descriptors. Each segment descriptor is a { time enumeration, destination value }pair.

C structs

struct envElem

{

byte destValue;

ubyte envTimeIndex;

};

#define AMPENVCURVE 0x80

struct envStruct

{

descHdr envHdr;

ubyte envConfig;

CTRL trigCtrl;

CTRL attRateCtrl;

CTRL decayRateCtrl;

CTRL relRateCtrl;

byte startValue;

envElem envSegs[];

}

These envelopes go through 5 basic states. When the note is struck, the envelope starts in the attack state where the note processes sequentially through the attack segments. If the note is held long enough to pass through all of the attack segments, the note goes to the decay state and processes all of the decay segments. Once all of the decay segments are processed, the envelope enters the decay-done state where the envelope maintains its final decayed value.    When the note is released, the envelope goes immediately to the release state and processes the release segments until complete upon which time it enters the release-done state which holds the final value of the envelope.

FUN

FUN is short for function generator. Blue FUNs are an extension of the FUNs found in the K2000 engine. Simply put, FUNs take inputs, perform a defined function on those inputs, and produce an output. The most fundamental difference between K2000 FUNs and Blue FUNs are that Blue FUNs are 3 valued FUNs (as opposed to 2 for the K2000). This allows such FUNs as RANGE where the output is 1 if    input 1 is between input 2 and input 3. Also (and this is important for envelope control) SCALE where the input 1 is scaled so that it’s entire range is between input 2 and input 3. Other FUNs that are inherited from the K2000 will include: Diode, Add (3 input), etc. This list is still being fleshed out. For a current list see prog.h

C struct

struct FUNstruct

{

descHdr funDesc;

ubyte funSelector;

CTRL funInput1;

CTRL funInput2;

CTRL funInput3;

}

#define SUM			0	/* add all three inputs */

#define SUMSCALE          	1	/* (a+b)*c */

#define DIFFSCALE		2 	 /* (a-b)*c */

#define SCALE3ADD		3	/*    a + (b*c) */

#define SCALE3DIFF		4 	 /*    a - (b*c) */

#define MULT3		5	//    a*(b << 8)*c

/* #define SUB3		6	//    a-b-c    ??    NOTIMPLEMENTED */

#define INRANGE		7	/* if(a >= b) && (a <= c) out = 1 else 0 */

#define SCALE		8	/* scale a to between b and c */

#define DIODE		9	/* if (a-b) > 0 out = a-b else 0 */

#define TRACK		10 	/* out = 1 if b > 0 else 0 */

#define SAMPLEHOLD	11 	/* out = a when b is less than HALFCTRL else hold 						last value*/

#define LOWPASS		12	/* lowpass the controller - makes change more 						gradual than input */

#define HIGHPASS		13	/* remove slowmoving components */

#define QUANTIZE		14	/* quantize a into b steps */

#define ABSVALDIODE	15 	/* take the |a| and diode with value */

#define COSINE		16	/* take the cosine of the controller value    */

#define AND			17	/* treat the inputs as switches and AND values

(a & b & c) */

#define OR			18	/* treat the inputs as switches and OR values

(a | b | c) */

#define LIMIT			19 	/* limit A to be between b and C */

#define MAXFUN		20 	/* maximum of input A and Input B */

#define MINFUN		21 	/* minimum of input A and Input B */

Coefficient Control Descriptors

Coefficient control descriptors are the data structures that connect the modulators to the various signal processing functions in the MA-1. In addition, each CCD has particular functions that are unique to the type of signal processing resource that the CCD is controlling. The current list of CCD’s are Pitch, Filter Coefficient and Amplitude. These are described below.

The general structure of CCD’s is function specific fixed information followed by a variable number of modulator inputs. The modulator inputs are the same as the more complex form of the K2000 modulators, namely, each modulator input has an associated Min depth, a Max depth, and a depth control.    To calculate the contribution from each modulator input the following calculation is performed.

output = inputCtrl * (minDepth - ((maxDepth - minDepth) * depthCtrl));

modulator C Structure

struct ModStruct

{

CTRL inputCtrl;

byte minDepth;

byte maxDepth;

CTRL depthCtrl;

}

Thus the contribution from each of the modulator inputs is summed and used to calculate the next value of the coefficient that is being modulated.

Each CCD is a varData structure and as such starts with a descHdr structure. Also each CCD has a Mabel header in it    A Mabel header contains specific configuration information for the MA-1. The information in each CCD is specific to that CCD.

/* describes a Mabel filter object */

typedef struct

{

ubyte filtCfg;        /* the configuration byte for the filter */

ubyte mfiltPad;      /* pad to a 16 bit limit */

}MabelHdr;

Pitch CCD

The Pitch CCD has the following form:

Coarse Pitch (in semitones) - A coarse adjust to the pitch (+/- 128 semitones)

Fine Pitch ( in cents) - (+/- 50 cents)

Hz detune (in hertz) -    A controller that expresses the Hz detune as a fraction of 6Hz

Following these parameters are a list of modulators of the common form.

typedef struct

{

descHdr    pitchHdr;

MabelHdr mpitchHdr;

uword keymapNum;

byte coarsePitch;

byte finePitch;

CTRL hzDetune;

ubyte pitchModCount;

ModDesc pitchMods[];

}MPitchCCD;

Pitch is further modified by a hardwired source of pitch bend and a hardwired source of tune. The depth for this is controlled by the pitch bend depth control registered parameter for that channel and the tune RPN. Default is 2 semitones.

Since the required range and accuracy of the depths is greater than one byte can convey,    depths in the modulators are found through a table rather than direct.

None of the fields of the MabelHdr are used in the Pitch CCD.

Filter CCD

Each MA-1 layer has 2 filter CCDs, one for each coefficient available for an MA-1 Voice. Each coefficient type descriptor can be different. Currently, there are 2 types - The null configuration and the other one.

As discussed above the lower 4 bits of the typeAndId byte of    the varData for MA-1 define the algorithm. The currently available algorithm types are:

/* the mabel filter slot types */

#define MNULLFILT        0 - Does nothing

#define M1PLPFILT            1 - Implements a 1 pole lowpass filter

#define M1PHPFILT          2    - Implements a 1 pole high pass filter

#define M2PLPFILT1        3 - implements the first half of a 2 pole lowpass w/ resonance

#define M2PLPFILT2        4 - implements the second half of 2Pole filter.

#define MSAWPLUS          5 - adds a sawtooth of a specified frequency to sampled osc.

#define MSINEPLUS          6 - adds a sine wave of specified frequency to sampled osc.

#define MDIST8                          7 - controls a non-linear distortion function to osc.

The data structure for the Null configuration is simply the vardata header and the appropriate Mabel header entries.

typedef struct

{

descHdr nullHdr;

MabelHdr mnullHdr;

}MNullCCD;

The second data structure is the same for all processing algorithms. However, the units are specific to the algorithm type.

typedef struct

{

descHdr filtHdr;

MabelHdr mfiltHdr;

ubyte coarseFilter;

byte    fineFilter;

ubyte filtPad;

byte velTrack;

byte keyTrack;

ubyte filtModCount;

ModDesc filtMods[];

}MFilt1PLPCCD;

The following algorithms interpret this data structure as a frequency following the standard pitch curve.

M1PLPFILT, M1PHPFILT ,M2PLPFILT1, MSAWPLUS, and MSINEPLUS.

Coarse Filter Adjust (in semi tones)

Fine Filter Adjust (frequency in Bents)

Velocity Tracking - indicates the max amount of filter to add based on vel. in 1/2 semitones

Key Tracking - in 4 bent increments. This is mono-directional. That is it just adds to the filter cutoff.

For the M2PLPFILT2, the fields are interpreted as follows:

Coarse Filter Adjust - Resonance in dB (between -12 and + 18)

Fine Filter Adjust - resonance in 256th of a dB.

Key and Velocity tracking - Resonance in DB/per key. from -2.0 to +2.0 per unit 	velocity or key.

For MDIST8, the fields are interpreted as arbitrarily.

Coarse Filter - adjust “distortion “ between 1.0 and -1.0.

Fine Filter - ignored.

Key and velocity tracking are viewed as fractions per key with each having a range 	of +2 to -2.

The resulting value is clipped to +1 to -1.

The filtCfg byte of the two filter CCD’s currently need to be configured as follows:

For slot 0 the filtCfg byte must be equal to the filter slot type.

For    slot 1 the filtCfg byte must be equal to the filter slot type ANDed with    0x10;

Amplitude CCD

The Amplitude CCD    has the following parts:

Coarse Adjust - This is a coarse adjust amount for amplitude, measured in dB

Keytrack - Amplitude per key

Velocity Tracking - Maximum amplitude added for maximum velocity

Xfade Control - This indicates the controller for controlling the cross fade. Cross fade is a particular amplitude modulator that allows two sources to be mixed    in a varying amounts but maintains a constant output

C structure

/* describes how the amplitude is to be modulated */

typedef struct

{

descHdr ampHdr;

MabelHdr mampHdr;

byte coarseAdjust;

byte velTrack;

byte keyTrack;

CTRL xfadeCtrl;

ubyte pad;

ubyte ampModCount;

ModDesc ampMods[];

}MAmpCCD;

The amplitude is further modulated by a hardwired link to MIDI    Volume and Expression.

The filtCfg byte for the amplitude CCD has the following format.

4                3    2            1      0

+------+--------+--------+

|        1          |    HD1 |    HD0    |

+------+--------+--------+

The two HD or headroom fields    allow the user to add attenuation at various places in the processing chain. In particular, the HD0 allows the signal to be attenuated between the osc and the first filter slot. The HD1 allows the signal to be attenuated between the first and second slots. The values are as follows:

00 - no attenuation

01 -    -3 dB

10 -  -6 dB

11 -  -9 dB

Example Program

An example program would look like this:

struct Program

{

Prog 		progData; 	// the layer offset array would have two entries

LFO 		globalLfo;

FUN		globalFun;

LSP		layer1;

ENV		lyr1Env;

LFO		lyr1Lfo;

PitchCCD	lyr1Pitch;

FiltCCD	lyr1Filt;

AmpCCD	lyr1Amp;

LSP		layer2;

FUN		lyr2FUNA;

FUN		lyr2FUNB;

PitchCCD	lyr2Pitch;

FiltCCD	lyr2Filt;

AmpCCD	lyr2Amp;

}

Data Bases

The software maintains several databases that are strictly a translation between an object number and pointer. These are implemented as arrays of pointers whose index is the requested object address. Currently there are 3 of these: A sample header index, a keymap index, and a program index.

External Database Interface

Blue provides interfaces into it’s internal    object database. There will be 5 SYSEX messages that are used to send these messages across a MIDI interface to the synth.

getObjectHdr - This interface method gets the contents of the BlueObj hdr and returns it.

getObject    - This interface method returns the contents of an entire object.

Add object - This routine takes a fully formed object and copies it into the appropriate database and sets up the appropriate index entry.

Delete object - This routine removes the requested object from the database, closes the hole in the database memory and rebuilds the indices.

Non Registered Parameter Numbers

The non registered Parameter Numbers (NRPN) allow the user to control aspects of the Blue engine that are more amenable to controller control than SYSEX. This section discusses the NRPN Implementation.

Hi Byte      Low Byte

NRPN          0                                    1 - load the reverb from the database associated with the data entry 			value.

NRPN          0                                    2 - load the chorus effect from the database associated with the data 			entry value

NRPN          0                                    3 - indicate which translation table to use for controller 7

NRPN          0                                    4 - indicate

SYSEX Description

There are 5 SYSEX messages defined corresponding to the 5 editing interface routines. The messages are divided into requests and responses, as indicated by the request type field.

The message format is:

0xf0 	/* sysex start */

0x07 	/* Kurzweil Manufacturer ID */

0x21	/* Blue Engine Identifier */

0xXX    /* interface method request type - 1 = request; 2 = reply */

0xXX    /* interface method request id    - See table below*/

.	/* data in 7+1 sysex format */

0xf7

Data format

Since MIDI in it’s infinite wisdom saw fit to not allow 8 bit bytes, we have to go though the usual gyrations to "7bitize" the data stream. Blue does it the usual way, the byte stream is viewed as a continuos string of bits that are chunked into 7 bit bytes for the purposes of the transmission. They will be returned to a more sane format for processing.

Request Ids

GetObjectHdr Id = 1

request structure is the Above objectReq with an object header as data The only valid parts of the request object header are the Type and ID.; reply is the BlueObj header in 7 bit format;

GetObject    ID = 2

Request structure is objectReq above;    with the object header as data.    The only valid parts    of the object ID for this request are the object type and Id. The rest are ignored. The reply is the entire object. in 7bit format.

Add Object ID = 3

The data is the entire object in 7+1 format; No reply

Delete object ID = 4

Uses the objectReq header above with a data field of an object header. The only two fields used are the Id and Type. No reply

Add Sample ID = 5

The message adds a sample to the downloadable sample RAM. The format for this message is:

messageHeader msg;

uword sampleId;

ulong sampleLength;

ubyte samples[];

This sample is “linked” to the sample header of the same ID.

Add Sample Alias = 6

This makes another Sample ID point to a previously loaded sample. This allows multiple sample headers to point to the same sample memory.

messageHeader msg;

uword BaseSampleId;

uword LinkedSampleId;

Delete Sample ID= 7

Delete the associated sample. If the sample is a link, just the link is deleted. If the sample is a base sample but is linked by other samples, the delete is not performed until all linked samples are deleted.

Report Synth Status ID=8

TBA.

Diag Status ID = 9

This command has no data. The reply is a single 32 bit word. Faults are bit mapped across the 32 bit word, thus a fully functional board would report a status of 0. The bits have the following meaning.

bit 0 - CPU fault

bit 1 - EPROM Fault

bit 2 - Processor RAM fault

bit 3    - Mabel fault

bit 4 - Sound ROM fault

bit 5 - Sound RAM fault.

bit 6 - Bit 31 reserved and always 0

These diags are not to be thought of as exhaustive, but as the results of a cursory examination of the indicated subsystems.

Version request ID = 8

This request has no data. The reply is three 16 bit words indicating the firmware version ,the sound ROM, and the database version.

messageHeader msg;

uword    FirmwareVer;

uword    DatabaseVer;

uword    SoundROMVer;

Memory RAM Config request ID = 9

Request - No Data

Reply - The reply consists of two 32 bit words. The first word is the amount of Database RAM available at Boot. (e.g. No added objects). The second word is the size of the Sound RAM.

messageHeader msg;

ulong    DatabaseRAMConfig;

ulong    SoundRAMCOnfig;

Available Memory Config ID = 10

Request - Not Data

Reply - The data structure of the reply is the same as Config request except that the values returned are the current free sizes.