Vector to Two-Line Elements

(VEC2TLE) Software

User Manual

Version 9331

26 July 1993

Kenneth J. Ernandes

16 Freshman Lane

Stony Brook, NY 11790

Kenneth J. Ernandes

16 Freshman Lane

Stony Brook, NY 11790

Vector to Two-Line Elements (VEC2TLE) Software User Manual

Copyright 1993 by Kenneth J. Ernandes.  This publication may be

reproduced and distributed as long as the final product is a

complete, unaltered reproduction of the original document.  All

other rights are reserved.

IBM is a registered trademark of International Business Machines

Corporation.  Microsoft, Windows, and MS-DOS are registered

trademarks of Microsoft Corporation.

TABLE OF CONTENTS

1.0  INTRODUCTION  ..........................................   1

1.1  Vector Types Supported ............................   1

1.2  Keplerian Orbital Elements ........................   3

1.3  Orbit Propagation Models ..........................   4

2.0  INSTALLATION ...........................................   6

3.0  USER INTERFACE DESCRIPTION .............................   8

3.1  Menu Activation ...................................   8

3.2  Menu Item Selection ...............................   9

3.3  Dialog Boxes ......................................   9

3.4  Windows ...........................................  12

4.0  OPERATING INSTRUCTIONS .................................  13

4.1  Startup ...........................................  13

4.2  File ..............................................  13

4.3  Settings ..........................................  16

4.4  Compute TLE .......................................  17

4.5  Re-Epoch TLE at Asc Node ..........................  19

4.6  Impulsive Delta-V .................................  19

4.7  Program Termination ...............................  20

5.0  REFERENCES .............................................  21

6.0  ACKNOWLEDGEMENTS .......................................  22

7.0  LICENSE STATEMENT ......................................  23

7.1 Registration .......................................  23

7.2 Distribution .......................................  23

7.3 Limited Warranty ...................................  24

7.4 Governing Law and General Provisions ...............  25

APPENDIX A  --  HOT KEY SUMMARY .............................  26

APPENDIX B  --  SEQUENTIAL DAY OF THE YEAR ..................  27

APPENDIX C  --  ACRONYMS AND ABBREVIATIONS ..................  29

APPENDIX D  --  VECTOR FILE FORMATS .........................  31

APPENDIX E  --  SOURCES OF STATE VECTORS ....................  34

APPENDIX F  --  SOURCES OF TLEs .............................  36

APPENDIX G  --  GENERAL INFORMATION .........................  39

APPENDIX H  --  ABOUT THE AUTHOR ............................  42

Page 1

1.0       INTRODUCTION

The Vector to Two-line Elements (VEC2TLE) software provides the

user with the capability to convert position/velocity/time state

vectors of a variety of formats to National Aeronautics and Space

Administration (NASA) Compatible Two-Line Element (TLE) sets.

These element sets contain orbit descriptions compatible for

propagation with the Simplified General Perturbations (SGP) and

SGP Version 4 (SGP4) orbit theories developed for use by the

North American Aerospace Defense Command (NORAD) and United

States Space Command (USSPACECOM).

TLEs have become an increasingly-popular source of orbital data.

This is due mainly to the fact that this format of data is

available for nearly all Earth-orbiting satellites.  As a result,

the popular satellite tracking software packages such as STSPLUS

and TRAKSAT, use TLEs as their primary sources of input data

along with the SGP4 orbit propagator.

TLE data is available for most Earth-orbiting satellites from

several sources (see Appendix F).  VEC2TLE is not intended to

replace these as primary sources of TLE data.  Rather, its

primary purpose is for circumstances in which this data may not

be available in a timely fashion.  A prime example is the Space

Transportation System (STS) where position and velocity state

vectors may be the only orbital information available in near

real time.  In the past, users of the tracking software had

little choice but to wait (hours) until the TLE data was

available following thruster firings that changed the Shuttle's

orbit.  It is this  situation that resulted in the creation of

VEC2TLE.  The TLEs computed by VEC2TLE are fully compatible with

SGP or SGP4 as true and accurate renditions of the orbit

specified by the input state vectors.

Sections 1.1, 1.2, and 1.3 provide an overview of the technical

features of VEC2TLE.  Section 1.1 discusses the various vector

types that may be input to compute TLEs.  Section 1.2 is a brief

description of Keplerian orbital elements.  Section 1.3 is an

overview of the SGP and SGP4 propagation models.  It should be

noted, however, that VEC2TLE takes care of all of the intricate

details, requiring the user only to know the specific type of

input vector.  Thus, this information is just provided for the

benefit of the interested reader.

1.1       Vector Types Supported

VEC2TLE supports a variety of vector types in the creation of

TLEs.  The two basic vector types are Earth-Centered Inertial

(ECI) and Earth-Fixed Greenwich (EFG).  Both ECI and EFG

represent right-handed orthogonal cartesian reference frames with

the origin at the Earth's center of mass.

Page 2

1.1.1     Earth-Centered Inertial

The ECI coordinate frame is an inertial frame in the sense that

its orientation remains practically fixed in space.  The primary

axis (X-axis) is in the equatorial plane and points in the

direction of the vernal equinox.  The Y-axis is also in the

equatorial plane, and points 90 degrees to the east of the vernal

equinox.  The Z-axis is perpendicular to the equatorial plane,

and points through the north pole.

The ECI coordinate frame, in truth, is not inertial.  The vernal

equinox is not a fixed direction in space due to precession

imparted on the Earth's rotational axis by the moon, sun, and

other planets in the solar system.  Precession is a long-term

effect.  The earth's equator/poles also do not maintain a fixed

obliquity to the ecliptic.  The moon and sun also impart a short-

term periodic wobble on the Earth's rotational axis called

nutation.  The result is a change in the orientation of the X-,

Y-, and Z-axes over time.  Reference 1 has a detailed discussion

of precession and nutation.

VEC2TLE provides two standard systems for specifying the

orientation of the ECI coordinate axes:  1) True Equator, True

Equinox of Date and 2) Mean Equator, Mean Equinox of 1950 (M50).

The former, as its name implies, orients the X-, Y-, and Z-axes

to the actual directions of the vernal equinox and polar axes at

the applicable time of the vector.  The latter orients the axes

to the respective directions of the equinox and pole at the

beginning of the Besselian Year 1950.

1.1.2     Earth-Fixed Greenwich (EFG)

The EFG coordinate frame rotates with the Earth.  The primary

axis (E-axis) is in the equatorial plane and is directed through

the Greenwich Prime Meridian (GPM).  The F-axis is also in the

equatorial plane and points 90 degrees to the east of the GPM.

The G-axis is perpendicular to the true equator, and points

through the north pole.  The EFG system is also known as the True

of Day Rotating (TDR).

The EFG coordinate frame is particularly useful for launch

planning in that the components of the position/velocity state

vectors remain fixed.  The applicable times for each state vector

are generally referenced as an offset from launch time.  Based on

the predicted or actual launch time, TLEs for all orbital

segments can be computed with VEC2TLE by entering the vectors and

adding the time offsets to the launch time.

Page 3

1.1.3     Vector Units

VEC2TLE can compute TLEs from state vectors in any one of the

following sets of units:

- kilometers (km), kilometers per second (km/s)

- nautical miles (nm), nautical miles per second (nm/s)

- feet (ft), feet per second (ft/s)

1.2       Keplerian Orbital Elements

Keplerian orbital elements provide a geometric description of the

size, shape, orientation, and in-plane phasing of a satellite

orbit.  The following is a brief description of the Keplerian

elements.  For further details, see references 2 through 5.

The size and shape of an elliptical orbit are described by the

semimajor axis and eccentricity respectively.  These parameters

also specify the perigee (closest distance to Earth) and apogee

(farthest distance from Earth) of the orbit.

The semimajor axis is half the longest distance across the

orbital ellipse.  The size of the semimajor axis determines the

orbital period, which is the time it takes for the satellite to

make one complete revolution or orbit.  The period is sometimes

used instead of semimajor axis.  The mean motion, which is

essentially the reciprocal of the period, is also commonly used

in place of semimajor axis (as is the case with TLEs).

The eccentricity describes how much the orbit deviates from

circular.  A perfectly circular orbit has an eccentricity of

zero.  The orbit becomes more elongated as eccentricity

increases.  Eccentricity must remain less than one for an orbit

to be elliptical.  A value greater than or equal to one describes

an escape trajectory (parabolic/hyperbolic).

The orientation of the orbital plane is described by the

inclination and the right ascension of the ascending node (RAAN).

The inclination describes the tilt of the orbital plane with

respect to the equatorial plane.  The inclination specifies the

northern most and southern most latitudes over which the

satellite will directly pass.  An inclination of zero degrees is

an equatorial orbit; an inclination of 90 degrees is a polar

orbit.  Inclinations greater than 90 degrees describe orbits

moving against the rotational direction of the Earth

(retrograde).

Page 4

The ascending node of an orbit is its south to north equatorial

crossing point.  In astronomical terms, right ascension is the

angle measured eastward from the vernal equinox to the point of

interest.  Thus the RAAN is the angle measured eastward from the

vernal equinox to the satellite's south to north equatorial

crossing point.

The final orientation angle is the argument of perigee.  This is

the angle measured from the ascending node to the perigee point,

in the direction of satellite motion.

The in-plane phasing is specified by the true anomaly.  This is

the angle measured from the perigee point to the position of the

satellite at the epoch time of the element set.  Since satellites

in elliptical orbits do not travel at a constant speed, true

anomaly has a counterpart called mean anomaly, which describes

the angle from perigee to the satellite were the satellite in a

circular orbit.  There is a standard mathematical conversion

between mean and true anomaly, based on the eccentricity of the

orbit.  Mean anomaly is often used in place of true anomaly (as

is the case with TLEs).

1.3       Orbit Propagation Models

This section provides a general discussion of the SGP and SGP4

orbit propagation models.  A detailed discussion of the

mathematics involved is beyond the scope of this manual.

Elements representing the physical Keplerian parameters are

referred to as "osculating" elements.  The data in TLE format

represent fictitious "mean" values for the various orbital

elements.  This was done to accommodate both speed and

convenience in the process of computing positions and velocities

of the satellites.  Thus the introduction of osculating elements

for propagation in SGP or SGP4 would result in erroneous

predictions.

The procedure to compute position and velocity state vectors for

any desired time using SGP or SGP4 is well understood and

detailed in reference 6.  The basic perturbations that cause a

satellite's path to deviate from an ideal Keplerian orbit result

from 1) the non-spherical mass distribution of the Earth and 2)

atmospheric drag.  SGP and SGP4 apply these perturbational

effects to orbits by the technique know as variation of

parameters, where the parameters being changed are the orbital

elements.  If these effects were ignored and the orbit were

propagated using 2-body (i.e., Keplerian) orbit theory, the error

in the predictions would be apparent within the span of 2-3

hours.

Page 5

1.3.1     Earth Gravitational Modeling

SGP uses a 3rd order geopotential model to describe the mass

distribution of the Earth.  This includes the equatorial bulge

(2nd order) and the greater amount of mass in the southern

hemisphere (3rd order) that describes the characteristic "pear"

shape of the Earth.  SGP4 uses a 4th order geopotential model

which includes an additional mass deviation that is smaller than

the second and third order deviations.

The geopotential deviations from an "ideal" spherical mass

distribution result in predictable changes to the orbit.  The

primary gravitational perturbational effects are on the orbital

plane and the orientation of the orbit's apogee-perigee (or

apsidal) line.  The primary effects are "secular" in nature as

they represent constant drift rates for the ascending node and

the apsidal line as a function of time.  The constant drift rates

are a function of the semimajor axis, eccentricity, and

inclination of the orbit.  The secondary effects are periodic in

nature and consist of both long- and short-term effects.  The

long term periodics are superposed on the secular effects.  The

short-term periodics, in turn, are superposed on the long-term

periodic effects.

1.3.2     Atmospheric Drag Modeling

Both SGP and SGP4 use static methods to model the effects of

atmospheric drag on satellite orbits.  SGP assumes a quadratic

variation of the mean motion as a function of time.  The

quadratic coefficients are one sixth the second derivative of

mean motion with respect to time (nddot/6) for time squared and

one half the derivative of mean motion with respect to time

(ndot/2) for time.  SGP4 models the density of the Earth's upper

atmosphere using the fourth power of the orbital altitude.  SGP4

applies drag effects to the orbit using a pseudo ballistic

coefficient (B*), normalized to the orbital altitude and current

atmospheric density profile.  For both SGP and SGP4, the drag

coefficients (n dot/2, nddot/6 or B*) are usually empirically-fit

(based on long-term behavior) in the orbit determination process.

Page 6

2.0       INSTALLATION

The VEC2TLE software is designed to operate on an International

Business Machines (IBM) compatible Personal Computer (PC) using

the Microsoft Disk Operating System (MS-DOS) version 3.2 or

higher.  The computer must be equipped with a hard disk and a

video display compatible with one of the following standard PC

formats: CGA, Hercules, EGA, MCGA, or VGA.  The VEC2TLE software

automatically detects the display adaptor format and displays at

the highest possible resolution.

VEC2TLE works on IBM compatible PCs in the MS-DOS environment.  A

math coprocessor is recommended for 8088 and 80287-based systems.

The typical computation times for a 12 Mhz 80287-based system is

about 3 seconds with a coprocessor versus about 15 seconds

without a coprocessor.  A 40 Mhz 80387-based system without a

coprocessor computes a TLE in approximately the same time as a 12

Mhz 80287-based system with a coprocessor.  Using a 50 Mhz 80486-

based system, there is no perceptible computation delay.

The software must first be installed on either a floppy diskette

or the computer's fixed (i.e., hard) disk.  You may install

VEC2TLE automatically by placing the distribution diskette in a

floppy drive and setting that floppy as the default drive.

Automatic installation is accomplished by initiating the

INSTALL.BAT batch file by typing:

INSTALL [drive:][path]

The default drive and path for INSTALL.BAT is C:\VEC2TLE.  (You

may specify an alternate destination drive and path as shown

above in the optional command line parameters.)

The installation procedure may also be accomplished manually.

Let us assume an installation from floppy disk A: to hard disk C:

Place the software floppy diskette into drive A:

Set drive C: as the default drive:

C: [ENTER]

Create a directory for VEC2TLE:

MD C:\VEC2TLE [ENTER]

Set the VEC2TLE directory as the default path:

CD \VEC2TLE [ENTER]

Page 7

Copy all files to the default directory:

COPY A:[path]*.* [ENTER]

Note that the manual installation procedure may be modified to

accommodate a different destination drive letter, a different

installation drive letter, or a different VEC2TLE directory name

as desired.  This is accomplished by making the appropriate

changes to the above procedure as follows:

a) DESTINATION DRIVE - substitute the actual destination

drive letter (followed by a colon) for C:

b) INSTALLATION DRIVE - substitute the actual installation

drive letter (followed by a colon) for A:

c) PATH - substitute the actual directory names

(i.e., paths) for \VEC2TLE (or the installation path)

If a mouse is to be used with the software, it must have its

driver installed prior to initiating the VEC2TLE software.  For

further information, consult the documentation provided with your

mouse.

When posted on a bulletin board system, VEC2TLE and its

associated files are compressed into a .ZIP format file.  The

files name is indicative of the version of VEC2TLE contained

within.  The file naming convention is "V2Lyyww.ZIP".  The V2L is

the prefix indicating that this is a compressed version of

VEC2TLE.  the yy digits indicate the year in which the current

version was created.  The ww digits indicate the week (1-54) in

which the current version was created.  This is consistent with

the version number indicated on the copyright notice display.

For example, the current version of VEC2TLE (9331) would be found

in a file named V2L9331.ZIP when posted on a Bulletin Board

Service (BBS).  This file naming convention, provides a

convenient mechanism by which users can determine whether or not

they have the most current version of the software.

The files contained in the compressed (i.e., .ZIP) file are as

follows:

INSTALL.BAT         Installation Batch File

VEC2TLE.EXE         Executable program file

VEC2TLE.CFG         Program configuration file

VEC2TLE.DOC         Software user manual

VEC2TLE.ICO         Icon suitable for use with Microsoft Windows

REGISTER.FRM        Registration Form

STS-57.VIF          Sample State Vector from STS-57 in VEC2TLE-

readable format

VEC2TLE.NEW         Changes since the previous release

Page 8

3.0       USER INTERFACE DESCRIPTION

The user interface is designed to be both intuitive and

functionally efficient.

The intuitive characteristics of the interface include its use of

many de facto standards for PC graphic user interfaces (GUIs) as

well as liberal use of labels and comments.  Access to different

screens flow in a consistent and logical (e.g. top-down or

left-to-right) fashion.  The "look and feel" of the GUI is

intended to follow the standards and conventions for Microsoft

Windows applications.  The intent is to minimize the need for the

new user to refer to this user manual.

The author also recognizes that experienced users prefer to have

"short cuts" rather than be required to perform a tedious series

of actions for an operation.  Thus the functionally efficient

aspect of the user interface is that many operations can be

accessed by single action "hot keys."

Other features of this interface also include the ability to

access an operation by several different methods and the ability

to gracefully exit from any operation as necessary.

Much of the user interface may be driven with a

Microsoft-compatible mouse.  The left button of the mouse

activates the mouse cursor at its present position on the screen.

3.1       Menu Activation

Individual menus are identified by their title labels on the menu

bar at the top of the screen.  Each menu title label has a

highlighted activation character.  Menus may be activated by

either the mouse or the keyboard.  When a menu is activated, its

functional selections appear below the menu title in the work

screen area.

To activate a menu by the mouse, place the mouse cursor over the

menu title (on the top menu bar) and press the left mouse button.

To activate a menu by the keyboard, simultaneously press the

[ALT] key plus the highlighted character's key in the desired

menu title (e.g. press ALT-F to activate the "File" menu).

Page 9

3.2       Menu Item Selection

All menu items are identified by a descriptive label.

Additionally, some menu items also have a "hot key" (listed to

the right of the label).  The menu item labels have a highlighted

selection character.  One menu item is always highlighted in

reverse video as the default for selection whenever a menu is

active.  A menu item may be selected by any one of the following

methods:

Place the mouse cursor on the label of the menu item and press

the left mouse button.  OR:

Move the highlighting selection (i.e. reverse video) bar with the

"up" or "down" arrow keys to the desired menu item and press the

[ENTER] key.  OR:

Press the key corresponding to the menu item label's highlighted

selection character.  OR:

Press the "hot key" associated with that menu item.  (Note that

the corresponding menu need not be active to select a menu item

by the "hot key.")

3.3       Dialog Boxes

The dialog boxes (or data input screens) are designed to

facilitate data entry, provide a degree of error checking, and in

many cases prevent the entry of erroneous data.  All dialog boxes

consist of a rectangular window with a double line border and a

drop shadow.  On the top bar of the window is a short title

descriptive of the data to be input in the dialog box.

3.3.1     Data Items

Data items in any dialog box may be selected by either the

keyboard or the mouse.  The [TAB] key is the selection device to

be used for the keyboard.  Pressing the [TAB] key selects the

data items in sequential order.  Note that when the last item is

selected, pressing the [TAB] key will select the first data item

on the list.  When using the mouse, place the mouse cursor on the

desired data item and press the left mouse button.

3.3.1.1   Text Items

Text items may be entered or edited via the keyboard.  Text items

are identified by a rectangular input box.  Some text fields have

a label that is highlighted when the text box is selected.  When

working with non-blank text items, it is important to know

whether you are in the "entry" or the "edit" mode.

Page 10

The text item is in the "entry" mode if the data presently in the

input line is highlighted (i.e., in reverse video).  (The cursor

must also appear as a blinking underscore character.)  In this

mode, the old value of the text input line will be deleted and

replaced with the characters typed from the keyboard as soon as

the first character key is pressed.  The "entry" mode may be

activated by moving the cursor to the input line by the Tab key.

You may also enter this mode by placing the mouse cursor on the

input box and pressing the left mouse button twice in rapid

succession (i.e., double-clicking).

The text item is in the "edit" mode if the data in the input box

is not highlighted and/or the cursor appears as a blinking

rectangle.  In this mode, the data may be edited in a fashion

similar to that for most word processor software (i.e.,

backspace, delete, right arrow, left arrow, etc.).  This mode may

be entered by changing the shape of the mouse cursor with the

[INSERT] key, or by a single press of the left mouse button while

the cursor is on the input box.  It should be noted at this point

that the shape of the cursor indicates whether new text will be

inserted in front of the cursor or will overwrite the existing

text.  If the cursor is the blinking underscore, new text will be

inserted in front of the cursor; if the cursor is the blinking

rectangle, then new text will overwrite the existing text.  It

should further be noted that the size of all text fields are

limited and when the maximum size is reached, no further

characters may be inserted.

3.3.1.2   Buttons

Most dialog boxes have two buttons: OK and Cancel.  These buttons

control the manner in which processing is to be performed using

the data from the input screen.  When a dialog box is in a

sequence, it may have a Prev button which allows the user to

return to the previous dialog box.  These buttons may be accessed

by either the keyboard or the mouse.

3.3.1.2.1 OK Button  Activating the OK button instructs the

software to process the input screen data.  The OK button is

activated by either of the following methods:

Move the cursor (with the [TAB] key) to any item except the

Cancel button and the press the [ENTER] key.  OR:

Place the mouse cursor on the OK button and press the left mouse

button.

3.3.1.2.2 Cancel Button  Pressing the Cancel button instructs the

software to abandon the current operation.  This has the effect

of ignoring any data input.  The Cancel button is activated by

any one of the following methods:

Page 11

Move the cursor (with the [TAB] key) to the Cancel button and

press the [ENTER] key.  OR:

Press the keyboard [ESCAPE] key.  OR:

Place the mouse cursor on the Cancel button and press the left

mouse button.  OR:

Closing the input screen window (ALT-F3 on the keyboard or

selecting the button in the upper left corner of the window with

the mouse).

3.3.1.2.3 Prev Button  Pressing the Prev button instructs the

software to return to the previous dialog box in a sequence.  The

Prev button is activated by either of the following methods:

Move the cursor (with the [TAB] key) to the Prev button and press

the [ENTER] key.  OR:

Place the mouse cursor over the Prev button and press the left

mouse button.

3.3.1.3   Radio Buttons

Radio buttons are helpful data input items in that they promote

speed of data entry and do not allow the input of "illegal"

values.  A radio button is used to select one and only one item

from a set (cluster) of possible values.  A radio button always

has two or more items and one (and only one) of these items is

always selected.  Radio buttons are delimited by parenthesis ().

The selected item in a radio button cluster may be changed by the

up and down arrow keys or by placing the mouse cursor on the item

to be selected and pressing the left mouse button.  Note that

radio buttons generally have labels with highlighted characters.

In such cases, an item may also be selected by pressing the key

corresponding to the highlighted character.

3.3.2     Other Features

The dialog boxes are constructed to appear at the center of the

work screen area. Should the dialog box be blocking needed data

in another open window, it is possible to move the input window

out of the way with the mouse.  This is accomplished by placing

the mouse cursor on the top bar then holding the left mouse

button down while moving the window by mouse action.

It is not possible to have two dialog boxes open simultaneously.

Page 12

3.4       Windows

Windows provide a means of visual information input and output

for VEC2TLE.  The dialog boxes and error messages fall into the

general category of windows.  As such, the information generally

applies to these windows, but more importantly applies to the

data output windows displaying the computed TLEs.

All windows can be moved around the desk top by activating the

cursor on the top bar and dragging the window.  In the upper left

corner of the window is a "close" button that will destroy the

window.  The currently-active window may also be closed by an

Alt-F3 hot key action as well as by selecting the Close item from

the Window menu.

The TLE output windows stack on top of each other in the desk

top.  A new window assumes its position in the desk top stack

with its order being immediately after the currently-active

window.  The user may move through the stack of windows in the

desk top to view previously-computed data.  This is accomplished

by the Pg Up (previous window) or Pg Dn (next window) hot key

actions or by selecting the corresponding entries under the

Window menu.

An output window may also be shrunk down to a only a small

fraction of the desk top size by dragging the lower right corner

with the mouse.  However, this action is not generally

recommended since the window will obtain the data from the last

TLE computation when the window is restored to fit the full size

of the desk top.  Restoring and/or centering the active window to

the full desk top may be accomplished by the activating the mouse

cursor on the arrow in the upper right corner of the window, by

the Alt-Z hot key action, or selecting the Zoom option from the

Window menu.

Page 13

4.0       OPERATING INSTRUCTIONS

This section provides the operating instructions for the VEC2TLE

software including any applicable background information.  It

should be noted that detailed explanation of each input option is

not provided as it would be redundant.  Rather, the operator

should have a functional understanding of the user interface

before attempting any serious operation of the software.

4.1       Startup

The software is initialized by setting the Disk Operating System

(DOS) default drive and directory (i.e., path) to that where

software resides.  For the purposes of these instructions, it is

assumed that the software resides on hard drive C: in directory

\VEC2TLE.  (If this assumption is incorrect, the information

below needs to be modified accordingly.)

Change to the default drive:

C: [ENTER]

Change the directory to the program-resident directory:

CD \VEC2TLE [ENTER]

Start the program:

VEC2TLE [ENTER]

Note that VEC2TLE may also be initiated from the Microsoft

Windows operating environment.

4.2       File

This section provides a description of the functions listed in

the menu under the File heading.  As the name implies, these

functions are concerned with actions associated with input and

output files.  Files are selected using a list selector in a

dialog box.  The desired path and file name may be entered

directly or a "wildcard" may be entered altering the names

appearing in the scrollable list box.  Alternatively, the desired

file may be selected as a pick from the list box.  The last n

wildcard entries are accessible from the down arrow to the right

of the wildcard entry area.

4.2.1     Vector Input File

The Vector Input File function allows the user to identify a text

file from which state vectors may be read for conversion to TLEs.

The default wildcard is *.VIF for the list box when selecting a

Page 14

vector input file.  If entering the path and file name directly,

the identified file must exist.  If the file does not exist, you

must respond to a dialog box with the following message:

Error  Could not open file: [filename]

The identified file must contain a header line with a code

specifying the format and attributes of the state vectors.  The

code must apply to all state vectors in the file.  The code must

be a contiguous block of from one (1) to four (4) digits, must be

in the first 80 characters of the first non-blank line of the

file, and must be the first digit(s) appearing on that line

following an equal (i.e., '=') sign.  (Note that the first non-

blank line must be in the first ten [10] lines of the file.)  If

the code is less than four digits, leading zeros are implied.

The meaning of each digit is as follows:

Thousands      Inertial Reference Frame

0              True Equator and Equinox of Date

1              Mean Equator and Equinox of 1950

Hundreds       Vector Type

0              ECI

1              EFG

Tens           Vector Units

0              km, km/s

1              ft, ft/s

2              nm, nm/s

Ones           Vector Format

4              Single line per datum

5              Space delimited, two line

6              Comma delimited, one line

7              Single line per datum

If the code read in the file header is not within the bounds

listed above, VEC2TLE will present you with a dialog box with the

following message:

Error  Unknown vector format in file: [filename]

An explanation of the vector file formats appears in Appendix D.

Appendix D also includes examples of the file headers as well as

each of the three basic formats.  There is no default vector

input file.

Page 15

4.2.2     TLE Input File

The TLE Input File function allows the user to identify a text

file from which TLE may be read for delta-V application.  The

default wildcard is *.TLE for the list box when selecting a

vector input file.  If entering the path and file name directly,

the identified file must exist.  If the file does not exist, you

must respond to a dialog box with the following message:

Error  Could not open file: [filename]

The user is discouraged from using the same file as both the TLE

Input and Output files.  In such instances, an EOF character will

be appended after the last existing TLE.  While the new TLEs will

be appended to the end of the file, they will follow the EOF

character.  This makes the data after the EOF inaccessible to

VEC2TLE to read as well as to most text editor and word processor

software.  One know exception is the Microsoft Windows Notepad.

In the notepad, the EOF character appears as a solid rectangle.

If the EOF is edited out while in the Notepad, the data following

it becomes accessible to VEC2TLE and text editors and

wordprocessors.

4.2.3     TLE Output File

The TLE Output File allows the user to specify the file to which

TLEs are to be written.  The default wildcard is *.TLE for the

list box when selecting a TLE output file.  If the specified

output file name is entered directly and the output file does not

exist, it will be created; if the file does exist, any TLEs

written will be appended to the existing file.  If the file does

not exist and cannot be created, VEC2TLE will present you with a

dialog box with the following message:

Error  Could not open file: [filename]

When a TLE output file is selected, its path and name is saved to

the configuration file (VEC2TLE.CFG).  This path and name is used

to identify the default output TLE file, which is opened at

program startup.  If the no configuration file exists, or the

default configuration file is used, OUTPUT.TLE is the default TLE

output file.

4.2.4     Output TLEs

The Output TLEs menu item is a toggle control that allows the

user to enable/disable the writing of the computed TLEs to the

output file.  This setting may be changed any number of times

during the program execution.  The default setting is to enable

output to the [default] output TLE file.

Page 16

4.3       Settings

The selections made under the Settings heading apply to the

manual entry of position and velocity state vectors.  These

settings are format descriptors for the state vectors and their

associated times.

4.3.1     Defaults

The defaults are the settings saved in the program configuration

file (VEC2TLE.CFG).  If this file is not found at program

startup, a standard configuration file will be created.  Any

changes made under this menu item will be saved to the

configuration file once the OK button is activated or ENTER is

selected.  All configuration items are controlled by radio

buttons.  The following is a list of the configuration items:

INERTIAL REFERENCE  This setting applies only to ECI vector types

and selects the applicable inertial reference frame.  The choices

are:

-- True equator, true equinox of date

-- Mean equator, mean equinox of 1950

TYPE  Two cartesian vector types are supported:

-- Earth-Centered Inertial (ECI)

-- Earth-Fixed Greenwich (EFG) / True of Day Rotating (TDR)

VECTOR UNITS  Three sets of vector dimensional units are

supported:

-- km, km/s

-- nm, nm/s

-- ft, ft/s

TIME FORMAT  Two date and time formats are supported:

-- YYYYDDDHHMMSSS.SSS  (Year, Day, Hour, Min, Sec, decimal

Sec)

-- YYYYDDD.DDDDDDDDDD  (Year, Day, decimal Day)

Note that VEC2TLE allows for implicit two-digit years to be

entered from 1950 to 2018 (inclusively).  If the first two digits

of the year are 00-18, the years 2000-2018 are assumed; if the

first two digits are from 50-99, the years 1950-1999 are assumed.

EPOCH CONTROL  The epoch time of the computed TLE may be one of

two times:

-- The time of the State Vector

-- The time of the Ascending Node (south to north equatorial

crossing) prior to the State Vector epoch

Page 17

Note that epoching the TLE to the time of the State Vector is

slightly more accurate and is thus the preferred method when

there is a choice.

4.3.2     Inertial Reference

This menu item allows the user to override the default setting

for inertial reference without changing the value in the

configuration file.

4.3.3     Vector Units

This menu item allows the user to override the default setting

for vector units without changing the value in the configuration

file.

4.3.4     Epoch Control

This menu item allows the user to override the default setting

for the TLE epoch control without changing the value in the

configuration file.

4.4       Compute TLE

The menu items under the Compute TLE heading is where VEC2TLE

performs its computational function.  The Vectors may be input

manually by or read from vector input files.  For both cases, a

reasonability check is run to ensure that the input state vector

represents a closed trajectory (circular, elliptical), that the

apogee of the orbit is above the surface of the Earth, and the

epoch time of the vector is not zero.  If any if these conditions

is violated, a error message is displayed and the user is given

the option to edit the state vector or cancel the operation.

It should be noted, for all TLE computation methods in this

section, that if either the ndot/2 or the Bstar drag parameter is

zero  and the orbit theory is SGP4, the zero drag parameter will

be automatically computed from the non-zero drag parameter and

elements from the resulting TLE.

4.4.1     Input State Vector

The Input State Vector menu item allows the manual input of

position and velocity state vectors and associated data in order

to compute TLEs.  The following is a summary of the fields in the

input dialog box:

Page 18

Satellite Name or ID          - 12 characters (max) free-form

Catalog Number                - 5 digit integer USSPACECOM ID

number

International Designator      - International Astronomical Union

(IAU) designator*

Position Vector Components    - 16 Character floating point or

scientific notation

Velocity Vector Components    - 16 Character floating point or

scientific notation

Vector Type                   - Radio Button

Time Format                   - Radio Button

Orbit Theory                  - Radio Button

ndot/2                        - 10 Character floating point or

scientific notation

nddot/6                       - 10 Character floating point or

scientific notation

Bstar                         - 10 Character floating point or

scientific notation

Element Set #                 - 3 digit integer

Rev #                         - 4 digit integer revolution number

Numeric fields are truncated when the first digit in the input

character string is not a legal part of an integer or floating

point number as applicable.  All characters after the first

illegal character are ignored.  Blank fields or those beginning

with an illegal character are interpreted as zero.

*The IAU designator has three components: year, sequential launch

of the year, and the sequential piece designator for that launch.

The year is invariably a two-digit number; the sequential launch

is invariably a three-digit number.  The piece designator is one

to three characters in length, with legal characters being A-Z,

excluding the letters "I" and "O".  The IAU designator is

generally right-justified with a space between the three

components if the sequential piece designator is a single letter.

The latest data manually entered in the vector input dialog box

under the Input State Vector function is automatically saved in a

test file called OUTPUT.VIF.  This file is in the labeled data,

one item per line vector file format (see appendix D for

details).  This feature saves the user unnecessary typing if an

error in the inputs is detected after the program has been

exited.  This feature also provides a convenient method by which

state vector data may be easily shared with other VEC2TLE users.

4.4.2     Read Vector File

The Read Vector File menu reads the next sequential vector from

the input vector file and otherwise processes the data with the

same procedure for Input State Vector menu item.  If no vector

Page 19

file has been selected, the following error message will be

displayed:

Error  You must first identify a vector input file!

If the end of file (EOF) is reached when attempting to read the

state vector, the following error message be displayed:

Error  End of File reached for: [filename]

See appendix D for vector file formats and examples.

4.5       Re-Epoch TLE at Asc Node

VEC2tle has the ability to move the epoch time on an existing TLE

to the time of the ascending node prior to the TLE's current

epoch time.  It should be noted that this feature is slightly

less accurate than deosculating a state vector directly at the

ascending node.  Thus, if the TLE must be epoched at the

ascending node and if the choice of inputs is an osculating state

vector or a TLE, the state vector is the data of choice.

The source for the TLE is the TLE input file if it has been

specified.  VEC2TLE reads the TLEs from this file in a sequential

fashion.  If the user wishes to skip to a TLE later in the file,

the 'Cancel' button should be activated until the desired TLE is

loaded.  If the user wishes to use a TLE earlier in the file, the

file should be re-opened under the TLE Input File option.  If

either no TLE input file has been designated or the TLE file has

been read past the last TLE, VEC2TLE allows the user to manually

input the TLE data.

4.6       Impulsive Delta-V

VEC2TLE has the ability to impart an impulsive or quasi-impulsive

delta-V to a TLE, computing the appropriate changes to the TLE.

This feature approximates the post-burn state vector and thus is

somewhat less accurate that a direct state vector deosculation.

The source for the TLE is the TLE input file if it has been

specified.  VEC2TLE reads the TLEs from this file in a sequential

fashion.  If the user wishes to skip to a TLE later in the file,

the 'Cancel' button should be activated until the desired TLE is

loaded.  If the user wishes to use a TLE earlier in the file, the

file should be re-opened under the TLE Input File option.  If

either no TLE input file has been designated or the TLE file has

been read past the last TLE, VEC2TLE allows the user to manually

input the TLE data.

Page 20

VEC2TLE performs some error detection and/or correction on the

TLE.  VEC2TLE cannot correct for: negative or zero mean motion,

eccentricity of one or more, or date/time errors in the TLE.

It should be noted that the element number and the revolution

number will be automatically incremented by VEC2TLE while it is

computing the post-burn TLE.

The delta-V burn data is entered in a dialog box following the

TLE input dialog.  This data must be entered manually.  The user

may return from this dialog box to the TLE dialog box by the Prev

button.

VEC2TLE will perform an impulsive (i.e., instantaneous) delta-V

at the burn ignition time if the burn duration, initial mass, and

final mass are all zero.  Otherwise, a quasi-impulsive burn is

assumed.  The difference between an impulsive and quasi-impulsive

delta-V is the effective time of the delta-V application.  The

quasi-impulsive delta-V assumes an impulsive burn at the time one

half the total burn delta-V is achieved.

If the delta-V is null (i.e., the three components are zero),

VEC2TLE will compute a new TLE representative of the orbit at the

effective time of the delta-V application.  This feature is

useful when the epoch on an element set needs to be moved to a

different time.

When a quasi-impulsive delta-V is requested, the mass/weight

units of the spacecraft and thruster propellant flow rate are at

the discretion of the user; the only restriction is that they

must be common units.  The total mass of the spacecraft must be

greater than the consumed propellant mass (i.e., initial mass

must be greater than the product of the burn duration and the

propellant consumption rate).

VEC2TLE cannot correct for the following in the delta-V data:

date/time errors, propellant consumption exceeding spacecraft

mass, or delta-V placing the spacecraft in an escape trajectory.

4.6       Program Termination

The program may be exited by either menu selection or by "hot

key."

To terminate the program by menu selection, activate the "Exit"

menu item in the "File" menu.

To exit the program by "hot key," depress ALT-X.

Page 21

5.0       REFERENCES

1.        Spherical Astronomy, Robin M. Green, Cambridge

University Press, 1985.

2.        Fundamentals of Astrodynamics, Roger R. Bate et al,

Dover Publications, Inc., 1971.

3.        Orbital Mechanics, Vladimir A. Chobotov, American

Institute of Aeronautics and Astronautics, 1991.

4.        Orbital Motion, Third Edition, A.E. Roy, Adam Hilger,

1988.

5.        Methods of Orbit Determination, Pedro Ramon Escobal,

Robert E. Krieger Publishing Company, 1976.

6.        PROJECT SPACE TRACK, Models for Propagation of NORAD

Element Sets, Felix R. Hoots and Ronald L. Roehrich,

Spacetrack Report No. 3, ADC/DO6, December 1980.

7.        Computertalk Space Object Catalog, Joel Runes, 1992.

Page 22

6.0       ACKNOWLEDGEMENTS

The author would like to express his appreciation to Dave Ransom

(Sysop RPV Astronomy BBS and author of STSPLUS) and Joel Runes

(Journalist and Computer Consultant) for their encouragement and

enthusiasm.  Without their timely and constructive comments,

VEC2TLE would be a less useful program.

Both Dave and Joel also provided valuable testing and validation

support during the STS-55, STS-56, and STS-57 missions as well as

key contributions to this User Manual.

Page 23

7.0       LICENSE STATEMENT

This software is protected by copyright law of the United States

of America.  This software is licensed as "Shareware".  This

software may be evaluated for a period of up to thirty (30) days.

If you wish to continue using this software after the initial 30

day trial period you must register.  You may not modify,

decompile, disassemble, or reverse-engineer the software, and you

may not remove or obscure the copyright notices.   Each

registered copy of this software may be used on only one computer

or one computer network at a time.

7.1       Registration

Registration of this software is accomplished by completing the

registration form (REGISTER.FRM) and remitting the $10.00 (U.S.)

registration fee as applicable in this section.  The registration

fee is applicable and required for all professional and

commercial users.  For the purposes of this section, a

professional or commercial user is defined as an individual or

organization who uses the data generated by the software in the

conduct, support, or pursuit of their profession or business.

Distribution of VEC2TLE by an Electronic BBS or by other

commercial shareware distribution methods is not considered

professional or commercial use of this software.  Distribution of

VEC2TLE-generated data (i.e., TLEs) are considered professional

or commercial use of this software.  Registration of a previous

release of VEC2TLE does not constitute registration of any future

releases.

The registration fee is not required (but would be appreciated)

for those using the software in support of a hobby.  For the

purposes of this section, a hobby is considered to be an activity

or pursuit for entertainment or relaxation purposes which is not

supporting professional or commercial activities.

Users who remit the $10.00 registration fee shall be licensed to

use the software for professional and/or commercial purposes.

Those licensed to use the software for professional and/or

commercial purposes shall receive a distribution diskette.  Users

who remit $20.00 shall receive a license to use the software for

professional and/or commercial purposes as well as a printed

version of the user manual found in the VEC2TLE.DOC file

(included with all distributions of the software).

7.2       Distribution

This software may be posted on Electronic Bulletin Board Services

(BBS) or otherwise distributed to individuals interested in its

capabilities.  This distribution is for the purpose of providing

Page 24

those individuals with the opportunity to evaluate its

capabilities under the above shareware restrictions.  When this

software is distributed, all original distribution files must be

included.  The following is a complete list of the original

distribution files:

INSTALL.BAT         Installation Batch File

VEC2TLE.EXE         Executable program file

VEC2TLE.CFG         Program configuration file

VEC2TLE.DOC         Software user manual

VEC2TLE.ICO         Icon suitable for use with Microsoft Windows

REGISTER.FRM        Registration Form

STS-57.VIF          Example State Vector from STS-57 in VEC2TLE-

readable format

VEC2TLE.NEW         Changes since the previous release

Except as stated in this section, you may not otherwise

distribute, lend, transfer, rent, lease, sub-license, or time-

share the software, diskettes, or documentation.

7.3       Limited Warranty

The publisher (Kenneth J. Ernandes) warrants the physical

diskette(s) and documentation provided to registered users (as

applicable) to be free of defects in materials and workmanship

for a period of ninety days (90) from the registration date.  If

the publisher is notified of defective materials or workmanship

during the warranty period, and such notification is determined

to be valid by the publisher, the publisher shall replace the

defective diskette(s) or documentation.

The entire and exclusive liability and remedy for breach of this

Limited Warranty shall be limited to replacement of defective

diskette(s) or documentation and shall not include or extend to

any claim for or right to recover any other damages, including

but not limited to, loss of profit, data, or use of the software,

or special, incidental, or consequential damages or other similar

claims, even if the publisher has been specifically advised of

the possibility such damages.  In no event will the publisher's

liability for any damages to you or any other party ever exceed

the registration fee paid for the license to use the software,

regardless of any form of the claim.

KENNETH J. ERNANDES SPECIFICALLY DISCLAIMS ALL OTHER WARRANTIES,

EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO, ANY IMPLIED

WARRANTY OF MERCHANTABILITY OR SUITABILITY FOR A PARTICULAR

PURPOSE.  Specifically, the publisher makes no representation or

warranty that the software is suitable for any particular purpose

and any implied warranty that the software is suitable for any

particular purpose and any implied warranty of merchantability is

Page 25

limited to the ninety-day duration of the Limited Warranty

covering the physical diskette(s) and documentation only (and not

the software) and is otherwise expressly and specifically

disclaimed.

This limited warranty gives you specific legal rights; you may

have others that vary from state to state.  Some states do not

allow the exclusion of incidental damages or consequential

damages, or the limitation on how long an implied warranty lasts,

so some of the above restrictions may not apply to you.

7.4       Governing Law and General Provisions

This license statement shall be construed, interpreted, and

governed by the laws of the State of New York.  If any provision

of this statement is found void, invalid, or unenforceable, it

will not affect the validity of the balance of this statement,

which shall remain valid and enforceable according to its terms.

If any remedy provided is determined to have failed of its

essential purpose, all limitations of liability and exclusion of

damages set forth in the Limited Warranty shall remain in full

force and effect.  The terms of this statement may only be

modified in writing signed by you and an authorized officer of

the publisher.  All rights not specifically granted in this

statement are reserved by Kenneth J. Ernandes.

Page 26

APPENDIX A -- HOT KEY SUMMARY

Menu Selection:

F10                           Menu Bar Selection

Alt-space                     =

Alt-F                         File

Alt-S                         Settings

Alt-C                         Compute TLE

Alt-W                         Window

Menu Item Selection:

=

About...                      N/A

File

Vector Input File             Alt-V

TLE Input File                Alt-E

TLE Output File               Alt-T

Output TLEs                   Alt-O

Exit                          Alt-X

Settings

Defaults                      F2

Inertial Reference            F3

Vector Units                  F4

Epoch Control                 F5

Compute TLE

Input State Vector            Alt-I

Read Vector File              Alt-R

Re-Epoch TLE at Asc Node      ALT-N

Impulsive Delta-V             Alt-D

Window

Close                         Alt-F3

Next                          PgDn

Previous                      PgUp

Zoom                          Alt-Z

Page 27

APPENDIX B -- SEQUENTIAL DAY OF THE YEAR

The tables in this appendix represent the sequential day of the

year corresponding to any date for both conventional years and

leap years.  The year and sequential day of the year is the most

common date format used for orbital applications.  The following

tables provide a look-up reference for conversion.

Table B.1  Day of the Year Calendar -- Non-Leap Years

Day  Jan  Feb  Mar  Apr  May  Jun  Jul  Aug  Sep  Oct  Nov  Dec

1   001  032  060  091  121  152  182  213  244  274  305  335

2   002  033  061  092  122  153  183  214  245  275  306  336

3   003  034  062  093  123  154  184  215  246  276  307  337

4   004  035  063  094  124  155  185  216  247  277  308  338

5   005  036  064  095  125  156  186  217  248  278  309  339

6   006  037  065  096  126  157  187  218  249  279  310  340

7   007  038  066  097  127  158  188  219  250  280  311  341

8   008  039  067  098  128  159  189  220  251  281  312  342

9   009  040  068  099  129  160  190  221  252  282  313  343

10   010  041  069  100  130  161  191  222  253  283  314  344

11   011  042  070  101  131  162  192  223  254  284  315  345

12   012  043  071  102  132  163  193  224  255  285  316  346

13   013  044  072  103  133  164  194  225  256  286  317  347

14   014  045  073  104  134  165  195  226  257  287  318  348

15   015  046  074  105  135  166  196  227  258  288  319  349

16   016  047  075  106  136  167  197  228  259  289  320  350

17   017  048  076  107  137  168  198  229  260  290  321  351

18   018  049  077  108  138  169  199  230  261  291  322  352

19   019  050  078  109  139  170  200  231  262  292  323  353

20   020  051  079  110  140  171  201  232  263  293  324  354

21   021  052  080  111  141  172  202  233  264  294  325  355

22   022  053  081  112  142  173  203  234  265  295  326  356

23   023  054  082  113  143  174  204  235  266  296  327  357

24   024  055  083  114  144  175  205  236  267  297  328  358

25   025  056  084  115  145  176  206  237  268  298  329  359

26   026  057  085  116  146  177  207  238  269  299  330  360

27   027  058  086  117  147  178  208  239  270  300  331  361

28   028  059  087  118  148  179  209  240  271  301  332  362

29   029       088  119  149  180  210  241  272  302  333  363

30   030       089  120  150  181  211  242  273  303  334  364

31   031       090       151       212  243       304       365

Page 28

Table B.2  Day of the Year Calendar -- Leap Years

Day  Jan  Feb  Mar  Apr  May  Jun  Jul  Aug  Sep  Oct  Nov  Dec

1   001  032  061  092  122  153  183  214  245  275  306  336

2   002  033  062  093  123  154  184  215  246  276  307  337

3   003  034  063  094  124  155  185  216  247  277  308  338

4   004  035  064  095  125  156  186  217  248  278  309  339

5   005  036  065  096  126  157  187  218  249  279  310  340

6   006  037  064  097  127  158  188  219  250  280  311  341

7   007  038  067  098  128  159  189  220  251  281  312  342

8   008  039  068  099  129  160  190  221  252  282  313  343

9   009  040  069  100  130  161  191  222  253  283  314  344

10   010  041  070  101  131  162  192  223  254  284  315  345

11   011  042  071  102  132  163  193  224  255  285  316  346

12   012  043  072  103  133  164  194  225  256  286  317  347

13   013  044  073  104  134  165  195  226  257  287  318  348

14   014  045  074  105  135  166  196  227  258  288  319  349

15   015  046  075  106  136  167  197  228  259  289  320  350

16   016  047  076  107  137  168  198  229  260  290  321  351

17   017  048  077  108  138  169  199  230  261  291  322  352

18   018  049  078  109  139  170  200  231  262  292  323  353

19   019  050  079  110  140  171  201  232  263  293  324  354

20   020  051  080  111  141  172  202  233  264  294  325  355

21   021  052  081  112  142  173  203  234  265  295  326  356

22   022  053  082  113  143  174  204  235  266  296  327  357

23   023  054  083  114  144  175  205  236  267  297  328  358

24   024  055  084  115  145  176  206  237  268  298  329  359

25   025  056  085  116  146  177  207  238  269  299  330  360

26   026  057  086  117  147  178  208  239  270  300  331  361

27   027  058  087  118  148  179  209  240  271  301  332  362

28   028  059  088  119  149  180  210  241  272  302  333  363

29   029  060  089  120  150  181  211  242  273  303  334  364

30   030       090  121  151  182  212  243  274  304  335  365

31   031       091       152       213  244       305       366

Page 29

APPENDIX C  -- ACRONYMS AND ABBREVIATIONS

ASCII          American Standard Code for Information Interchange

AFIT           Air Force Institute of Technology

BBS            Bulletin Board Service

CGA            Color Graphics Adapter

CIM            CompuServe Information Manager

ECI            Earth-Centered Inertial

EFG            Earth-Fixed Greenwich (see also TDR)

EGA            Enhanced Graphics Adapter

EOF            End Of File

ft             feet

ft/s           feet per second

GSFC           Goddard Space Flight Center

GPM            Greenwich Prime Meridian

GPS            Global Positioning System

GUI            Graphic User Interface

IAU            International Astronomical Union

IBM            International Business Machines

km             kilometers

km/s           kilometers per second

M50            Mean Equator, Mean Equinox of 1950

MS-DOS         Microsoft Disk Operating System

NASA           National Aeronautics and Space Administration

nm             nautical miles

nm/s           nautical miles per second

NORAD          North American Aerospace Defense Command

OIG            Orbital Information Group

PC             Personal Computer

RAAN           Right Ascension of the Ascending Node

RAID           Reports and Information Dissemination

RBBS           Remote Bulletin Board System

RPV BBS        Rancho Palos Verdes Astronomy BBS

SGP            Simplified General Perturbations

SGP4           Simplified General Perturbations Version 4

STS            Space Transportation System

STSPLUS        STS Orbit Plus

Page 30

TDR            True of Day Rotating (see also EFG)

TLE            Two-Line Elements

USSPACECOM     United States Space Command

UTC            Universal Time Coordinated

VEC2TLE        Vector to Two-Line Elements

VGA            Video Graphics Array

Page 31

APPENDIX D -- VECTOR FILE FORMATS

This section outlines the vector file formats input VEC2TLE.  It

should be noted that these formats are modifications to formats

output by Dave Ransom's STSPLUS software.  The modifications were

coordinated with Dave and were made such that they were

generalized without impacting VEC2TLE's ability to read an

STSPLUS state vector.

The vectors read in by VEC2TLE are in standard PC text format or

American Standard Code for Information Interchange (ASCII).  The

specifications for these files is not overly rigid since VEC2TLE

is intended to be reasonably flexible in terms of ability to read

vector files.

D.1       Header Line

The header line defines the parameter definitions of the vector.

This is the input vector file's counterpart to the configuration

parameters.  The header line must be the first non-blank line in

the vector file and must occur within the first 10 lines of the

file.  The header line may have a maximum of 80 characters, but

will be terminated when the first character with ASCII code

outside the range 21-127 is detected.  (This range covers most

keyboard characters exclusive of the carriage return and line

feed.)  VEC2TLE searches for the first digit (0-9) in the header

line following the equal (i.e., '=') sign.  Once a digit is

detected, up to the next three characters are read until a non-

digit character is read.  VEC2TLE forms up to a 4-digit integer

that specifies the parameters applicable to all vectors that

follow in the file.  The first contiguous integer string (up to 4

digits) specifies the file format.  Note that leading zeros are

implied and need not be included.

The significance of each digit is listed below:

THOUSANDS DIGIT     - INERTIAL REFERENCE

0         True equator, true equinox of date or ECI

1         Mean equator, mean equinox of 1950

HUNDREDS DIGIT      - VECTOR TYPE

0         Earth-Centered Inertial (ECI/XYZ)

1         Earth-Fixed Greenwich (EFG)

TENS DIGIT          - VECTOR UNITS

0         km, km/s

1         ft, ft/s

2         nm, nm/s

Page 32

ONES DIGIT          - DATA MODE FORMAT

4         Labeled data, one item per line

5         Two-line space delimited data

6         Single-line comma delimited data

7         Labeled data, one item per line

Note that a '4' or a '7' for the data mode format indicates the

same vector file format.  This was done to accommodate vector

data output from STSPLUS.  (Data mode '4' in STSPLUS is identical

to data mode '7' except that state vectors are only written to

the output file immediately after the satellite has crossed the

ascending node.)

D.2       Vector Formats

This section specifies the three vector formats that VEC2TLE is

capable of correctly reading.  Each format has an example,

provided courtesy of Dave Ransom, using True equator/equinox ECI

vectors in units of ft and ft/s.  All vector formats have a

date/time format with a two-digit year and decimal day.

D.2.1     Two-Line Space Delimited

The two line space delimited format has the following data on the

first line: satellite catalog number, vector date/time format,

and the three components of the position vector.  The second line

is the three components of the velocity vector.  Each line may be

up to 80 characters long.

The example below has two state vectors in it.  Note that some of

the spaces have been removed in the interest of easily fitting

the data within the margins.

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 15

20580 93110.045081 13656864.66720 17514322.54968 -5452252.42794

-16168.27686974290  15789.75251859515  10248.33566657315

20580 93110.045139 13575822.39276 17593013.13238 -5400930.14914

-16248.48336702945  15686.35334359047  10280.38786725583

D.2.2     Single-Line Comma Delimited

The single-line comma delimited format has data in the following

order: epoch flag (single digit -- ignored), units flag (single

digit -- ignored), date/time, the three components of the

position vector, and the three components of the velocity vector.

Each vector line may be up to 130 characters long.

The example below has two state vectors in it.  Note that the

vectors have been placed on multiple lines in the interest of

easily fitting the data within the margins.

Page 33

STSORBIT PLUS Data Output to STSPLUS.LOG, Data = 16

0,1,20580,93110.0482060185,8771281.06140276,20969911.6226162,

-2491608.1429704,-19762.0576284838,9623.02409449012,

11524.6952898439

0,1,20580,93110.0482638889,8672341.7437806,21017717.5456987,

-2433947.77946384,-19813.4647395816,9499.28476766938,

11539.2506632381

D.2.3     Labeled Data, One Item Per Line

The labeled data, one item per line format has data in the

following order: satellite name or ID, satellite catalog number

and IAU designator, date/time, descriptive date time (ignored),

the components of the position vector, the components of the

velocity vector, ndot/2, nddot/6, and Bstar.  Each line may be up

to 80 characters long, but characters with ASCII codes outside

the range 21-127 are ignored.  VEC2TLE will also ignore any data

in columns 1-24 on the satellite name or ID line and will also

ignore data in columns 1-17 on the remaining lines.  The IAU

designator must be in column 32, following the satellite catalog

number.

The example below has one state vector in it.

Vector format = 1017

Satellite Name:         STS-57

Catalog Number:         22684  93 37  A

Epoch Date/Time:        93180.63736111111

06/29/1993  15:17:47 UTC

ECI X:                     8024498.723185 ft

M50 Y:                   -19652913.243254 ft

Z:                     6620097.066346 ft

Xdot:                     20418.75391 ft/s

Ydot:                     11591.42188 ft/s

Zdot:                      9232.20312 ft/s

ndot/2 (drag):              0.00002834000 rev/day^2

nddt/6:                       0.00000E+00 rev/day^3

Bstar:                        5.17270E-05 1/Earth Radii

Elset #:                               71

Rev @ Epoch:              126.10967365930

Page 34

APPENDIX E -- SOURCES OF STATE VECTORS

This section provides sources of state vector information.

E.1       NASA Spacelink

State vectors for current Space Shuttle missions may be obtained

from NASA Spacelink (see Appendix F.5 for log-on details).  There

is usually one vector posted each evening (approximately 17:00

Central Time) during an active Shuttle mission.  These may be

found under the following menu sequence: Current NASA

News::Current Shuttle Flight::Keplerian Orbital Elements/State

Vectors.  While these are usually accompanied with TLEs, there

are times when the state vector is more current than the TLE.  At

times planned maneuver data is posted as well, which is useable

in the Impulsive Burn function.

E.2       RPV BBS

Dave Ransom's RPV BBS  (see Appendix F.3 for details) is a source

of pre-launch nominal EFG vectors for Space Shuttle missions.

Due to the nature of EFG vectors, the nominal orbits of an entire

mission may be computed using VEC2TLE once the Shuttle liftoff

time is known.  This is accomplished by adding the vector offset

time to the launch time and using the EFG position and velocity

components as listed.  While this data are the predicted state

vectors, it can be very helpful when real-time information is not

available.

E.3       NASA Select TV

NASA Select TV is a real-time source for Space Shuttle maneuver

burn data (the state vectors are not particularly legible).  NASA

Select TV is available by direct downlink from Satcom F2R,

Transponder 13 and on some cable TV systems.  (For what it's

worth, a group in Houston got their local cable TV company to

provide NASA Select TV by providing a petition with 300 names on

it.)  The audio portion of NASA Select TV is rebroadcast by many

HAM radio clubs through repeaters on VHF and UHF frequencies.

For a listing of known repeater locations and frequencies, see

file STSAUDIO.ZIP on the RPV BBS.

The burn data is useful in that it supplies the changes to the

Shuttle's velocity vector at the burn time.  If you supply

VEC2TLE with the Shuttle's most current TLE and burn data, you

can estimate the post-maneuver TLE using the Impulsive Delta-V

function.

Page 35

E.4       CompuServe

The author periodically posts state vectors related to a Space

Shuttle mission in the CompuServe Astronomy, Space, and HamNet

Forums.  These vectors are part of general informational forum

messages (with the VEC2TLE-readable portion encapsulated between

dotted lines.  This portion of the message may be copied to a

text file (with a .VIF extension recommended) for use by VEC2TLE.

The CompuServe Information Manager (CIM) software has a copy-and-

paste feature and text file support that are well-suited to

accomplish this task.

E.5       Other Orbit Propagators

VEC2TLE may be used to convert the elements of other orbit

propagators to TLEs.  This assumes that you have that orbit

propagator available to you in order to compute positions and

velocities.  Those velocities, in turn, may be input to VEC2TLE

in order to compute TLEs representative of the other propagator's

orbital elements.

E.6       The NASA OIG RAID-RBBS

The Sysop on the NASA OIG RAID-RBBS is exploring the possibility

of posting state vectors in real-time for the Space Shuttle.  The

plan would be to automatically post vectors with updates as often

as every hour.  The plan is also to format the vector file in the

labeled data, one item per line format that is readable by

VEC2TLE.  If this is successfully implemented, this will provide

a maximum of a one-hour response time for obtaining accurate

orbital data on the Shuttle following events such as a thruster

firing that would change the current orbit.

I would encourage all Shuttle trackers to write to the Sysop of

NASA OIG RAID-RBBS and indicate strong support for implementation

of automatic State Vector postings on this system.  See section

F.1 for the NASA OIG RAID RBBS address and details for gaining

access to this system.

Page 36

APPENDIX F -- SOURCES OF TLEs

[This section was provided courtesy of Joel Runes.]

One of the most frequently asked questions is "Where can I obtain

TLEs?"  There are several different sources for TLEs which are

outlined in this section.

F.1       The NASA OIG RAID-RBBS

This source is the primary method to obtain TLEs on over 6,000

unclassified objects.  More than eight years in development, this

bulletin board system became operational in September, 1991, and

has revolutionized near real-time random access dissemination of

TLEs.  For a long time NASA discharged its responsibilities to

disseminate orbital elements to non-military users by mailing

Prediction Bulletins with TLEs, node crossing times and

longitudes, and information on other latitude crossings relative

to the node crossings.  This was a direct descendant of the

satellite observer network infrastructure of the late 1950's and

early 1960's.  The Prediction Bulletin system was quite limited.

Users were allowed to select a maximum of 20 satellites from a

list of only a thousand or so objects available at all.  The

predictions are in hardcopy form, received after postal delays,

and had to be hand-transcribed into computer systems to be of

much use.  The RBBS is a bulletin board system which replaced the

old Prediction Bulletins, saved innumerable forests from

destruction, saved the government money and provides useful

information to users WHILE it is still useful.  The RBBS is

maintained by a Goddard Space Flight Center (GSFC) contractor.

In order to use the RBBS, you MUST submit a WRITTEN request to

the address below.  No big song and dance is required in the

request (nor paid attention to).  Just state that you would like

to use the OIG's RAID-RBBS to download TLEs.  Mail the request to

the address below and they will mail you a copy of the User's

Guide and you User ID and initial password.  Why can't you

register online as you can with almost any other bulletin board?

Because NASA is part of the government and they say you can't.

The address:

NASA Goddard Space Flight Center

Project Operations Branch, Code 513

Greenbelt, MD 20771  (USA)

ATTN: OIG RAID-RBBS Access Request

When logged on to the RBBS there are two basic methods to obtain

TLEs.  For popular objects, such as manned spacecraft, ham sats,

weather satellites, communications satellites, Global Positioning

Page 37

System (GPS) satellites, visible satellites and recently launched

satellites, the RBBS has downloadable files with the most recent

TLEs.  For other satellites, rocket bodies and debris objects,

the RBBS has a random access database of TLEs which you may

query.  The database is keyed to the objects NORAD number.  A

cross-reference facility is available to correlate NORAD numbers

and Int's Designations.

F.2       Celestial BBS

While impatiently awaiting NASA's step forward into the era of

personal computers, Major T.S. Kelso, a professor at the Air

Force Institute of Technology (AFIT) in Ohio founded the

Celestial BBS.  It has been a source for TLEs since 1986 or so.

Originally the TLEs on his bulletin board were transcribed from

Prediction Bulletins.  Capitalizing on his Air Force contacts, he

has been able to distribute TLEs obtained from Space Command

since 1990.  The TLEs available on Celestial BBS cover hundreds

of satellites frequently of interest to the user community.  The

TLEs are maintained in downloaded files.  The bulletin board also

contains many programs to track satellites and maintain TLEs and

is a messaging facility for many satellite users and observers.

You CAN become a user of Celestial BBS by registering online,

the phone number is (513) 427-0674.  The bulletin board is a free

access bulletin board but contributions are accepted and

deserved.  Even though the government has finally gotten around

to distributing TLEs in electronic form through the RBBS, Maj.

Kelso's bulletin board continues to provide an invaluable service

to satellite users and observers.

F.3       Dave Ransom's RPV BBS

While there is a number of bulletin boards which redistribute

TLEs from Celestial BBS and the RAID-RBBS, special mention should

be made of Dave Ransom's RPV BBS in Rancho Palos Verdes, CA.  The

phone numbers are (310) 541-7299 for the main BBS and (310) 544-

8977 for the Hotline BBS.  In addition to compilations of TLEs

redistributed from the RBBS, Celestial BBS and the Canadian Space

Society BBS, the RPV BBS contains up-to-date versions of

satellite tracking software and utilities (including VEC2TLE) and

catalogs of orbiting objects.  It also contains much high quality

astronomical software,  Dave Ransom is the author of the STSORBIT

and STSPLUS satellite tracking programs.  STSPLUS is particularly

well-suited for use in conjunction with VEC2TLE.  In addition,

the RPV BBS is a premiere source for CURRENT TLEs during Space

Shuttle missions.  Shuttle TLEs are frequently available on the

RPV BBS hours sooner than they are available from any other

source due to Dave Ransom's persistence and resourcefulness.

Page 38

F.4       CompuServe's Astronomy Forum

For members of CompuServe, files containing TLEs are maintained

in ASTROFORUM Lib 3.  Messages relating to satellite viewing are

in Message Section 3 of the ASTROFORUM.  Access to CompuServe is

usually obtained on the first try as opposed to frequent busy

signals for Celestial BBS and RPV BBS.

F.5       NASA SpaceLink

NASA SpaceLink BBS is maintained by the Marshall Space Flight

Center in Huntsville, Alabama.  It can be accessed at (205) 895-

0028.  The BBS is primarily oriented to educators and contains

many files related to various spaceflight activities and

spaceflight-related educational opportunities.  It operates 24

hours/day and users may register online.  SpaceLink posts TLEs

for Shuttle missions and some other satellites.  It is also a

source for state vectors and press kits for NASA missions in

electronic form.

Page 39

APPENDIX G -- GENERAL INFORMATION

This section is used for information pertaining to general topics

related to VEC2TLE.

G.1       Suggested Uses for VEC2TLE

[This section was provided courtesy of Joel Runes.]

While the normal use of VEC2TLE is to generate TLEs from State

Vectors as outlined throughout this document, there are a number

of applications of VEC2TLE which may not be immediately obvious.

This section discusses some of those potential applications, but

this discussion should be considered representative rather than

exhaustive.  Uses of the program are constrained by the

imagination of the user more than by the limitations of the

software.

One use of VEC2TLE is to explore the effect of an orbit adjust

maneuver on the orbit of a spacecraft.  If the user has TLEs for

the object from before the orbit adjust maneuver, an alongtrack,

crosstrack, and radial delta-velocity may be applied to change

the TLE parameters or STSPLUS may be used to generate a state

vector at the time of the maneuver.  Given the latter case,

adjusting the values of Xdot, Ydot, and Zdot to reflect the

velocity adjustments associated with the maneuver, VEC2TLE can be

used to generate post-burn TLEs.  To compare the pre-burn

trajectory with the post-burn trajectory, simply use the pre-burn

and post-burn TLEs in a prediction program such as STSPLUS.  This

application has been successfully demonstrated during Shuttle

missions STS-56 and STS-55.  The result was useful post-burn TLEs

prior to availability of post-burn TLEs from Space Command.

Another use of VEC2TLE is to explore the effect on a satellite's

orbit due to minor dispersions in velocity and position during

the launch phase.  When a spacecraft such as the Space Shuttle is

launched, the desired post-launch state vector is used by the

spacecraft's and/or launch vehicle's guidance software.  But the

orbit achieved may vary from the planned orbit.  What effect does

that variation have on the orbit?  VEC2TLE is a valuable tool for

the exploration of this question.  Suppose that the desired post-

launch state vector is known (or assumed), and that the position

and time of the state vector are held constant.  Suppose further

that the achieved velocity in each axis varies by � 0.2 ft/sec.

VEC2TLE can generate a number of TLEs, first for the nominal

trajectory, then for trajectories where the velocity components

are adjusted up and down by the 0.2 ft/sec dispersion assumption.

The resulting TLEs may be used with prediction programs such

STSPLUS to show the effect of the injection errors on the orbit

achieved and the satellite position at any time thereafter.

Page 40

Another use of VEC2TLE is to explore the required velocity change

to put a satellite into an orbit different from its initial

orbit.  There a large number of situations that can be examined

with the aid of VEC2TLE along these lines, but let's consider a

very simple situation.  A spacecraft is in an orbit with a very

low eccentricity -- approximately a circular orbit.  We desire to

raise the orbit's apogee by 500 km.  First we construct TLEs that

exhibit the low eccentricity and initial altitude.  The process

of constructing the initial TLEs may be simplified by using the

tables from Part 0 of the Computertalk Space Object Catalog

(reference 7 -- may be found on CompuServe Astronomy Forum and

many of the BBSs listed in Appendix F, files NC00.ZIP -

NC13.ZIP).  The next step is to generate a state vector for the

time when the apogee raising maneuver is to be performed (this

can be accomplished using STSPLUS).  In VEC2TLE, apply the

posigrade burn by incrementing the magnitudes of Xdot, Ydot, and

Zdot by the same percentage and maintaining the same sign for

each of the velocity components.  (An excellent first guess would

be to compute the two-body elliptical velocity using the standard

formula, which may be found in references 3-5.)  VEC2TLE can now

create a new set of TLEs which will have a perigee of the

original orbital altitude and an apogee which is greater

according to the percentage increase of the velocity components

applied.  Using either the Computertalk Space Object Catalog or

displays from a prediction program, you can determine whether the

apogee was raised by more or less than the desired 500 km.

Return to the original state vector and adjust the percentage

change to Xdot, Ydot, and Zdot until the desired 500 km apogee

increase is obtained.  To calculate the total change of velocity

which raised the apogee by 500 km, take the square root of the

sum of the squares of the changes to Xdot, Ydot, and Zdot.

While the previous example was relatively simple, the same basic

method of iteratively creating and examining the resultant TLEs

can be used to explore strategies for accomplishing the

rendezvous between orbiting objects.  Taking into account the

dispersions in the orbit determination process and the

effectiveness of orbit adjust maneuvers, the user can calculate

the required velocity change to accomplish a rendezvous sequence.

The ways in which VEC2TLE, in conjunction with satellite tracking

programs such as STSPLUS, may be used for the teaching of orbital

mechanics and astrodynamics in the high school, undergraduate, or

graduate school environments are limited only by the imagination

of the educators.  Educators who utilize VEC2TLE for such

instructional purposes are encouraged to contact the author so

that those educational strategies may be detailed in future

versions of this documentation.

Page 41

G.2       Planned Upgrades for VEC2TLE

This section provides information regarding planned upgrades to

VEC2TLE.  Registered users are always encouraged to suggest items

to be added to this list.  The following upgrades are planned for

the next (or some subsequent) release of VEC2TLE:

-- The ability to estimate the TLE drag parameters (ndot/2,

nddot/6, and Bstar) based on spacecraft ballistic coefficient

and a static and/or dynamic atmosphere model.

-- The ability to compute TLEs that are fully compatible with

SDP4 (deep space version of SGP4).

-- The ability to read and write Keplerian elements in Amateur

Radio Satellite (AMSAT) format if there is sufficient

interest.

Page 42

APPENDIX H -- ABOUT THE AUTHOR

Ken Ernandes is an aerospace consultant specializing in

astrodynamics analysis, system simulation, and software

development.  He has a BS in Physics/Mathematics from Manhattan

College in New York City (1980) and a ME in Space Systems

Engineering/Engineering Management from the University of

Colorado at Boulder (1990).

Ken started in the space program after joining NORAD in October

1981, working as a crew Orbital Analyst in Cheyenne Mountain.

The job entailed detecting, tracking, identifying, and computing

orbits for all man-made Earth-orbiting satellites as well as

real-time analysis of foreign space events, support of manned and

unmanned space missions, direction of the worldwide tracking

network, precision orbital decay predictions, management of on-

line computer activities, and operational command briefings.

Ken was selected to be an Orbital Analyst Instructor in November

1981.  His job responsibilities included developing and

conducting initial and follow-on training for all crew-certified

Orbital Analysts as well as space indoctrination training for

senior and flag rank officers.  His greatest achievement as an

instructor was the development of the Orbital Analyst Positional

Handbook.  A derivative of this training manual and reference is

still in operational use today.

After leaving active service in the Air Force, Ken joined Martin

Marietta Aerospace, Denver, CO in July 1984.  During his tenure

at Martin Marietta, Ken was responsible for astrodynamics and

mission analysis as well as system and mission simulation for

design-phase spacecraft.

Ken joined Science Applications International Corporation in

October 1985.  At SAIC he was the lead orbital engineer in the

development of test scenarios; exercising Cheyenne Mountain's

computers under wartime conditions.  This led him to being the

lead test designer for integration testing of the Space Defense

Operations Center (SPADOC) computer upgrade.  Ken left SAIC in

April 1992 in order to become an independent consultant.  In

addition to his consulting work, Ken is an Assistant Professor of

Physics (Adjunct Faculty) at the State University of New York

(SUNY) at Farmingdale.

Ken may be contacted at the following addresses:

Ken Ernandes

16 Freshman Lane

Stony Brook, NY 11790-2712

CompuServe:  70511,3107

Internet:    70511.3107@cis.com