This page is not intended to be bound into the printed manual

Printing this Manual

This manual is in Microsoft Word for Windows version 6.0 format.  We have chosen this format because most users will have access to a computer that has this wordprocessor.  If you do not have access to a copy of Word you can download a free copy of the Microsoft Word Viewer from Microsoft’s home page at http://www.microsoft.com.  Word Viewer will allow you to display this manual on screen and to make a printed copy.

Unfortunately the USA uses 8.5 by 11 inch Letter size paper, while most of the rest of the world uses 210 by 297 mm A4 paper.  In order to get the correct layout of this manual on both sizes of paper it is necessary to adjust the margins to keep the printed area the same.  Please use File | Page Setup to select the size of paper that your printer uses, and then set the margins as shown below.   If you are going to print the manual on both sides of the paper then select the Mirror Margins check box.

Letter Size		  A4 Size

inch	cm		inch	cm

Left		1.0	2.54		1.0	2.54

Right		0.98	2.49		0.75	1.91

Top		0.8	2.03		1.04	2.64

Bottom		0.6	1.52		1.04	2.64

The distribution copy of the manual will be preset to the margins required for A4 Size paper.

The quality of the graphics and screen images in this document is very dependent on the printer you use, and how you set up your printer.  We have included the screen images in colour so that you can take advantage of this if you have a colour printer.

If you are printing in black only, it is important that you set the dithering options for your printer to the best quality possible.  For example, with the Hewlett Packard Series 4 LaserJet printers you can get excellent quality grayscale images.  If you print your copy of the manual on one of these printers use the File | Print option (ie instead of clicking the printer icon), and then select “Properties”.  On the Graphics page set the “Dithering” option to “Error Diffusion”.  This may slow down your printing slightly, but it generates the best quality.  Re-set the dithering to “Coarse” when you are finished if you want the fastest speed for general printing.

This page is not intended to be bound into the printed manual

PACKED

COLUMN

CALCULATOR

Version 1.1   August 1998

Copyright (C) 1998 Katmar Software

PO Box 654, Westville 3630, South Africa

NOTE :

Please write your registration details in the spaces below when you print out your copy of the manual.  If you ever move your copy of Packed Column Calculator to a new computer it will revert to being a shareware version.  You will then have to use the Register | Permanent menu option and enter this information to convert it to the registered version.

Registered name :	________________________________________

Registration code :	________________________________________

"Windows", “Windows 3.1”, and “Windows 95” are trademarks of Microsoft Corporation.

"Delphi" is a trademark of Borland International

Table of Contents

Part 1  User Manual

1 Introduction to the Packed Column Calculator Program

2 Hardware and Software Requirements

3 Installation and Start-up

4 Quick Start for Experienced Users

5 Printing the Manual

6 Detailed Operating Instructions

6.1 Start-up

6.2 Getting Online Help

6.3 Setting Up the Calculation Parameters

6.4 Entering Data

6.4.1 Clearing the Worksheet

6.4.2 Entering and Editing Packing Data

6.4.3 Entering and Editing Process Information

6.5 Performing the Calculation

6.5.1 Displaying the Generalised Pressure Drop Correlation

6.5.2 Displaying the Operating Diagram

6.6 Reading Existing Files of Data

6.7 Saving Your Data to Disk

6.8 Editing the Project Description

6.9 Entering your Registration Key

6.10 Printing the Calculation Results

6.11 Exiting from the Program

7 Software Support

8 Bug Reports and Wish Lists

9 Revision History

Part 2 - Tutorial

1 Detailed Worked Examples

Part 3 - Appendices

1 APPENDIX A - Discussion of Eckert and Robbins Correlations

2 APPENDIX B - Accuracy Considerations in Packed Column Design

3 APPENDIX C - The Shareware Distribution System

4 APPENDIX D - The Packed Column Calculator Licensing Policy

5 APPENDIX E - Ordering a Copy of Packed Column Calculator

5.1 Ordering from PsL

5.2 Order Form for Credit Card Purchases by Fax or Mail from PsL

5.3 Ordering from Share-It

5.4 Order Form for Credit Card Purchases by Fax or Mail from Share-It

5.5 Ordering from Sw-Reg

5.6 Ordering from Shareware-Australia

5.7 Order Form for Credit Card Purchases by Fax or Mail from Shareware-Australia

5.8 Ordering from Chempute Software

6 APPENDIX F - Order Form for Ordering directly from Katmar Software

7 APPENDIX G - Contacting Katmar Software

8 APPENDIX H - Bibliography

9 APPENDIX I - Other Programs available from Katmar Software

# Part 1  User Manual

## Introduction to the Packed Column Calculator Program

The design of new columns and the rating of existing columns are tedious tasks to perform manually because of the trial and error nature of the established design methods.  The Packed Column Calculator program changes this situation completely by performing all the number crunching, freeing the engineer up to do the true design work and thinking involved.

The program is set up to easily allow the calculation of the required diameter for new columns, using as a design basis the allowable pressure drop, the approach to flood or the approach to maximum operating capacity.  Existing columns of known diameter can also be rated for proposed changes in operating conditions, or to evaluate the effect of using a different packing.

An additional capability built into the program is the calculation of the operating diagram.  Although every text book dealing with the design of packed columns refers to the operating diagram, it is very rare for these diagrams to be prepared where the calculations are being done by hand.  If it is a tedious job to do one design by hand, the need to perform hundreds of designs to prepare a meaningful operating diagram makes it impossible by hand.  The computer can do this for you in a few seconds, and you can immediately see the sensitivity to changes in gas and liquid flowrates.

The Packed Column Calculator program incorporates two of the most widely used design methods.  The first of these is the Generalised Pressure Drop Correlation (GPDC), originally developed by Leva (1933) and used here in the form given by Eckert (1975).  The second method is that developed by Robbins (1991).

These two methods complement each other in that the GPDC is based on original work correlating flooding data that was then extended to the lower liquid flow regime, while the Robbins method is based on the flow of gas through dry beds of packing with extensions to the formulas to accommodate the flow of liquid.  These two method are discussed in greater detail in Appendix A.

Another reason for incorporating two design methods is to reinforce the fact that packed column design methods are not absolute, and different correlations will give different answers.  This variability has as much to do with the nature of the process as it has to do with the differences between the correlations.  The question of design accuracy is further discussed in Appendix B.

When used on columns that are being designed for the typical operating regimes, the different correlations will tend to give similar answers and will give added confidence to the design.  However when used at the boundaries of the possible operating regimes the answers are likely to diverge, thus highlighting the fact that you are working in a riskier area and that further investigation is warranted.

As good as we believe this program is, no computer program can be a substitute for proper training and experience.  This program should not be used by anyone who does not understand the underlying process, and who could not perform the design by hand, given sufficient time.  Katmar Software cannot guarantee the results of the program because of the extremely wide range of possible applications.

As always, critical designs must be checked by independent means.  Your packing supplier will be keen and able to check your final designs.  If the supplier is not keen and able, you should think about changing your supplier!

Those who are accustomed to using computer programs for engineering design will be able to get going quickly by reading through the "Quick Start for Experienced Users" section in the manual.  Newcomers to computerised design methods are recommended to work through the Tutorial in Section 2.  This will take you step by step through the details of how to use this program.

This program is distributed as shareware.  It is not free software.  The evaluation version is fully functional (except for the restricted packing factor range) and is designed for you to be able to thoroughly evaluate the program.  The evaluation copy will operate for at least 30 days.  At the end of that time (or preferably during that time!) you can purchase the registered version which eliminates the shareware reminders and the time limit.  Thanks to the power of the Internet you can receive the registered version within minutes of deciding to purchase it.  Full details on the licence conditions and ordering procedures are included in the Appendices.  For those of you new to the shareware distribution system, a detailed description of shareware is also included in Appendix C.

Katmar Software also publishes a steam tables package and a project risk analysis program as shareware.  Details of these programs are given in Appendix I.

## Hardware and Software Requirements

The hardware and software requirements are simple.  If your computer is running Windows 3.1 or Windows 95 it will run the Packed Column Calculator.  You will need 3 Mb of hard disk space for a full installation, although you will temporarily need about 6 Mb while you print the manual.

## Installation and Start-up

An automatic installation program is included with Packed Column Calculator.  The package is usually distributed as a single zipped file called pcol1611.zip.  Unzip the file to a temporary directory on your hard disk.  From the Windows program manager select File | Run and then Browse to wherever you have the unzipped Packed Column Calculator package.  Select "INSTALL.EXE" and run it.  This will guide you through the installation procedure.

The default directory for the Packed Column Calculator files is C:\PACKCOL, but the installation procedure will allow you to change this before it copies the files.  Note that this is a 16 bit application and although you can install it under Windows 95, you cannot use long directory and file names.  After the files are copied the installation procedure will create a new program group in the Program Manager and will create icons for Packed Column Calculator and some of its support files.

To run the program simply select the Packed Column group and then the Packed Column Calculator icon.

After you have installed the program, do not forget to delete the temporary directory and its files.  This is a bit different from the usual procedure with commercial software where you receive a box with ten or more floppy disks and you can install directly from the floppy disks.  Our primary distribution method is the Internet and we prefer to have a single distribution file which is as small as possible for obvious reasons.

If the copy of Packed Column Calculator that you have received does not have the installation program (ie INSTALL.EXE), it is very easy to do a manual installation as described below, or you can download the latest full copy from our web page - see Section 7 for the address. The complete package consists of the following files.  Only the files marked with a * are absolutely essential.  Create a new sub-directory, typically named C:\PACKCOL, and copy all the files you have from this list to the new sub-directory.

* PCOL.EXE			Executable program file

* PCOLHELP.HLP		On-line help

* PCOLDEFS.DTA		Options data file

INSTALL.EXE		The installation utility for Packed Column Calculator

WINSTALL.INF		Parameter file for installation utility

WINSTALL.HLP		Help file for installation utility

PCOLBOOK.DOC		Reference manual and tutorial

README.TXT		Basic information on installation, costs and licence

FILE_ID.DIZ			Automated file format for BBS’s and web sites

PCOL_EGn.COL		Example data files      n = 1, 2 & 3

REGISTER.TXT    		Form for ordering directly from Katmar Software (paying by check only)

PSLFAX.TXT      		Form for ordering from PsL by fax using your credit card

SHAREIT.TXT     		Form for ordering from Share-It by fax using your credit card

SWAFAX.TXT     		Form for ordering from Shareware-Australia by fax using your credit card

If you are using Windows 3.1x use the Windows Program Manager function File | New to create a new group named "Packed Column" and then use File | New once more to add PCOL.EXE to the newly created group.  When you run Packed Column Calculator for the first time it will set itself up correctly as soon as it detects that the automatic installation routine was not used.

If you are using Windows 95, click the Windows Start button, and then point to Settings.  Click Taskbar, and then click the Start Menu Programs tab.  Click Add, and then click Browse.  Locate PCOL.EXE, and then double-click it.  Click Next, and then double-click the Programs menu, or the sub-menu in which you    want the program to appear.  Type the name, i.e. Packed Column Calculator, and then click Finish. When you run Packed Column Calculator for the first time it will set itself up correctly as soon as it detects that the automatic installation routine was not used.

## Quick Start for Experienced Users

If you have had any experience with other computer programs for engineering design you will probably be able to get up and running with Packed Column Calculator without reading the entire manual first.  Just read through this section, fire up the program, and use the on-line help if you get stuck.

In a nutshell, all you have to do is set the four radio button option boxes along the top of the screen, enter the required data, and then press one of the “Calc” buttons.

The first radio button box allows you to specify whether you are performing a new design, or rating an existing column.  If you select the “Rating” mode the “Design Basis” option box will not be active.  The second option box allows you to toggle between the Eckert and Robbins design methods.

The third option box allows you to select the basis that will be used in performing a new design.

The last option box allows you to select the units to be used.  You can accommodate mixed units by changing the selection in the middle of data entry to suit the available data.  For example, if all your data is in SI units, but the liquid viscosity is in centipoise, you should switch to metric units before you enter the viscosity and then back to SI once you have entered it.  The program will convert all previously entered data to the currently selected units.

There is one very important aspect to note with respect to the units.  The packing factor is always in the units as defined by Eckert.  Over the years various different “metricated” forms of the packing factor have been proposed.  Unfortunately there has not been much consistency in this regard and we have decided to avoid this confusion by using Eckert’s definition and units in all cases.  This means that the “Y” parameter (ie the vertical axis) scale on the GPDC is adjusted to accommodate the packing factor units in each case.

Once you have set the option boxes you should enter the packing details.  You can enter the data manually, or you can click on the “Select Packing” button to select the packing from the built in database. When selecting from the database work from left to right, ie, first select the packing type, then the material and finally the size.  When all your selections have been made click the “OK” button and the applicable data will be entered into the edit boxes. By the way, if there are particular packings that you use regularly that we have not included, please send us a copy of the data sheet and contact details of the supplier so that we can arrange to include them.

Not all of the packing information is required to perform the Basic Calculation.  If you leave out any required information the program will advise you as soon as you press a “Calc” button.  Less information is required to perform a Basic Calculation than a Detailed Calculation.

Enter the gas and liquid properties and flowrates, and then either the design basis value or the column diameter, depending on the options selected.  The labels on the edit boxes will of course change to tell you exactly what it is that you need to enter.  When entering data you can move from one field to the next by pressing TAB (as per the Windows convention) or by pressing ENTER (this is to accommodate the ex-DOS users!).

Once all the data is entered click on a “Calc” button.  The program will then calculate what it can with the available data, and will give you a warning if any required information is missing.

Once the calculation has been done, you can click on the “Op. Diagram” button to display the operating diagram.  The Operating Diagram will vary slightly with the design correlation used because each has its own limits of applicability.  You can also click the “G.P.D.C.” button if you are using the Eckert correlation.  This will plot the standard Generalised Pressure Drop Correlation chart, with the specified operating point plotted.

The tabular or graphical data can be printed out by clicking on the “Print” button on the applicable screen.

In order to exit from the Packed Column Calculator program you must first return to the Data Entry screen, and then click the “Quit” button.

## Printing the Manual

This manual is in Microsoft Word for Windows version 6.0 format.  We have chosen this format because most users will have access to a computer that has this wordprocessor.  If you do not have access to a copy of Word you can download a free copy of the Microsoft Word Viewer from Microsoft’s home page at http://www.microsoft.com.  Word Viewer will allow you to display this manual on screen and to make a printed copy.

Unfortunately the USA uses 8.5 by 11 inch Letter size paper, while most of the rest of the world uses 210 by 297 mm A4 paper.  In order to get the correct layout of this manual on both sizes of paper it is necessary to adjust the margins to keep the printed area the same.  Please use File | Page Setup to select the size of paper that your printer uses, and then set the margins as shown below.   If you are going to print the manual on both sides of the paper then select the Mirror Margins check box.

Letter Size		  A4 Size

inch	cm		inch	cm

Left		1.0	2.54		1.0	2.54

Right		0.98	2.49		0.75	1.91

Top		0.8	2.03		1.04	2.64

Bottom		0.6	1.52		1.04	2.64

The distribution copy of the manual will be preset to the margins required for A4 Size paper.

The quality of the graphics and screen images in this document is very dependent on the printer you use, and how you set up your printer.  We have included the screen images in colour so that you can take advantage of this if you have a colour printer.

If you are printing in black only, it is important that you set the dithering options for your printer to the best quality possible.  For example, with the Hewlett Packard Series 4 LaserJet printers you can get excellent quality grayscale images.  If you print your copy of the manual on one of these printers use the File | Print option (ie instead of clicking the printer icon), and then select “Properties”.  On the Graphics page set the “Dithering” option to “Error Diffusion”.  This may slow down your printing slightly, but it generates the best quality.  Re-set the dithering to “Coarse” when you are finished if you want the fastest speed for general printing.

## Detailed Operating Instructions

### Start-up

The Packed Column Calculator program must be run under Windows 3.1, or a later compatible version of Windows, such as Windows 95.  If you are using the Windows 3.1 Program Manager double click on the Packed Column group icon and then on the Packed Column Calculator program icon.  If you are using Windows 95 click on the Start button, then on Programs and then on Packed Column.

After briefly displaying the program title banner the program will display the Disclaimer Screen.  As soon as you click the OK button the Data Entry screen will be displayed (See Figure 6.1).  You can either enter new data at this screen (See section 6.4 below) or you can load existing project data from a disk file (See sections 6.6 and 6.7)

When you have finished using Packed Column Calculator, and you want to return to Windows, click on the Quit button in the bottom right corner of the Data Entry screen, or press Alt Q from the same screen.  If you have not saved your data the program will warn you before it shuts down.

Figure 6.1  The Data Entry Screen

### Getting Online Help

Help is available in standard Windows 3.1 format.  At any step in the program where you need help, click on the Help button or press F1 to get help on the current operation, or use the menu Help function if you want to search for help on a particular topic.

Whether you have registered your copy or not, you are welcome to contact us for help with this program. While we cannot undertake to teach you packed column design, we will be pleased to help you with anything related to the program.  See section 7 for contact details.

### Setting Up the Calculation Parameters

The first step in performing a calculation is to set up the parameters that define the type of calculation. This is done by selecting amongst the options in the radio button boxes along the top of the data entry screen.

The first decision to make is whether you want to design an entirely new column, or whether you are analysing an existing column for a new duty or for repacking.  This decision is made by clicking one of the options in the first radio button box, i.e. the one on the left side labelled “Mode” (See Figure 6.1 above).  The basic difference between the “Design” and “Rating” options is that in design mode the program will calculate the required diameter, whereas in rating mode the program expects you to enter the diameter, and it will calculate the pressure drop and loading.

If you are not used to using radio button boxes, you should be aware that only one option can be selected at a time.  As soon as you select a new option the previously selected option will be de-selected.

The second option box allows you to toggle between the Eckert and Robbins design methods.  These two methods complement each other in that the Eckert GPDC is based on original work correlating flooding data that was then extended to the lower liquid flow regime, while the Robbins method is based on the flow of gas through dry beds of packing with extensions to the formulas to accommodate the flow of liquid.  These two method are discussed in greater detail in Appendix A.

We recommend that you use both of these methods for each design problem.  If you get good agreement between the two results it will add to your confidence in the design.  Where you do not get good agreement it will generally mean that you are working in a flow regime that is towards the limit of the correlations and you should carefully consider which method is likely to be more applicable.  Plotting the Operating Diagram and the GPDC graph will help you visualise the operating conditions.

The third option box allows you to select the basis that will be used in performing a new design.  It is of course only applicable if you are performing a new design and if you select the “Rating” mode the “Design Basis” option box will not be active.

The options available under “Design Basis” are Approach to Flood, Approach to Maximum Operating Capacity (MOC) and Pressure Drop.

The Approach to Flood is calculated by firstly using the Kister and Gill (1991) correlation to calculate the pressure drop at flooding for the selected packing, and then calculating the flowrates that would give this pressure drop.  The traffic is then limited to the specified percentage of the flooding traffic and the column sized accordingly.

The Approach to MOC (ie Maximum Operating Capacity) is calculated by assuming that the MOC is 95% of the capacity at flood (Kister and Gill (1991)).  The MOC is the point at which the column efficiency begins to fall off rapidly, although the column would not yet be flooded.

The pressure drop per unit height can also be specified as the design criterion for those applications where the pressure drop has an influence on the process.  Please be aware that some of the earlier references on column design suggested that there was a fixed pressure drop at which all packings started to flood.  These references generally assumed that flooding started at a pressure drop of 2 to 3 inches of water per foot of packed height.  Later work has shown that modern, low packing factor packings start flooding at lower pressure drops (but higher flowrates) than the traditional packings like Raschig Rings and Berl Saddles.  You will see on the graphical and tabular results produced by the Packed Column Calculator program that the flooding pressure drop is always calculated for the chosen packing.

The last option box allows you to select the units to be used.  You can accommodate mixed units by changing the selection in the middle of data entry to suit the available data.  For example, if all your data is in SI units, but the liquid viscosity is in centipoise, you should switch to metric units before you enter the viscosity and then back to SI once you have entered it.  The program will convert all previously entered data to the currently selected units.

There is one very important aspect to note with respect to the units.  The packing factor is always in the units as defined by Eckert.  Over the years various different “metricated” forms of the packing factor have been proposed.  Unfortunately there has not been much consistency in this regard and we have decided to avoid this confusion by using Eckert’s definition and units in all cases. This means that the “Y” parameter (ie the vertical axis) scale on the GPDC is adjusted to accommodate the packing factor units in each case.

For information on printing of the input information and the calculation results see section 6.10 below on "Printing the Calculation Results".

### Entering Data

#### Clearing the Worksheet

If you have been working on a design or rating, and then want to start work on an entirely new calculation, the existing information in the program can be cleared by using the menu function File | New.  This will clear the entire data entry worksheet as well as the heading information.  The program will warn you before it clears unsaved data.

If you want to change only some of the information you can just edit the applicable fields, and then use the File | Save As function to save the new calculation in its own file.  Please be aware that if you change data and then save using the File | Save function the original disk file will be over-written.

#### Entering and Editing Packing Data

The data that characterises the packing can be entered in one of two ways.  The easiest way is to simply select a packing from the built-in database.  The second way is to enter each factor individually and this will have to be done for packings that are not included in the database.  (If there are specific packings that you use regularly that we have not included please send us a data sheet and details of the supplier and we will arrange to have them included.)

In order to select a packing from the built-in database simply click on the “Select Packing” button under the table of packing details on the Data Entry Screen.  This can be seen in Figure 6.1 above.  Once you have clicked on this button the Packing Selection Window shown in Figure 6.4.2 will be displayed.

Figure 6.4.2  -  Packing Selection Window

When selecting from the packings database you must work from left to right, ie, first select the packing type, then the material and finally the size.  When all your selections have been made, click the “OK” button.  This will close the packing selection window and you will see that the applicable data will have been entered into the edit boxes on the Data Entry Screen.

If you want to enter the packing details yourself, simply click in the top edit box in the group on the left of the screen in the panel labelled Packing Details.  Enter the data you have available for each of the entry fields.  Please see the box below for details on how to move the cursor from one data field to the next.

The top four fields (i.e. Type, Material, Thickness and Size) are descriptive fields only and have no bearing on the actual calculation.  In fact only the Packing Factor has to be entered in order to perform a basic calculation.  The remaining information is used when you click the “Detailed Calc.” button if it has been entered.

A fundamental change was made in the model used for multi-field data entry forms when the world switched from DOS to Windows.  In DOS we used to press Enter every time we wanted to move to the next data field.  This would continue all the way through the form, until all fields had been filled.  In most Windows applications you must press Tab to move from one field to the next, or press Shift Tab to move to the previous field. Under Windows the Enter button is used to press the default button, which is usually the “OK” button.  The default button is the one with the darker outline (See the “OK” button in Figure 6.4.2).  As a compromise between these two models we have set up the Data Entry fields so that Enter or Tab can be used to move to the subsequent field, and the default button only becomes active when the last field has been edited.

#### Entering and Editing Process Information

The process information is entered in the group of edit boxes on the right of the screen in the panel labelled System Information (See Figure 6.1 above).  This is entered in the same way as described above for manually entering packing details.  In particular, you should read the box in section 6.4.2 which describes the convention adopted for moving the cursor from one field to the next.

All of the fields in this panel, except for the bed height, must be filled in before a Basic Calculation can be performed.  If any information is missing, the program will display a warning message and will highlight the empty field.  The only situation where not all the fields have to be filled in is if you are using the Robbins correlation and you have a zero liquid flowrate.  Under these circumstances you obviously do not have to fill in the liquid density and viscosity.

If you press the Detailed Calc button the program will attempt to calculate more parameters, but will simply ignore those for which it does not have sufficient information.

### Performing the Calculation

Once all the information has been entered as described above, you can click one of the Calc buttons to actually perform the calculation.  The difference between the two buttons (ie the Basic and the Detailed Calc buttons) is that the Basic Calculation only calculates the three items of information shown on the Data Input Screen, whereas the Detailed Calculation includes all of the items shown in Figure 6.5.

Do not be concerned about what input information is required before you click one of the Calc buttons.  If any information is missing the program will warn you and request it to be entered.

A hardcopy of the calculation results can be printed from the Data Entry Screen.  If you have performed a Detailed Calculation and you want to print out the results you must first close the window shown in Figure 6.5, and then press the Print button on the Data Entry Screen.  You should fill in the calculation description as described below in Section 6.8 before you print the results so that you can identify your printouts.

Figure 6.5  -  Detailed Calculation Results

#### Displaying the Generalised Pressure Drop Correlation

The internal calculations are of course all done algebraically, but it is reassuring to be able to visualise those calculations on a diagram, the way that we have all become accustomed to while doing them by hand.  The program therefore includes the capability of displaying the GPDC curves in the format proposed by Eckert.  The actual operating point is also plotted on the GPDC diagram so that you can get a feel for how close to the limits of the correlation you are.  A typical GPDC diagram is shown in Figure 6.5.1.

The scale on the vertical axis (ie the “Y” parameter) on the displayed diagram may be different from the printed diagrams you have used in the past.  The actual scale depends on the definition of “Y” and on the units used.  Details on the form of the “Y” parameter used here are given in Appendix A.  Note also that in the PackCol program the packing factor is always in the units originally proposed by Eckert.  This has been done to avoid the confusion that has been introduced over the years by various “metricated” packing factors.

Figure 6.5.1  -  Typical GPDC Diagram

When you start up the program you will see that the GPDC button on the Data Entry Screen is “grayed out” and is not enabled.  As soon as you have performed a Basic or Detailed Calculation the GPDC button will be enabled.  This is provided that you have selected the Eckert correlation.  If you are using the Robbins correlation you cannot display the GPDC.

The GPDC diagram can be printed by pressing the Print button on the window shown in Figure 6.5.1.  This will give a print out that includes the project description and the main calculation results.

#### Displaying the Operating Diagram

Although every text book dealing with the design of packed columns refers to the operating diagram, it is very rare for these diagrams to be prepared where the calculations are being done by hand.  It is a tedious job to do a single design by hand, and the need to perform the hundreds of designs necessary to prepare a meaningful operating diagram makes it virtually impossible by hand.  The computer can do this for you in a few seconds, and you can immediately see the sensitivity to changes in gas and liquid flowrates.

The Operating Diagram button on the Data Entry Screen is “grayed out” and disabled until you have performed a calculation.  As soon as the calculation results are displayed this button (labelled “Op. Diagram” - see Figure 6.1 above) will be enabled.  Clicking on the button, or pressing Alt O, will display a window with the operating diagram.

The format of the operating diagram will vary slightly according to the correlation being used.  Figure 6.5.2a shows a typical operating diagram when the Eckert correlation has been used, and Figure 6.5.2b shows the operating diagram for the same input data, but using the Robbins correlation.  Note that these example diagrams are not based on the same data as Figure 6.5.1.

In both forms of the operating diagram the lines shown indicate constant pressure drop conditions.  The ranges for the gas and liquid flowrates extend from zero to approximately twice the design rates.  The flooding curve is plotted if it applies to this range, and as many of the standard pressure drop lines as possible are shown as well.

The operating diagram can be printed by pressing the Print button on the windows shown in Figure 6.5.2a and 6.5.2b.  This will give a print out that includes the project description and the main calculation results.

Figure 6.5.2a - Operating Diagram based on Eckert Correlation.

The Eckert correlation is limited to a range of 0.1 to 4.0 of the “X” parameter (see definition in Appendix A), and to a pressure range from 0.05 inchWG/ft up to the flooding pressure drop.  The pressure drop curves above the flooding line are shown as well to aid visual interpolation, but of course they are not strictly applicable.

Note that the limiting lines of constant “X” parameter are vertical in the GPDC diagram, but are straight lines through the origin on the operating diagram as their slopes represent the liquid to gas flowrate ratios.

The liquid rate can vary from zero up to the correlation limit for the Robbins correlation.  This limit is shown if it occurs within the plotted range.

In the example shown the liquid to gas ratio is relatively high, and the correlation limit has been reached.

Figure 6.5.2b - Operating Diagram based on Robbins Correlation.

In some operating diagrams produced using the Robbins correlation it is interesting to see how the slope of the constant pressure drop lines changes rapidly as the liquid flowrate approaches zero for the higher pressure drops.  This is probably why Eckert and those who have used the GPDC approach have limited the gas to liquid ratio factor X to a minimum of 0.1.  If you are designing for very low liquid rates you should be aware that the pressure drop rises very rapidly with liquid flowrate initially, before becoming more gentle.  This is a perfect example of the need for the operating diagram, rather than designing for a single point.

### Reading Existing Files of Data

Column and system information that you have previously entered and saved, can be read back from disk into the program.  To do this, use the File | Open command on the main menu which is displayed on the Data Entry screen.  The menu system can be by-passed completely by simply pressing F3 from the Data Entry screen.

When you open a file it will over-write anything that is currently in memory.  If you try to open a file while you have unsaved information in the spreadsheet, the program will warn you before it over-writes the existing information.

After you have invoked the File | Open command a standard Windows dialog box will be opened.  The dialog box will default to the current directory and will display all the Packed Column Calculator data files it finds there.  (PackCol data files have an .COL file extension.)  If you want to load a file from a different directory or drive use the middle section of the dialog box to browse to the directory you want.  Once you have the correct directory displayed you can scroll up and down with the window on the left of the dialog box until you find the file you want.  When you have located the required file, click on it and then click on the OK button, or simply double click on the name of the file.

The file will then be loaded and displayed on the Data Entry Screen.  The project heading and description information is also loaded, but is not immediately displayed.  If you want to see the project heading use the Edit | Description menu command.

### Saving Your Data to Disk

Once you have entered the data for a new calculation you may want to save it for later use.  This is done with the File | Save or the File | Save As commands on the main menu which is displayed on the Data Entry screen.  A short cut for the File | Save command is to press F2.

The first time you use File | Save on new data the program will open a dialog box asking you to specify the drive, directory and name for the file.  The drive and directory are specified as usual by browsing with the central portion of the dialog box.  The file name must be typed in the edit box on the top left of the dialog box.  The file name can be any legal 8 character combination.  You do not have to type the extension; PackCol will automatically add the required .COL extension for you.

Please note that if the current data was loaded from a disk file, or if you have already saved the data, the File | Save (or F2) command immediately saves your data using the same filename that was used before.  The old file on the disk is over written without asking you to confirm the filename, and the old file cannot be retrieved.  If you want to save the data in a different file use the File | Save As command.

If you have existing data that you want to modify and save under a different name, or if you want to save a copy of your file to a different directory, use the File | Save As command.  This will give you a chance to specify the new drive, directory or filename before the file is saved to disk.

The project heading and description information are also saved with the data.  If you want to add or edit the project heading use the procedure described below in the section entitled "Editing the Project Description".

### Editing the Project Description

You can also save descriptive information specifying the client name, project name, project number, column reference and the name of the designer.  This information is added or edited by using the Edit | Description command on the main menu.  Once this information has been added it will be automatically saved whenever the cost data is saved.  This window is shown in Figure 6.8.

This information will be included on all printed reports you generate with Packed Column Calculator to identify your work.

Figure 6.8  Editing the Project Heading

### Entering your Registration Key

In order to speed up the delivery of registered copies of Packed Column Calculator we use a software key (password) system which unlocks and converts your evaluation copy into a full registered version.  This enables us to email or fax the software key immediately upon notification of your registration, rather than having to mail you a new disk.

When you place your order with one of our agents you may (depending on from whom you order and how you order) be given a temporary key which allows you to convert your evaluation copy to a registered copy within minutes of deciding to purchase.  This temporary key will work for a few weeks, but we will send you your permanent key within a few days.  (This procedure depends on the order method you use.  Please see Appendix E for full details of the available order methods.)

Once you have received the temporary software key you can use the Register | Temporary function on the main menu to create your registered version.  You will be asked to enter your company name as you want it displayed on the printouts, and then you will be asked to type in the software key.  Note that the key contains upper case alphabetic characters only.  It contains no lower case letters, and no numbers.  For example an "O" will be an "OH" and not a zero.  Similarly you can be sure an "I" is an "EYE" and not a one or a lower case “ELL”.

When you receive your permanent key please follow a similar procedure, except that you will use the Register | Permanent menu function.  Please ensure that you make a note of your registration key, and keep it in a safe place.  A space has been provided on the front of this manual for you to make a note of your permanent key.  You will need this key whenever you install the program onto another computer.

Your permanent key is linked to your company name.  When using the Register | Permanent function you must enter your company name exactly as it is shown on the email message or fax that gives you your permanent key.

Part of the information contained in the key is the latest available version number of this program.  If you are using an out of date evaluation copy, the program will advise you that you need to download the newest version.  If possible, the key will unlock the old version you are using and you will be able to use it until you download the new one.

The permanent key will work with all upgrades released within 12 months of your date of purchase.  If you have supplied us with an email address we will advise you whenever an upgrade is released, and we will also let you know from where you can download the new file.

Please do not pass your temporary or permanent keys on to others.  Packed Column Calculator is distributed as shareware and we will be very grateful if you do pass evaluation copies on to others, but please keep your keys to yourself.  We have to be able to make some profit in order to stay in business and keep you supplied with upgrades and new titles!

### Printing the Calculation Results

The column details and system information shown on the Data Entry screen, together with the project description and calculation results, can be sent to your printer by using the Print button in the centre near the bottom of the window.  This will bring up the standard Windows print dialog box that allows you to set the print quality and the number of copies, or to call the printer setup dialog box for performing a detailed setup or selection for the printer.

The Calculation Results report is designed to be printed in portrait orientation.  It will automatically scale itself to try to accommodate the selected paper size, but will work best with Letter sized paper (8.5" x 11") or A4 sized paper (210mm x 297 mm).

In order to be able to identify your reports, make sure that you have entered the project description using the Edit | Description option on the Main Menu.

You can also make printouts of the GPDC diagram and of the operating diagram.  While these diagrams are displayed there will be a Print button near the bottom of the window containing the diagram.  Clicking on the button will send the diagram, together with the project description and the packing and system information to the printer.

### Exiting from the Program

In order to exit from Packed Column Calculator you must be at the Data Entry screen.  If you are not already there, close the current window first.  Once you are at the Data Entry screen click the Quit button, or press Alt Q, or use the Menu function File | Exit.

If you have not saved your data, PackCol will ask if you want to save it before you exit.

## Software Support

If you have any problems that you cannot solve by reading the on-line help or the manual, you are welcome to contact us.  When describing the problem please be as detailed as possible, and include your packing and system information.  If you have operational problems include details of your hardware, operating system and so on.

You are welcome to contact us whether you have registered or not.  We cannot undertake to teach you how to design packed columns, but we will address all problems experienced with the PackCol program.

Contact us by email at:

katmar@dbn.lia.net

Or write to us at :

Katmar Software

PO Box 654

Westville 3630

South Africa.

Or visit our WWW homepage at:

http://users.lia.net/katmar/

## Bug Reports and Wish Lists

We have put many hours into the checking of this programme and were not aware of any bugs at the time of release.  However, we are also aware that the most subtle bugs are unearthed after release.  We are therefore most grateful to those who report bugs to us.  Please give as much detail as possible on the bug so that we can re-create the circumstances on our equipment.

Our experience gained over many years and with the development of several packages has shown us that no matter how much effort we put into designing and planning, we can never hope to match the combined creativity of our users.  We would like to hear of any features you would like to see changed or added to the Packed Column Calculator program.

We would like to hear from you whether you have registered or not because all comments that can help us improve our programs are much appreciated.

## Revision History

Version		Date		Comment

1.0		Jun 98		First public release

1.1		Aug 98		Added operating diagram, fixed bug in Save routine

# Part 2 - Tutorial

## Detailed Worked Examples

This tutorial is not designed to teach you how to design packed columns.  It is assumed that you are already capable of that.  The PackCol program is designed as a tool to make packed column hydraulic design easier and faster for trained engineers.  This tutorial is therefore aimed only at teaching you how to use the PackCol program.  If you work through the examples that follow you will learn how to use all the facilities built into the program.  Various graphics are included to show you what the screen will look like as you work through the examples.  It is preferable to actually run these examples on your computer, and to use the included screen shots only as checks that you have followed the correct procedure rather than to simply read through the tutorial.

The pace of this tutorial is very slow.  The tutorial may be used by engineers with a very wide range of computer skills, and if you find that certain sections are too slow and detailed for you, please just skip them.  If you find that the whole thing simply insults your intelligence please read Section 4 above, which is entitled “Quick Start for Experienced Users”.  This may well be all you need to get up and running as the program is specifically designed to be easy to use.

Start up the program and click the OK button on the disclaimer screen so that your computer displays the Data Entry screen.  This is shown in Figure T-1.  This screen is made up of six areas.  These are :-

o   The menu across the top of the screen,

o   The four option boxes just below the menu,

o   The Packing Details panel on the left centre of the screen

o   The System Information panel on the upper right section of the screen,

o   The Calculation Results panel on the lower right section of the screen.

o   The control buttons along the bottom of the screen

Make sure that your screen looks like that shown in Figure T-1, and identify each of these six areas.  Note particularly that the three buttons on the left along the bottom of the screen are “greyed out” at this stage.

Use your mouse to click alternately on the “Design” and “Rating” options in the Mode option box on the top left.  Note that every time you click on the “Rating” option, the three options in the third option box (ie the Design Basis option box) are “greyed out”, and when you click on the “Design” option they are re-enabled.  Note also that as you switch between “Design” and “Rating” some of the labels and units in the “Calculation Results” panel change.

Figure T-1   The Data Entry Screen

If you leave the Mode option set on “Design” and click on the various options in the Design Basis box you will see that the second last label and the units in the System Information panel change to agree with the selected option.  Similarly, when you click on the different options in the Units option box the units are changed where applicable.

Now that you have got a feel for the Data Entry Screen let us try an example.

Example 1

Problem Statement :

Design a new column, to be packed with 2 inch Raschig Rings of 1.6 mm carbon steel, for the following duty:-

Gas Flowrate		3.5 kg/s

Gas Density		1.8 kg/m3

Liquid Flowrate		12.0 kg/s

Liquid Density		875 kg/m3

Liquid Viscosity		3.5 centipoise

Pressure Drop		0.4 inch WG/ft

Bed Height		3.0 m

Solution :

Start off by setting the four option boxes across the top of the screen.  The left option box must be set to “Design” as we want to design a new column.  The second option box allows us to select the correlation to be used.  Select the “Eckert (1975)” option on the top line at this stage (we will have a look at the effect of the correlation selection later).  In the third option box you need to select “Pressure Drop” as this is the design basis specified in the problem statement above.  Select “Metric” in the Units option box.

At the bottom of the Packing Details panel there is a button labelled Select Packing.  Click on it, or press Alt S.  This will bring up the Packing Selection screen shown in Figure T-2.  The three option boxes on this screen need to be filled in from left to right, ie first select the packing type, then the packing material and finally the packing size.

Click on a few of the different packing types in the left box.  You will see that a picture of each appears in the bottom right hand corner of the window as you select it.  You will also see that the middle and right option boxes change for each packing selection.  Now select “Raschig Ring” at the top of the list.  The material specified is 1.6mm carbon steel, so you need to click on the last item in the centre option box.  Finally, select the 2.0 inch (50mm) size and click the OK button.  You will be returned to the Data Entry screen, and the information for each of the fields in the Packing Details panel will be filled in.

Figure T-2   The Packing Selection Screen

When you are returned to the Data Entry screen the first edit box in the System Information panel will be highlighted, waiting for you to key in the gas flowrate.  Type in the number given above.  Once you have entered this number you need to move the cursor to the gas density edit box.

The default procedure to advance the cursor in Windows programs is to press the Tab button, whereas in the days of DOS (and before) the accepted procedure was to press Enter.  We have implemented both of these methods in PackCol so you can use whichever you are more comfortable with.  However, implementing both of these defaults does mean that the Enter button does not do what you would expect in a Windows program i.e. you would normally expect the Enter button to activate the default button.  In PackCol’s Data Entry screen there is no default button until all the fields have been filled in.  This seems to be the best compromise between a pure Windows procedure and what many users have become used to over the years.

Carry on in this way to fill in the liquid flowrate and properties.  When you get to the edit box for entering the Pressure Drop you will see that it is expecting the pressure drop to be entered in mm WG/m, but the problem statement above gives the pressure drop in inch WG/ft.  Before you enter the pressure drop, click with the mouse on the “English” option in the Units option box in the top right corner of the screen.

You will see that the label for the Pressure Drop units in the System Information panel has changed to inch WG/ft.  Note also that all the other labels for the units in the System Information panel (and actually in all the panels) have changed to English units.  It is important to note that not only have the units changed, but the values already entered have been converted to the new selection of units.  This allows you to enter the values in whatever units you have them in, and then convert them to the units in which you want to display your calculation results.

Let us get back to the specifics of our example.  You have selected the “English” units option.  Now click in the edit box for the pressure drop and enter “0.4”, and then click on “Metric” in the units option box again.  The value of 0.4 inch WG/ft will be converted to 33.366 mm WG/m.

Fill in the final edit box with the Bed Height, and then check that all your entries are correct.  In order to actually perform the calculation, click on the “Basic Calc” button in the Calculation Results panel.  Depending on the speed of you computer, the calculated values will appear almost immediately in the Calculation Results panel.  At this stage your screen should look like that in Figure T-3.

The “Basic Calc” gives the three main results.  In this case they are the “Approach to Flood”, the “Approach to MOC” (ie Maximum Operating Capacity) and the “Column Diameter” which satisfy the specified pressure drop.

Figure T-3   The Completed Data Entry Screen

That was quite a bit easier than doing it by the traditional hand method wasn’t it?  No more struggling to interpolate on log-log plots.  No more scratching through manuals and catalogues to find packing factors.  No more units conversion hassles.  Thank goodness for computers!

At this stage let us save this data so that we can use it again later.  Use the File | Save menu function, or simply press F2, to bring up the standard Windows file save dialog box.  Type in the file name “MYEXAMPL” in the edit box in the top left corner of the dialog box.  Note that the program automatically adds the required “.COL” file extension.  Click the OK button, and the file will be saved to disk.

Now press F2 again.  This time the file save dialog box does not appear, but the disk light comes on and the file is clearly being saved.  This is because the data in the program is already associated with the file name “MYEXAMPL” and it is simply re-saved to that file name.  If you change some of the data and you want to save it to a different file name you must use the File | Save As menu function.  If you change the data and then simply press F2 the original data file will be over written with the new data.

We can now “play” with the data, knowing that it is safely strored on disk where we can retrieve it.  Imagine you have a client who wants to work in English units, but you have just done all your calculations in metric units.  Click on the “English” option in the Units option box and then click on the “Basic Calc” button again.  All your input data and the calculation results are now in the units required by your client!

Probably the next thing you would want to do is to print out these results for filing.  But before we do that we must label the data so that the print-outs will be easily identifiable.  Use the Edit | Description menu function to bring up the Project Heading dialog box.  This is shown in Figure T-4.  Key in any data you like to identify your data and calculation.

Once you have typed in all your information click on the OK button, and you will be returned to the Data Entry screen.  We want the description we have entered to be saved with our data, so press F2 again.

In order to test that the saving is working properly, use the File | New menu function to clear the Data Entry screen.  Now press F3 (or use the File | Open menu function) to call up the file open dialog box.  Select “MYEXAMPL” from the file list and click the OK button.  All your data will reappear in the Data Entry screen. Use Edit | Description to confirm that the description has also been restored.

Figure T-4   The Project Heading Dialog Box

Now we can get back to printing out the input data and calculation results.  If you have followed the process exactly as above the Calculation Results panel will have no data and the Print button in the middle of the bottom row will be “greyed out”.  First select the units you want.  Click on the Basic Calc button and you will see that the results appear and the Print button is enabled.  Click the Print button and you will see a standard Windows print dialog box appear.  Click the OK button and in no time at all you will have a very neat record of your calculation.

You will see that the printed report gives a lot more information than you saw on the Data Entry screen.  It is not necessary to print it out to see it.  Simply click the “Detail Calc” button on the Data Entry screen and a window will pop up with this additional information.  When you click the Close button on the Detailed Results screen you will be returned to the Data Entry screen.

The G.P.D.C. button, which was initially “greyed out” has become enabled by you clicking on a Calc button.  This will only happen if you have set the Correlation option box at the top of the screen to the Eckert (1975) option as it is based on the Generalised Pressure Drop Correlation.  Click the G.P.D.C. button.  A plot of the GPDC will be displayed with the current operating point and the flooding curve for the chosen packing superimposed on the plot.  This is shown in Figure T-5.  The values of the Y parameter on this curve may not match those on the GPDC plot that you have used previously.  This is discussed in Appendix A.

The GPDC plot can be printed out by clicking the Print button while the plot is displayed on the screen.  When you print out the GPDC plot it will also print the Project Heading, the input data and the principal calculation results.

While the GPDC plot is useful as a design tool, it is difficult to use it for sensitivity analysis.  This is because the log-log plot and the functions plotted do not lend them-selves to easy interpolation, and the gas flowrate appears on both axes.  Sensitivity analysis is better done with a standard Operating Diagram plot as shown in Figure T-6.

Figure T-5   The Generalised Pressure Drop Correlation Plot

The Operating Diagram shows lines of constant pressure drop on a plot of gas flowrate vs liquid flowrate.  This makes it very easy to see how the pressure drop varies with changes in gas or liquid flowrate.  The plot created also shows the Flooding curve if it appears in the region plotted.  The units used for the flowrates and for the pressure drop curves will be the same as those selected on the Data Entry screen.  The procedure for printing the Operating Diagram is identical to that for the GPDC plot.

Note that the limits of the X parameter become sloped lines through the origin on the Operating Diagram because they represent constant Liquid to Gas flowrate ratios.  This of course only applies when using the Eckert (1975) correlation.  We will see what an Operating Diagram based on the Robbins (1991) correlation looks like later.

The Operating Digram will also show the minimum liquid rate for adequate wetting of the packing if the necessary data was entered, and if it appears on the plotted region.

Figure T-6   The Operating Diagram  (Eckert 1975)

Now we will have a look at the effect of changing the correlation.  Return to the Data Entry screen by closing the plots if they are still open.  If you still have the data from the MYEXAMPL file intact you can proceed directly.  If you have changed any of the input values re-read the file by using the File | Open menu command.

Once you have the correct data in the Data Entry screen, click on the Robbins (1991) option in the Correlation option box.  As soon as you do this any results visible in the Calculation Results panel will disappear.  Click on the Basic Calc button and the results will reappear, but will be slightly different from what they were before.  The results from the two correlations are compared below.

|  |  | Eckert | Robbins |
| --- | --- | --- | --- |
| Approach to Flood | % | 65.5 | 66.5 |
| Approach to MOC | % | 68.9 | 70.0 |
| Column Diameter | m | 1.690 | 1.696 |

As you can see, the results using the two different methods are very similar.  You should try both methods for each problem you face.  It is as easy as clicking a button.  For critical designs you should use a completely independent design method.  Your packing supplier should always be willing to confirm your final calculations.

Now click on the Op. Diagram button.  An operating diagram very similar to that seen before will be displayed.  This is shown in Figure T-7.

Note that the sloped limiting lines have disappeared as the Robbins method is applicable right down to zero liquid flowrates.  This makes it useful for calculations when the packing is essentially dry, as is the case when a packing is used as a droplet separator.

Figure T-7   The Operating Diagram  (Robbins 1991)

You have now worked through a complete design problem, and should have a good grasp of the workings of the Packed Column Calculator program.  Whereas the first example was based on a problem where we wanted to design a new column, we will now tackle a rating type problem.

Example 2

Problem Statement :

Rate an existing column of 1.70m diameter and packed with 2 inch Raschig Rings of 1.6 mm carbon steel, for the following duty:-

Gas Flowrate		3.5 kg/s

Gas Density		1.8 kg/m3

Liquid Flowrate		15.0 kg/s

Liquid Density		875 kg/m3

Liquid Viscosity		3.5 centipoise

Bed Height		3.0 m

Note that this is very similar to the first problem - only the liquid rate has changed slightly.

Solution :

Start off by opening the file you saved for the data in the first example.  Now we must set the option boxes across the top of the screen for the new situation.  The left option box must be set to “Rating” as we want to rate an existing column.  The second option box allows us to select the correlation to be used.  Select the “Eckert (1975)” option on the top line at this stage (we will have a look at the effect of the correlation selection later).  The third option box will be “greyed out” as we do not need to select a design basis.  Select “Metric” in the Units option box.

Change the liquid flowrate to 15.0 kg/s.  You will also need to key in the diameter of the existing column in the edit box labelled “Column Diameter” in the System Information panel.

Now click on the “Basic Calc.” button.  Note that the pressure drop is now included in the “Calculation Results” panel in the place of the Column Diameter which was displayed when we solved Problem 1.  You can test the Detailed Calculation procedure and the plotting of the GPDC and Op. Diagram.  These all still function just as they did before.

There is one difference that we would like to illustrate here.  Change the correlation selection to Robbins (1991) and click on the “Basic Calc” button.  Now click on the “Op. Diagram” button.  The diagram shown in Figure T-8 will be displayed.  Note that on the right hand side of the plot the liquid to gas ratio has become too high for the Robbins correlation.  The PackCol program will always warn you if you are trying to operate in a zone outside the correlation limits.  Switch back to the Eckert correlation and vary the gas and liquid flowrates to see how the program warns you when you exceed the boundaries of the correlation.

There is one situation where the program will give you a warning, but will still try to calculate the results.  This is where the liquid rate is too low to wet the packing properly.  This low liquid rate will not affect the accuracy of the hydraulic calculations, but the mass transfer efficiency of the column will be decreased if the packing is partially dry.

Figure T-8   The Operating Diagram  (Robbins 1991)

# Part 3 - Appendices

## APPENDIX A - Discussion of Eckert and Robbins Correlations

The Generalised Pressure Drop Correlation, as presented by Eckert (1975), developed from early work by Sherwood and others on correlating the flooding point.  They noticed that the flood point tended to always be at the same pressure drop and various attempts were made to include lower pressure drops on the flooding diagram.

Although the GPDC seems to be fairly universal, it is only on the shape of the curves that there is good agreement between the different workers.  The ordinate used on the diagram has varied over the years, and varies from one worker to the next.  In early work by Eckert (1970) the ordinate included the liquid density squared in the denominator.  In his 1975 article Eckert reduced the exponent on the liquid density from 2 to 1, without any explanation.

This change may have been prompted by the work of Prahl (1970) who claimed that it was a mistake to include the liquid density in the ordinate at all.  Prahl describes a “mental experiment” involving a very dense liquid.  In this experiment, the 1970 version of the Eckert GPDC predicts that the pressure drop for a given gas rate is considerably less when irrigated with the dense liquid than when there is no liquid flow at all.  This is clearly impossible.  Even the 1975 correlation does not cope with this mental experiment.  Prahl therefore suggests that the exponent on the liquid density should be reduced from two to zero.

On the other hand, Leva (1992) claimed that Eckert had made his correlation less accurate by decreasing the exponent on the liquid density from two to one.  Although Leva (1992) does not explicitly put an exponent to the liquid density, he gives a graphical representation of the effect of the liquid density.  From this graph it is clear that the exponent used by Leva is 1.5.

It should be borne in mind that the bulk of the data originally used to develop the GPDC curves was based on the air-water system.  It is therefore probably unfair to try to use the GPDC for liquids with densities very different from water.  The liquid in Prahl’s mental experiment had an SG of 2.89.  We have put much tighter limits on the density range permissable in PackCol, and would suggest that if you have liquids with very high or very low densities you should try to get experimental results that you can use to base your design on.

Clearly, more work needs to be done on the effect of liquid density.  It is a huge achievement by the workers in the field that it is possible to model such a complex system as a packed column with a simple graph and a single characterisation factor for each packing.  It is probable that the effect of the liquid density will vary with the approach to flooding.  If we can indulge in a “mental experiment” of our own, we can try to visualise this.  At a low liquid rate a change in liquid density will have a small change on the free passage for the gas, and will probably not have much effect on the pressure drop.  Conversely, if the column is close to flooding a change in the liquid density will have a relatively large effect on the free space for the gas and a large effect on the pressure drop.

It should also be noted that in most references on the GPDC the ordinate also includes the gravitational constant gc.  The GPDC diagrams generated by PackCol assume that gc has been dropped from the ordinate and the Y scale adjusted accordingly.  This may not please the purists, but it makes calculations easier.

In the PackCol program, the ordinate on the GPDC always includes the packing factor in Eckert’s original units, irrespective of what selection of units has been made.  This has been done to avoid the confusion that has been caused over the years as various “Metricated” packing factors have been introduced by different researchers and vendors.  The difference in the packing factor units is accommodated by adjusting the scale on the Y axis.

Whereas the GPDC was based on work on the flooding point that was then extended to the lower pressure drop region, the Robbins (1991) correlation starts with the assumption that the flow of gas through a dry bed of packing can be modeled with a modified orifice equation.  Robbins uses experimental data to show that this is true, and then goes on to modify the equation to account for the effect of irrigating the packing.

The algebraic method proposed by Robbins is limited to fairly low liquid rates.  He does include a graphical method to extend this to higher liquid rates.  No attempt has been made in PackCol to include the range covered by the graphical method.  The Robbins correlation has been included to add confidence to calculations involving low liquid rates, as it has a stronger theoretical base in this regime.  If you need to do calculations close to the flooding point it would probably be better to use the GPDC than to try to extend the Robbins method too far from its theoretical base.

## APPENDIX B - Accuracy Considerations in Packed Column Design

A great deal has been written about the accuracy of the various design methods for packed columns, and unfortunately some of the writings have been of the “my method is better than yours” type.  By selective use of the available data it is possible to show that any particular correlation is better than any other.  It is therefore difficult to objectively say which is the best method.

However, the purpose of this Appendix is not to prove which is the best design method.  We believe that the two correlations included in this program are as reliable as any other non-proprietary design methods available. What follows is a discussion of what factors are likely to have the biggest influence on causing a packed column to behave differently from what it was designed to do.  If you are able to minimise these effects you will be able to improve the confidence in your designs.  As we have said repeatedly, your final designs should always be checked by independent means.  Your packing supplier will have the means to check any calculations on equipment using their packings.

One of the biggest problems in comparing research data is that each worker seems to have a different definition of flooding.  When a scatter diagram is plotted showing calculated and predicted flooding points from different sources, it is difficult to know if you are comparing all data on the same basis.

Probably the biggest influence on the final performance of a packed column is the liquid distribution.  Maldistribution of the liquid can throw your pressure drop calculations out, and also your HETP calculations.  A good liquid distribution system may seem expensive, but its cost will be small relative to the cost of the whole column.  The liquid distributor is the last place to try to save money.

Tall columns will need liquid re-distributors as well.  The spacing of these depends on the packing type. Follow your manuafacturer’s recommendations.  If beds are made too deep the packing pieces may become distorted at the bottom of the bed.  Thus beds that are too deep can result in maldistribution and/or high pressure drops.

The gas distributor is much less critical than the liquid distributor.  As long as the open area of the support plate is close to the cross sectional area of the column it will not have a large influence.

The packing factors quoted in the literature are average figures.  It is known that these vary slightly with the flow regime.  You may be able to get more detailed tables of packing factors from your supplier.

The ratio of the column diameter to the packing diameter is important in that if it is too small the packing will not be truly random and the voids between the column wall and the packing pieces will reduce the pressure drop.  For Raschig Rings this ratio should be at least 20:1 and preferably 30:1.  More modern packings can function down to 10:1.  Follow your manufacturer’s recommendations.  This factor can lead to problems when scaling up from lab work where small diameter columns are used.

Generic packings (such as Raschig Rings, Berl Saddles and Pall Rings) can vary in dimensions from supplier to supplier.  Ensure that you use the correct packing factor from the supplier.  Even packings from a single supplier can vary.  This is particularly true of ceramic packings.

Packing factors can also vary with time.  If you have ever seen packings taken from a column that has operated for some time you will be aware of how they can change shape through breakage, corrosion and bad treatment.  New plastic and stainless steel packings often do not wet easily (unless specially treated) and the pressure drop may vary slightly over the first week or so of operation.

The packing factor achieved in a column is also dependent on the method of installing the packing.  If the packing is dumped dry into the column it will tend to compact more than if the column is first filled with liquid.  This results in more pieces per unit volume and higher pressure drops.

## APPENDIX C - The Shareware Distribution System

Although shareware is often thought of as a type of software, it is actually a form of software distribution.  This section gives details on the shareware system and how it works.

The shareware distribution system developed in the environment of free enterprise and innovation that characterised the PC industry in the early 1980's.  It has continued to grow and today there is a huge variety of high quality software available at very reasonable prices.  Shareware in fact covers a wider range of subjects than commercial software.  Some of the areas that  are particularly well represented are databases, word processors, spreadsheets, CAD, games, utilities, menuing systems, education,         accounting and programming support.

A support industry that included magazines, disk distributors and bulletin board systems grew up around the shareware system.  These have now been largely supplanted by the Internet, the online services such as CompuServe and AOL, and the CD-ROM distributors.  Today you can search amongst 10's of thousands of programmes on the Internet and download a trial copy in a matter of minutes.

If you are new to shareware you may like to have a look at some of the software available from the Internet archives.   There is no charge for downloading trial copies and only if you like the software, and decide to continue using it, do you pay the author.

Some of the better archives on the Internet are :-

Public (software) Library.  aka PsL.  This is one of the original shareware businesses and one of the very few that has kept up with the evolution of the shareware system.  Actually that is an unfair statement.  PsL, and its owner Nelson Ford, have probably done more than any other organization to set the course of shareware and keep it abreast of the rest of the computer world.  The ultimate in professional shareware.  Here you can search, download and pay for your shareware.  They also publish excellent compilation CD-ROMs.  What more can I say?  Try them first.

http://www.pslweb.com/

Ziff-Davis ZD-NET.  This is a general shareware / freeware / demoware site that can be browsed or searched.  It is very well organised and they review and rate all software they carry.

http://www.hotfiles.com/

Albert's Ambry.  This is one of a new category of sites where you can search for software, download an evaluation copy and then pay for and download the registered version if you decide to buy.

http://www.alberts.com/

Download. Another general shareware / freeware / demoware site that can be browsed or searched.

http://www.download.com/

Winsite.  Specialising in Windows shareware. (Does anyone remember an operating system called DOS?  What about CP/M ??)

http://www.winsite.com/

Garbo.   Another of the classic shareware organisations.  Located at the Business School of the University of Uwasa in Finland and run by the inimitable Prof Timo Salmi.  You won't find any games here (OK - maybe a few educational ones), this is serious stuff.  If you are looking for business, maths and stats utilities or Pascal programming tips (Timo's speciality) this is the place to look.  Unfortunately, due to resource constraints, they have not kept up to date with the latest shareware available.  However, if you are looking for something fairly specialised or technical, and you don’t mind an old fashioned interface, this is the place to look.

http://garbo.uwasa.fi/

SimTel.    Another of the classics.  This one is mirrored at many sites around the world and there is a good chance that there will be a mirror just around the corner from you.

http://www.simtel.net/simtel.net/

ASME.   A specialist engineering archive is maintained by the American Society of Mechanical Engineers.  Very well organised into categories, with lists of file descriptions that can be downloaded and read off-line.  It should be compulsory for all engineers to bookmark this page.

http://www.asme.mecheng.org/

Shareware.Com    This site does not archive any shareware itself, but maintains a constantly updated index to the major shareware archives.  It is very useful if you are searching for something specific.

http://www.shareware.com/

The basic concept of shareware is that the author retains the copyright and all other commercial rights to the software and documentation, while allowing, and indeed encouraging, wide copying and distribution of the package.

There are a few restrictions that authors place on distribution.  All copies must be full copies of the entire package with no modifications.  Copies may not be sold for more than the cost of the disk, the labour of copying and a reasonable profit.  When selling copies the distributor must state that the software is free for evaluation purposes only and that payment must be made to the author if continued use is made of the programme.

There are many advantages in shareware to users and authors.  The cost to the user is low because no money is wasted on advertising and fancy packaging that add no real value to the software.  The biggest advantage to the user is that it is a true "try-before-you-buy" system.  The software can be fully evaluated on the user's hardware and in his/her own environment before payment is  made.  Shareware therefore has the ultimate money-back guarantee -- if you don't use the product, you don't pay for it.

The user is also closer to the author and is able to write directly to him for help and advice.  Shareware authors generally encourage users to send in "wish-lists" of desired features, and are quite responsive to those requests.  In effect the users become the author's market research department.

There are, of course, also advantages to the authors who use shareware to distribute their products.  The cost of launching a new product via shareware is much less than via normal commercial channels.  This has allowed smaller software houses to market innovative and highly specialised software that would otherwise simply not be profitable.  Running costs are also lower as there are almost no marketing or advertising costs.

The success of the shareware system has prompted many of the commercial software companies to use the same distribution channels for trial versions and demo versions of their software.  This has led to a blurring of the distinction between shareware and commercial software.

This blurring has also been caused by the fact that there are various "degrees" and definitions of shareware.  In its purest form, shareware software is fully functional and can be used for as long as it takes the user to decide to pay for it.  Unfortunately very few users do ever pay for it under these circumstances. (But some do - we have had a fair number of registrations for a DOS steam tables program called WASP which was pure shareware.)  So authors have worked out a variety of ways to "encourage" users to pay.  Some authors offer extra features or printed manuals with their registered versions.  Some "shareware" is nothing more than a demonstration.  Some authors program time limits into the software to enforce the 30 day trial period.  Another system widely used is to put watermarks on printed reports.  Some authors disable the most useful functions in the evaluation versions.  A common system is to add "nag screens" that pop up randomly to remind the user to pay.

All in all, these various schemes have resulted in confusion over what shareware really is.  A wide variety of names have been coined to describe the different systems eg DemoWare, NagWare, TrialWare, and even BeWare, but mostly they all go under the generic title of shareware.

We have put an enormous amount of thought into the "incentives" that we have built into this program.  We believe that the user should be able to see and test every feature of the program, including the manual, before being asked to part with any money.  There should be absolutely no doubt in the user's mind as to whether the program is suitable or not.  With this in mind, we have made the evaluation version of this program fully functional, but we have programmed a time limit into it and have put watermarks on some of the printouts.  There is also a restricted range of packing factors that can be used.  We hope that you find this an acceptable compromise and would appreciate your comments on what you believe is a fair system.

Registration agents are another aspect of shareware that work to the user's advantage.  These organizations take registrations on behalf of the shareware authors.  By registering via an agent the user is assured that the shareware author is still around and still supports his software.  The agent would stop taking registrations as soon as anyone complained that they had not received support from that author.  Let's face it, it takes quite an act of faith to put a check into the mail and send it to someone on the other side of the world who may or may not fulfil his side of the deal.  This risk is removed by registering via a reputable agent.

The agents do not charge the user any commission; their fees come off the author's payment.  You can contact most agents via phone, fax, email or WWW and pay with your credit card.  A recent (1996) development in this system is that some agents will, upon payment,  immediately give you a software key to convert your evaluation copy to a registered copy, or will allow you to download the registered version.  With this system you can have the full version on your computer within minutes of deciding to purchase.  For details on using this system to register this program see the Appendix E - "Ordering a Copy of Packed Column Calculator".

## APPENDIX D - The Packed Column Calculator Licensing Policy

The evaluation version of PackCol will function for at least 30 days for you to evaluate its capabilities and its suitability for your needs.  During this time the program will remind you from time to time that it is an evaluation version, and the printouts will also have remarks that they were produced by an evaluation version.  When you pay for and register your copy of PackCol these time limits and reminders will be eliminated.  If you decide that you want to purchase the registered version of PackCol, details on ordering are included in Appendix E.

The license for PackCol is available in two forms, ie a single user version and a site licence.  You can install the site licence version onto as many computers as you like, provided they are all on the same site and belong to the same organization.  This licence of course also covers portable computers that are based at that site, but are used off the site from time to time.  A single user licence costs US$80-00 and a site licence costs US$250-00.

The license for PackCol also includes free upgrades to any new version of the program that is released within 12 months of the date of purchase.  When you register PackCol you will be given a software key (ie a password) that you enter together with your company name to convert the shareware version to the registered version.  This key will work with any version you download that was released within a year of your original purchase date.  Any upgrades after the first year will be charged as a new purchase, but will then include a year's free upgrades from that date again.  We have done this to avoid the complication of multiple pricing levels, and hopefully the free upgrades together with the full priced ones will average out at good value.

You are also entitled to free software support by email.  See section 7 "Software Support" in the main part of the manual for details.

Over the years there has been much debate over what constitutes a fair licensing policy for shareware, particularly over the use of positive and negative "incentives".  We have put an enormous amount of thought into the "incentives" that we have built into this program.  We believe that the user should be able to see and test every feature of the program, including the manual, before being asked to part with any money.  There should be absolutely no doubt in the user's mind as to whether the program is suitable or not.  With this in mind, we have made this program fully functional, but we have programmed a time limit into it and have put watermarks on some of the printouts.  There is also a restricted range of packing factors that can be used. We hope that you find this an acceptable compromise and would appreciate your comments on what you believe is a fair system.

## APPENDIX E - Ordering a Copy of Packed Column Calculator

If you have a credit card, you can get a registered copy of PackCol within a few hours, or even minutes on a good day.  This rapid ordering procedure, as well as various other options, is described below.  A single user licence costs US$80-00 and a site licence costs US$250-00.

Thanks to the power of modern communications you can order your registered copy of PackCol from any of our agents, irrespective of where you are, but you may prefer to use the agent closest to you.  You can also order directly from us, but we encourage you to rather order via an agent.  It may sound strange that we would want to install a middle man (who must be paid!), but it is for your protection and peace of mind that we recommend this.  The agents we have selected will not accept an order for a program that is not properly supported by its author.  We believe this protection for the user is important.  If you want to order directly from us, but are concerned about whether we are still around (this file could sit on the internet for years!)  please feel free to email us first.

The agents we use are Public (software) Library, usually known as PsL, in the USA,  Share-It in Germany, SW-Reg in Britain, Shareware Australia in Australia and Chempute Software in South Africa.  Each of these agents uses a different ordering procedure and these are described individually below.

An order form for ordering directly from Katmar Software is also attached as Appendix F.

### Ordering from PsL

PsL are very flexible in that you can order via the World Wide Web, by toll-free phone, by normal phone, by fax, by email, or by physical mail.  All orders are charged against your credit card.  If you order by phone or via the WWW, PsL will immediately give you a temporary software key (password) that will convert your shareware evaluation version to a registered version.  You should use the Registration | Temporary function on the main menu to enter your company name and software key to disable the shareware time limit and reminders.   PsL will advise us of your registration and we will send you your permanent key.  This may take a few days, but usually occurs within 48 hours.  Please give PsL your email or fax address so that we can get this key to you as quickly as possible.

PsL produces a monthly CD-ROM containing thousands of shareware programs for home and business.  The CD-ROM normally sells for US$29.95 plus shipping.  Because this CD-ROM is so popular and widely distributed, many of the top shareware authors use it as a primary route for distributing their latest programs.  When ordering Packed Column Calculator from PsL you can get the latest version for only US$6.00 plus shipping, still with the full money back satisfaction guarantee.   Shipping is US$4.00 in continental USA and US$8.00 outside continental USA.  Please discuss this with the order clerk when you phone in your order for PackCol, or tick the appropriate box on the Fax or WWW order form.

On the page after these instructions you will find a PsL order form that you can use if you are ordering by mail or fax, or which you can use as a checklist to establish the information they will ask you for if you are going to order by phone, email or WWW.

Please note that PsL only take orders on our behalf.  The PsL addresses given below are for orders only. They cannot answer technical queries or give any other support.  All support queries and any other correspondence must be directed to us at Katmar Software.  Please see Section 7 in the main manual for full details on software support.

The PsL product code for Packed Column Calculator is #15421.

You can order with MC, Visa, AmEx, or Discover from Public (software) Library using the following methods :-

Online via the WWW:  Go to our Web site at http://users.lia.net/katmar/ and click on the link to Packed Column Calculator and then on the link to Online Ordering.  Then click on the link to PsL and fill in the form that appears.  As soon as your credit card details have been verified you will receive your temporary password from PsL. If you have problems getting through to our web site you can go directly to PsL at www.pslweb.com and search by program name or by using the product code of 15421.

By Fax: Please use the Fax order form over the page and send it to 713-524-6398.  Please type or block print very plainly.  The order form is also included in the .ZIP file as the file PSLFAX.TXT  This is a plain ASCII file that you can load into your word processor and fill in electronically if you do not have a typewriter. (Does anyone still have typewriters?!?)

By Email: Use the Fax order form over the page to see what information you need to send.  Send your email message to 15421@pslweb.com

By Snail Mail: Please use the Fax order form over the page and send it to PsL at PO Box 35705,   Houston, TX 77235-5705, United States of America.

By Phone: Please use the Fax order form over the page as a check list for the information you will have to provide when you phone your order through.  PsL’s operators are available from 8:00am to 6:00pm (ie 18h00) CST Monday to Friday at 800-242-4775 Ext. 15421 from within the USA or 713-524-6394 Ext. 15421 from other countries.

THE ABOVE NUMBERS ARE FOR ORDERS ONLY

Any questions on the status of the shipment of the order, refunds, registration options, product details, technical support, non-credit card orders etc must be directed to Katmar Software, PO Box 654, Westville 3630, South Africa.  (email address katmar@dbn.lia.net).  PsL will notify us by E-mail the day of your order and we will email or fax the permanent software key directly to you.

### Order Form for Credit Card Purchases by Fax or Mail from PsL

Use this form only if you are ordering by FAX or mail from Public (software) Library.  DO NOT SEND CREDIT CARD ORDERS DIRECTLY TO KATMAR SOFTWARE.  Send the completed form to :-

Public (software) Library		**NOTE**  This address is for

PO Box 35705				ORDERS ONLY. Any questions on

Houston				the status of the shipment of

TX 77235-5705				the order, refunds,

United States of America		registration options, product

details, technical support etc

FAX  713-524-6398			must be directed to Katmar

Software's South African address.

Registration for    :	Packed Column Calculator by Katmar Software (product code #15421)

__

Licence type    :		Single User Licence US$ 80-00		|__|	(tick

Site Licence US$ 250-00		|__|	 as

PsL CD-ROM :		In continental USA US$ 10-00		|__|	 appropriate)

Outside continental USA US$14-00	|__|

Registration Details

Your Name 		: ________________________________________________

Your Company Name 	: ________________________________________________

Your Address 		: ________________________________________________

________________________________________________

________________________________________________

________________________________________________

Your Email Address	: ________________________________________________

Your Fax Number	: ________________________________________________

Credit Card Details

Name as on Card	: ________________________________________________

Billing Address		: ________________________________________________

________________________________________________

________________________________________________

Credit Card type 	:   MC / Visa / AmEx / Discover    (circle one)

Credit Card Number	: ________________________________________________

Expiration Date		: ___________________________ ( mm/yy )

Total Amount in US$ 	: ___________________________ (see cost above)

Signature		: ___________________________   Date : ______________

### Ordering from Share-It

Share-It are based in Germany.  However, you can order from them from anywhere in Europe and indeed from anywhere in the world.

You can order from Share-It via the World Wide Web, by phone, by fax, by email, or by physical mail.  Orders can be charged against your credit card or paid for by money order, cash or Eurocheques (only in Europe).  If you order by phone or via the WWW, Share-It will email you a temporary software key (password) that will convert your shareware evaluation version to a registered version.  This email will reach you with 24 hours of registering.  You should use the Registration | Temporary function on the main menu to enter your company name and software key to disable the shareware time limit and reminders.   Share-It will advise us of your registration and we will send you your permanent key.  This may take a few days, but usually occurs within 48 hours.  Please give Share-It your email or fax address so that we can get this key to you as quickly as possible.

On the page after these instructions you will find a Share-It order form that you can use if you are ordering by mail or fax, or which you can use as a checklist to establish the information they will ask you for if you are going to order by phone, email or WWW.

Please note that Share-It only take orders on our behalf.  The Share-It addresses given below are for orders only. They cannot answer technical queries or give any other support.  All support queries and any other correspondence must be directed to us at Katmar Software.  Please see Section 7 in the main manual for full details on software support.

The Share-It product code for Packed Column Calculator is 102172.

You can order with Mastercard/Eurocard, Visa, American Express, or Diners Card from  Share-It  using any of the following methods, or with cash, money order or Eurocheque by mail :-

Online via the WWW:  Go to our Web site at http://users.lia.net/katmar/ and click on the link to Packed Column Calculator and then on the link to Online Ordering.  Then click on the link to Share-It and fill in the form that appears.  As soon as your credit card details have been verified you will receive your temporary password from them.  If you have problems getting through to our web site you can go directly to Share-It  at www.shareit.com and search by program name or by using the product code of 102172.

By Fax: Please use the Fax order form over the page and send it to +49-221-240 7278 (Germany).  Please type or block print very plainly.  The order form is also included in the .ZIP file as the file SHAREIT.TXT  This is a plain ASCII file that you can load into your word processor and fill in electronically if you do not have a typewriter. (Does anyone still have typewriters?!?)

By Email: Use the Fax order form over the page to see what information you need to send.  Please ensure that you include the Product code 102172  Send your email message to register@shareit.com

By Snail Mail: Please use the Fax order form over the page and send it to :-

Share-It

Habsburgerring 3

D-50674  Koeln

Germany

By Phone: Please use the Fax order form over the page as a check list for the information you will have to provide when you phone your order through.  Share-it’s operators are available on +49-221-240 7279 (Germany).

### Order Form for Credit Card Purchases by Fax or Mail from Share-It

Use this form only if you are ordering by FAX or mail from Share-It.  DO NOT SEND CREDIT CARD ORDERS DIRECTLY TO KATMAR SOFTWARE.  Send the completed form to :-

**NOTE**  This address is for

Share-It				ORDERS ONLY. Any questions on

Habsburgerring 3			the status of the shipment of

D-50674  Koeln				the order, refunds,

Germany				registration options, product

details, technical support etc

FAX  +49-221 240 7278			must be directed to Katmar

Software's South African address.

Registration for    :	Packed Column Calculator by Katmar Software

(product code 102172)

Licence type    :		Single User Licence US$ 80-00		|__|	(tick one)

Site Licence US$ 250-00		|__|

Registration Details

Your Name 		: ________________________________________________

Your Company Name 	: ________________________________________________

Your Address 		: ________________________________________________

________________________________________________

________________________________________________

________________________________________________

Your Email Address	: ________________________________________________

Your Fax Number	: ________________________________________________

Credit Card Details

Name as on Card	: ________________________________________________

Billing Address		: ________________________________________________

________________________________________________

________________________________________________

Credit Card type 	: Mastercard/Eurocard / Visa / AmEx / Diners Club  (circle one)

Credit Card Number	: ________________________________________________

Expiration Date		: ___________________________ ( mm/yy )

Total Amount in US$ 	: ___________________________ (see cost above)

Signature		: ___________________________   Date : ______________

### Ordering from Sw-Reg

Sw-Reg are based in Britain.  They take credit card orders using a secure order form on the WWW.  All orders to SW-Reg are charged in British Pounds Sterling.

You can order from them using your credit card (Visa or Mastercard only) by visiting our homepage at http://users.lia.net/katmar/ and clicking on the link to Packed Column Calculator.  Then click on Online Ordering.  Then click on the link to Sw-Reg and fill in the form that appears.  Note that you must use the Product ID 201708 for a single user licence (cost GBP 50-00) or Product ID 201709 for a site licence (cost GBP 150-00).

As soon as your credit card details have been verified you will receive your temporary password from Sw-Reg.

If you are operating your computer from behind a firewall you will not be able to use Sw-Reg’s secure ordering system.  In this case we would recommend using PsL’s non-secure option (their site has both secure and non-secure servers).  If you are not sure if you have a firewall in place check with your system administrator.  If you don’t have a system administrator you can be almost certain you also do not have a firewall.  In any case, Sw-Reg’s server will advise you if you have a problem when you access it.

You should use the Registration | Temporary function on the main menu to enter your company name and software key to disable the shareware time limit and reminders.   Sw-Reg will advise us of your registration and we will send you your permanent key.  This may take a few days, but usually occurs within 48 hours.  Please ensure that you give Sw-Reg your correct email address so that we can get this key to you as quickly as possible.

If you have problems getting through to our site you can go directly to their site at www.sw-reg.com and registering using the Product ID 201708 for a single user licence (cost GBP 50-00) or Product ID 201709 for a site licence (cost GBP 150-00).

### Ordering from Shareware-Australia

Shareware-Australia are based in Australia (believe it or not).  They have facilities for you to order by WWW, phone, fax, email and snail mail.  You can pay with your Australian credit card (eg Bankcard) or international credit card (eg AmEx, Visa and M/C), by cheque drawn on an Australian bank, with Australian money orders, bank transfer or with cash.

Shareware-Australia quote all their prices in Australian dollars.  The prices in Australian dollars are $Aus125-00 for a single user licence, and $Aus400-00 for a site licence.

Once you have received your password from Shareware-Australia, you should use the Registration | Temporary function on the main menu to enter your company name and software key to disable the shareware time limit and reminders.   Shareware-Australia will advise us of your registration and we will send you your permanent key.  This may take a few days, but usually occurs within 48 hours.  Please ensure that you give Shareware-Australia your correct email address so that we can get this key to you as quickly as possible.

Online via the WWW:  Go to our Web site at http://users.lia.net/katmar/ and click on the link to Packed Column Calculator and then on the link to Online Ordering.  Then click on the link to Shareware-Australia and fill in the form that appears.  As soon as your credit card details have been verified you will receive your temporary password from them.  If you have problems getting through to our web site you can go directly to Shareware-Australia at www.shareware-australia.com and select the “PRODUCTS” link.

By Fax: Please use the Fax order form over the page and send it to (02) 9565-2362 (Australia).  Please type or block print very plainly.  The order form is also included in the .ZIP file as the file SWAFAX.TXT  This is a plain ASCII file that you can load into your word processor and fill in electronically if you do not have a typewriter. (Does anyone still have typewriters?!?)

By Email: Use the Fax order form over the page to see what information you need to send. Send your email message to sales@shareware-australia.com.  You can also load the file SWAFAX.TXT into your email editor and fill it in before sending.

By Snail Mail: Please use the Fax order form over the page (or the file SWAFAX.TXT) and send it to :-

Shareware-Australia

POBox 1075

Maroubra NSW 2035

Australia

By Phone: Please use the Fax order form over the page as a check list for the information you will have to provide when you phone your order through.  Shareware-Australia’s operators are available on (02) 9565-2339  (Australia).

### Order Form for Credit Card Purchases by Fax or Mail from Shareware-Australia

Use this form only if you are ordering by FAX or mail from Shareware-Australia.  DO NOT SEND CREDIT CARD ORDERS DIRECTLY TO KATMAR SOFTWARE.  Send the completed form to :-

**NOTE**  This address is for

Shareware-Australia			ORDERS ONLY. Any questions on

PO Box 1075				the status of the shipment of

Maroubra NSW 2035			the order, refunds,

Australia				registration options, product

details, technical support etc

FAX  (02) 9565-2362			must be directed to Katmar

Software's South African address.

Registration for    :	Packed Column Calculator by Katmar Software

__

Licence type    :		Single User Licence $Aus 125-00	|__|	(tick one)

Site Licence $Aus 400-00		|__|

Note : Prices above are in Australian Dollars

Registration Details

Your Name 		: ________________________________________________

Your Company Name 	: ________________________________________________

Your Address 		: ________________________________________________

________________________________________________

________________________________________________

________________________________________________

Your Email Address	: ________________________________________________

Your Fax Number	: ________________________________________________

Credit Card Details

Name as on Card	: ________________________________________________

Billing Address		: ________________________________________________

________________________________________________

________________________________________________

Credit Card type 	: Bankcard / Mastercard / AmEx / Visa  (circle one)

Credit Card Number	: ________________________________________________

Expiration Date		: ___________________________ ( mm/yy )

Total Amount in $Aus 	: ___________________________ (see cost above)

Signature		: ___________________________   Date : ______________

### Ordering from Chempute Software

Chempute Software specialise in engineering software.  They are not generally in the business of shareware, but thanks to a long standing association with them, they take orders on our behalf in South Africa.

Their address details are :-

Contact : 	Dave Wickham or Andrew Craig

Tel : 		(031) 764-6840

Fax : 		(031) 764-5934

Email :		sales@chempute.com

Web :		http://www.chempute.com

Address :	PO Box 870,  Westville 3630,  South Africa

Chempute carry a very comprehensive range of technical software and their entire catalogue can be browsed on-line at their Web site (address above).  You can download demo versions of most of their range from that address.

If you phone or fax your order number through to them they will be able to give you a temporary password immediately to convert your evaluation copy of PackCol to a registered version.  They will then advise us of your registration and we will send you your permanent password.

## APPENDIX F - Order Form for Ordering directly from Katmar Software

Use this form if you want to order directly from Katmar Software.  Please note that you cannot send credit card orders directly to us, you must use one of our agents.  You must pay by check (cheque) or money order if you order directly from us.

Send your order to :   Katmar Software,  PO Box 654,   Westville,   3630,   South Africa

Date : ________________ 				Order Number : _____________

Ordered By :-

Name : 		_______________________________________________

Job Title :	_______________________________________________

Department :	_______________________________________________

Company : 	_______________________________________________

Email address:	_______________________________________________

Supply :-   ( please delete as appropriate )  (Prices inclusive of P&P)

1 copy of Packed Column Calculator (single user licence) at US$80-00

1 copy of Packed Column Calculator (multi-user site licence) at US$250-00

SUM Enclosed : US$  __________

Enclose a cheque or money order for the above amount. No purchase orders or COD's will be accepted.  If ordered from within the South African monetary area payment may be made in Rands at the current exchange rate.

Delivery Method

Our standard delivery method is to send the password by email.  If you do not have email you must supply either a physical mail address or a fax address.  Please tick one of the methods below:-

By email :	___

By fax :		___

By snail mail :	___

Additional Information

(This information is optional unless you have requested a delivery method that requires it)

Address : 	_______________________________________________

_______________________________________________

Town : 		_______________________________________________

State : 		_______________________________________________

Postal / ZIP Code :	_________________________________________

Country : 	_______________________________________________

Fax :		_______________________________________________

## APPENDIX G - Contacting Katmar Software

If you have any problems that you cannot solve by reading the on-line help or the manual, you are welcome to contact us. When describing the problem please be as detailed as possible.   If you have operational problems include details of your hardware, operating system and so on.

Contact us by email at:

katmar@dbn.lia.net

Or write to us at :

Katmar Software

PO Box 654

Westville 3630

South Africa.

Or visit our WWW homepage at:

http://users.lia.net/katmar/

We have put many hours into the checking of this program and were not aware of any bugs at the time of release.  However, we are also aware that the most subtle bugs are unearthed after release.  We are therefore most grateful to those who report bugs to us.  Please give as much detail as possible on the bug so that we can re-create the circumstances on our equipment.

Our experience gained over many years and with the development of several packages has shown us that no matter how much effort we put into designing and planning we can never hope to match the combined creativity of our users.  We would like to hear of any features you would like to see changed or added to the Packed Column Calculator program.

## APPENDIX H - Bibliography

The articles listed below have all been useful to us in developing the Packed Column Calculator program.  Some are referenced in the text above where they had a direct bearing on the methods used.

Coker A.K., Chem. Eng. Progress, 87, No. 11, (Nov. 1991), p.93-99,

“Understand The Basics of Packed-Column Design”

Eckert J.S., Chem. Eng. Progress, 66, No. 3, (Mar. 1970), p.39-44,

“Selecting the Proper Distillation Column Packing”

Eckert J.S.,  Chem. Eng., April 14, 1975. p.70-76.

“How Tower Packings Behave”

Kister H.Z. and Gill D.R., Chem. Eng. Progress, 87, No. 2, (Feb. 1991), p.32-42,

“Predict Flood Point and Pressure Drop for Modern Random Packings”

Kunesh J.G.,  Chem. Eng., Dec 7, 1987. p.101-105.

“Practical Tips on Tower Packing”

Leva M., Chem. Eng. Progress, 88, No. 1, (Jan. 1992), p.65-72,

“Reconsider Packed-Tower Pressure-Drop Correlations”

Prahl W.H.,  Chem. Eng., August 11, 1969. p.89-96.

“Pressure Drop in Packed Columns”

Prahl W.H.,  Chem. Eng., November 2, 1970. p.109-112.

“Liquid Density Distorts Packed Column Correlation”

Robbins L.A., Chem. Eng. Progress, 87, No. 5, (May. 1991), p.87-91,

“Improve Pressure-Drop Prediction With a New Correlation”

## APPENDIX I - Other Programs available from Katmar Software

All the programs published by Katmar Software are available in evaluation form directly from our web page.  See Appendix G above for details on where to find us.

Water and Steam Properties

This program, known as WASP, is written for Windows 95.  It replaces printed Steam Tables, but it goes further by providing a much wider set of properties, it eliminates the need for interpolating between data points, and it works in all the generally accepted units. It will find the conditions that give known values of enthalpy, entropy or density.  There is context sensitive, on-line Help available at all times.

It gives 15 point properties, and 17 temperature and pressure dependent properties for ice, water and steam.  Most of the correlations cover the range from -100  C to 800  C and up to 1000 bar.  Calculated data points match published data very closely.

The registration cost of WASP is US$80-00 for a single user licence, and US$250-00 for a multi-user site licence.

Project Risk Analysis

Project Risk Analysis is written for Windows 3.1, but will also run under Windows 95/98.

The purpose of this program is to enable capital risks on projects to be evaluated, and for the financial contingencies required to cover those risks to be calculated in a rational and defendable manner.  Too often the project contingency is calculated as a "gut feel" amount, without much consideration for the real risks involved.  The procedure followed in PRA encourages disciplined estimating, and will calculate the required contingency according to the proven statistical method known as Monte Carlo Simulation.

Although PRA uses sophisticated statistical techniques, it has been written for project professionals who are more at home with estimating and costing than with statistics.  All the statistical models and distributions required have been specifically set up for contingency analysis and no knowledge of statistics is required.  A tutorial on using statistical distributions in capital cost estimating is included in the manual.

The results calculated by PRA can be displayed in easy-to-read graphical formats that quickly and effectively give the estimator an overall appreciation of the risks. These graphs are also ideal as support documentation for presentations to senior management in the justification of requests for contingency funds.   The results are also available in detailed tabular format.

In the process of calculating the required contingency, PRA will also pick out and highlight the areas that are sensitive to risk.  By applying the Pareto 80/20 Principle you can then concentrate on the areas where the potential impacts are greatest.

Project cost estimates that have been developed in spreadsheets can be imported as ASCII text, and then analysed by PRA.

The registration cost of Project Risk Analyais is US$80-00 for a single user licence, and US$250-00 for a multi-user site licence.