Terry Bohning

11851 NW 37 Place

Sunrise, FL 33323

Compuserve [71036,1066]

original: 11/4/88

Rev. 1: 11/8/88

Corrected an error in the DEBUG source

was:

or     byte ptr [0411],16      ;game port indicator is bit 4 of high byte

changed to

or     byte ptr [0411],10      ;game port indicator is bit 4 of high byte

since DEBUG uses hex, not decimal.

Note that JOYFORCE.COM was OK, just the doc was wrong.

Force System Recognition of Joystick

Recently I fired up my AT clone's game port for the first time since

I purchased it and discovered that Striker, one of my favorite

public domain games, would not work with a joystick.

After  quite a few hours of work, I found that the DOS

Equipment Check Call (INT 11H) was saying that there was

no game port!  I examined my old XT BIOS source code and

found that the equipment flag is a word stored at 00:0410.

Bit 12 of the word signifies whether there is a game port

or not.

The BIOS sets this at power up based upon whether there is a

stick attached to the port or not. So if you boot up without

your stick attached, you don't have a game port.

Well, in my case, it didn't seem to matter whether the stick

was there at power-up or not, it just didn't work.

I keyed the program shown below in under DEBUG (minus

the comments of course) , saved it as JOYFORCE.COM

and now Striker works fine.

C>DEBUG

-a100

xor    ax,ax                   ;equipment flag is in segment 0

mov    ds,ax                   ;set Data Segreg to 0

;high byte of equipment flag is at 0x411

or     byte ptr [0411],10      ;game port indicator is bit 4 of high byte

int    20                      ;terminate program

-rcx

CX 0000

:0b

-njoyforce.com

-w

Writing 000B bytes

-q

C>

/********************* EOF *************************/

#################################################