# CHAPTER 10	INSTALLATION§§

## 1.10	HARDWARE INSTALLATION

### 1.1.1	Jumper Block Settings

Before installing the Aria audio board into a PC, be sure to check the following jumper block connections (refer to Figure 1.1 for location of jumper blocks).

µ §

Figure 1.1§  AUDIO BOARD JUMPER BLOCKS

JP1 - Aria DMA Channel

These jumpers allow you to select the PC DMA channel for use with Aria digital audio transfers.  Note that both jumpers must be moved to switch the channel.  Default Aria DMA channel is 5.

µ §

JP2 - Sound Blaster DMA Enable

These jumpers enable PC DMA channel 1 for use with Sound Blaster digital audio transfers.  Note that both jumpers must be on the board for DMA read and writes.

µ §

XE "DMA Channel Select" \r "DMASelect"§

JP3 - XE "IRQ"§ Aria/SCSI IRQ

These jumpers set the interrupt request (IRQ) level for both Aria and the optional SCSI data interface.  Default Aria IRQ is 10, default SCSI IRQ is 11.

µ §

JP4 - Sound Blaster/MIDI IRQ

These jumpers set the interrupt request (IRQ) level for both Sound Blaster emulation and the MPU-401 compatible MIDI interface.  Default Sound Blaster IRQ is 7, default MIDI IRQ is 2 (same as 9 on PC AT machines).

µ §

XE "IRQ Select" \r "IRQSelect"§XE "Jumper Blocks" \r "JumperBlocks"§

JP7 - Joystick SpeedXE "Joystick Rate"§

This jumper sets the joystick response speed.  The default NORMAL speed position is OPEN (no jumper) and the FAST speed selection is CLOSED (jumper inserted and shorting the pins).

SW1 - Aria Port Address Switch SettingsXE "Base Address"§

The following drawing shows the various settings for DIP switch 1 on the Aria board.  Default configuration is shown for all selections.

µ §

SW2 - Aria Microphone Input Switch SettingsXE "Audio Gain Select"§XE "Audio Impedance Select"§

The following drawing shows the various settings for DIP switch 2 on the Aria board.  Default configuration is shown for all selections.

µ §

### 2.1.2	Audio Board Installation

The audio board must be carefully inserted into a full-length, 16-bit slot inside your IBM PC AT or compatible (Figure 2.2).  If you have an internal CD-ROM drive with an audio output cable, it can be plugged into the CD-ROM audio jack on the top of the audio board (see Figure 2.3 for connector location).

µ §

Figure 2.4§  AUDIO BOARD INSTALLATION

The back panel of the audio board is configured as shown below in Figure 2.5.  HeadphonesXE "Headphones"§ or external amplified speakersXE "Speakers"§ may be plugged into the 1/8" stereo Line Output jack.  The Microphone InputXE "Microphone Input"§ jack (1/8" stereo) accepts a microphone level input for recording.  The External InputXE "External Input"§ jack only accepts line level input.  Also available is a JoystickXE "Joystick"§/MIDIXE "MIDI"§ adapter for attaching a joystick or external MIDI synthesizer.

µ §

Figure 2.6§  BACK PANEL CONFIGURATION

XE "Installation:Hardware" \r "HardwareInst"§

## 2.20	ARIA ADDRESSING VARIATIONS

The following table summarizes the addressing (base port addressXE "Base Address"§, interrupt addressXE "IRQ"§ and DMAXE "DMA"§ channel) selections available on the Aria boards.

|  | Aria Mode | Sound Blaster Emulation | Adlib Emulation |
| --- | --- | --- | --- |
| Base Port Address | 280h | 220h | 388h |
|  | 290h | 240h |  |
|  | 2A0h |  |  |
|  | 2B0h |  |  |
| Interrupt | IRQ 10 | IRQ 2/9 | None |
|  | IRQ 11 | IRQ 5 |  |
|  | IRQ 12 | IRQ 7 |  |
| DMA | 5 | 1 | None |
|  | 6 |  |  |
| External Midi Base Port | 320h | Same as | Same as |
|  | 330h | Aria | Aria |
| External Midi Interrupt | IRQ 2/9 | Same as | Same as |
|  | IRQ 5 | Aria | Aria |
|  | IRQ 7 |  |  |
| SCSI Memory Map Addr | C8000h | Same as | Same as |
|  | CA000h | Aria | Aria |
|  | CE000h |  |  |
|  | DE000h |  |  |
| SCSI Interrupt | IRQ 10 | Same as | Same as |
|  | IRQ 11 | Aria | Aria |
|  | IRQ 12 |  |  |
| Joystick Base Port | 200h | Same as | Same as |
|  |  | Aria | Aria |

## 2.30	SOFTWARE INSTALLATION

### 2.3.1	Aria Synthesizer Development Kit Software

To install the development tools and device drivers on your hard disk, insert Aria Synthesizer Software Development Kit DISK ONE into a floppy drive and switch to that drive by typing "A:" or "B:".  Type "INSTALL" at the DOS prompt and then follow the on-screen instructions.  An "\ARIASSDK" directory will be created if necessary and all files will be copied to your system.  Other subdirectories will also be created.

A "README.TXT" documentation file is included that describes any additional information not found in this manual.

### 2.3.1	Voyetra Multimedia Software

To install the VoyetraXE "Voyetra Multimedia Software"§ development tools on your hard disk, refer to the bundled Voyetra documentation.  An installation program is provided with the Voyetra software.  Subdirectories from directory "\VOYETRA" will be created and all necessary files will be copied to your system.

### 2.3.2	Aria Multimedia Windows Driver

The Aria audio driver for Microsoft Windows is included on a separate disk labeled "Aria Multimedia Windows Driver."  This driver is intended for use with Microsoft Multimedia Extensions (MME) 1.0 or Windows 3.1.  

The following steps outline the installation procedure for Windows 3.1 (note that MME installation may differ slightly):

1.	Make sure that the Aria board is properly installed in your system.  Note the configuration of the jumper blocks (port assignments and IRQs).

2.	Activate Multimedia Windows or Windows 3.1 on your system.  From the Main group, activate the Control Panel application.

3.	Activate the Drivers applet and press the Add... button.

4.	From the list of drivers that will be displayed, select Unlisted or Updated Driver and press OK.

5.	Windows will prompt you for a path name.  Insert the ARIA WINDOWS DRIVER diskette into your disk drive and make sure that the path name displayed is correct (i.e. A:\ or B:\).  Press OK.

6.	A window will appear with "Aria Multimedia Audio System" listed as a selection.  Press OK.  The appropriate files will be copied from the diskette to your \WINDOWS\SYSTEM\ directory.

7.	The Aria Configuration window will automatically activate.  Make sure the configuration settings displayed are correct for your system and press the Save button.  You will be informed of incorrect address selections.

8.	Press the Restart Now button for the new driver to be activated.

The MIDI MapperXE "MIDI Mapper"§ allows the musical synthesizer functions of the Aria system to work properly when playing MIDI files.  Either copy the supplied "MIDIMAP.CFG" file over the existing one in your \WINDOWS\SYSTEM directory or perform the following steps to manually initialize the Windows MIDI Mapper.

9.	When Windows returns, reactivate the Control Panel from the Main group.  Activate the MIDI Mapper applet and press the New... button.

10.	You will be prompted to enter a name and a description.  
For the name, type:  Aria.  For the description, type:  Aria Multimedia Audio System.

11.	A setup window will be displayed listing all 16 MIDI channels and channel assignments.  For MIDI channels 1 to 10 under the heading Port Name, pull down the selection list beside each entry (denoted with a down arrow ö) and select Aria Synthesizer.  When this is completed, press the OK button.

12.	Press the Close button on the MIDI Mapper.

Your system will now be ready to play MIDI files using Microsoft Media Player or other applications that make use of synthesized music.XE "Microsoft Windows" \r "MSWindows"§XE "Windows" \r "MSWindows"§

XE "Installation:Software" \r "SoftwareInst"§