*********************************************************************

*              NFORMAT - THE SUPERIOR DISK FORMATTER                *

*********************************************************************

NFORMAT is a disk formatter designed to dynamically format

floppy disks for MS/PC-DOS. The internal parameter editor or command

line options let you specify parameters that allow you to get up to

23% more data space from your floppy disks.

NFORMAT offers:

* A self-contained, user-friendly menu

* Mouse support!

* Automatic setup for standard and maximum formats

* An editor for the disk parameters

* Superior media defect detection

* Multiple verification techniques

* Easy to use command line parameters

* Support for older versions of DOS

Recommended for the simplest operation:

+ A standard 360k, 720k, 1.20m, or 1.44m drive

+ An AT class machine or better

+ MS/PC-DOS version 3.2 or higher

+ A mouse!

How to use NFORMAT:

Simply run the program NFORMAT. If you are using an AT class

machine with DOS 3.2 or higher, NFORMAT will automatically retrieve

the parameters for your drives. The parameters are then saved to disk

in the file NFORMAT.CFG.

The menu will list the drive letters followed by the data size

of the disk format. To select the format you desire, highlight the

format using the arrow keys and press ENTER. If you are using a

mouse, you can select by using the mouse cursor. Move the cursor to

the menu item and press the first mouse button.

NOTE: Throughout NFORMAT, the first mouse button has the same

function as the ENTER key. Also, the second mouse button

has the same function as the ESCAPE key.

The formatting screen will display all available information

about the format you requested. NFORMAT will prompt you to insert the

disk you wish to format. At this point, you may press ESCAPE to exit

back to the menu, or press any other key to begin formatting.

WARNING!: Formatting the disk permanently erases any

data on that disk!

While formatting, any bad sector information will appear in the

BAD SECTORS window in the upper right of the screen.  The current

process will appear in the box in the center of the screen.  Any

system errors will appear in the upper left of the screen, and prompt

for user action.

When NFORMAT finishes formatting the disk, NFORMAT will prompt

you to enter a volume label. Since the label is written directly to

the disk you may enter both upper and lower case characters.

If you should need to exit NFORMAT before formatting is

complete, you may press escape. The error box will prompt you to

confirm that you wish to exit.  Once formatting is complete, NFORMAT

will return you to the main menu. From the main menu you may, if you

wish, format another disk.

WARNING: Exiting the formatter before formatting is complete

will cause the disk to be unusable to DOS. To make the

disk useable it will need to be reformatted.

*********************************************************************

About the Menu:

When you are in the SELECT FORMAT menu, the entry that you wish

to use is highlighted. The highlight bar moves by using the up

arrow key and the down arrow key. Selecting a format may also be done

using the mouse.

Once you have highlighted the entry, you may press:

ESCAPE- Immediately leaves NFORMAT.

ENTER - This accepts the format and enters the formatter.

L     - Reloads the disk parameter configuration file (NFORMAT.CFG)

from disk.

V     - Turns disk verification on or off. Verification should

always be ON.

E     - Allows you to edit the parameters for your drive.

I     - Inserts a new record for the parameter table. This record is

temporarily filled by a copy of the format above or below it.

D     - Deletes an unwanted entry in the menu.

M     - Allows you to move an entry around in the menu.

S     - Saves the menu database with any changes to NFORMAT.CFG.

NFORMAT.CFG will be in the same path/directory as the copy

of NFORMAT.EXE.

R     - Reconfigures the menu so it will display the current

standard drive parameters.  This is easier than manually

erasing the NFORMAT.CFG file and then running NFORMAT again.

When you first run NFORMAT (with no parameters), the menu

program will automatically configure for your disk system. The

configuration routine scans for information about drives 0-3. It will

place the normal format for the drive followed by the maximum format

for that disk type. If the drive is high density, NFORMAT will add

the low density format and the maximum low density format for that

drive.

*******************************************************************

The parameter editor:

The parameter editor box appears to the right of the menu when

you select the edit function.  You may highlight the parameter you

wish to edit and then press return, or select with the mouse.  Some of

the values are in hexidecimal, and others in normal decimal. You may

enter values in normal decimal as well as hexidecimal. All hexidecimal

numbers must be preceded by a "$". Once you finish entering the new

value, NFORMAT will re-calculate the other parameters. You may notice

some of the other numbers changing.  The editor places a "*" by your

new value to show that it is protected. Being protected means that

NFORMAT can not recalculate or change your new parameter. If you need

to remove this protection, select the item you wish to unprotect.

Type a single space followed by ENTER, and the protection for that

item will disappear.

The parameters are as follows:

TRACKS ON DISK:   This number represents the number of positions

that the drive head may take. This number is

normally 40 or 80

SECTORS ON TRACK: This defines the number of separate data areas

that pass under the head each disk rotation.

NUMBER OF HEADS:  There are always at least 2 heads on a disk

drive. There is no need to change this value

unless the disk bios returns an incorrect value.

PHYSICAL DRIVE #: Normally, the physical drive values for A: is

zero and for B: is one. Some machines may have

drives as physical 2 and 3 (usually D: or E: ).

NFORMAT will always report physical $02 and $03

as X: and Y: and never in any way refer to normal

hard disks.

ROOT DIR ENTRIES: Allows you to select the maximum number of files

that may be in the root directory.  This can free

space, and is useful if you have a few large

files that you want to put on a disk. (The

maximum value is 244 files)

SECTORS PER CLUSTER:This lets you select 1 or 2 sectors per DOS

cluster.  Normally 360k and 720k use 2 sectors

per cluster, and 1.20m and 1.44m use 1 sector per

cluster. A FAT using 2 sectors per cluster is

half the size of a FAT using 1 sector per

cluster.

DISK FORMAT ID:   This byte, placed on the boot sector of the disk,

allows programs to quickly identify the disk

type.  Some versions of DOS use this as an

absolute ID. Normally, 1.44m is $F0, 1.20m and

720k are $F9 and 360k are $FD.

FORMAT FILL BYTE: This value affects the error detection ability.

The standard value is $F6. This is a trivial

item, and there is usually no need change it.

GAP LENGTH:       This defines the space between sectors. The less

space between sectors, the more sectors per

track. There always needs to be SOME gap.

INTERLEAVE:       This option allows you change the order of the

sectors on the disk. Normally the value is 1,

which means that the sectors are in the order of

1, 2, 3, 4, 5.... with one right after another. A

value of 2 will skip a sector: 1, 10, 2, 11, 3,

12 etc. The maximum value is one less than the

number of sectors per track.

MEDIA DISCRIPTOR: This is the most important of the parameters.

Usually, XT machines do not use this, but ATs

almost always do. This byte, sent to the BIOS

defines the density and some other items.

NFORMAT automatically calculates this byte to

work with the other parameters, so there is no

need to change this byte. Should there be a need

though, the byte is calculated as follows:

REFERENCE HEX|BINARY   FOR

****************************************************

DISK TYPE:$00|00000000| 360k disk in 360k drive

$01|00000001| 360k disk in 1.20m drive

$02|00000010| 1.20m disk in 1.20m drive

$03|00000011| 360k disk in 360k drive  CHK

$04|00000100| 360k disk in 1.20m drive CHK

$05|00000101| 1.20m in 1.20m drive     CHK

$06|00000110| reserved

$07|00000111| not 360k, 1.20m drive

****************************************************

MEDIA:    $00|00000000| Media type is not known

$10|00010000| Media type is known

****************************************************

STEPPING: $00|00000000| Single stepping (normal)

$20|00100000| Double stepping (WHY?!)

****************************************************

BAUD RATE:$00|00000000| 500k (high density)

$40|01000000| 300k (for 1.20m drives only)

$80|10000000| 250k ("double"/low density)

$C0|11000000| ???k (reserved)

****************************************************

take one value from each section and add them all

together to get the media byte you desire.

X SHIFT:          Normally, fast machines have a high step rate and

long gap length. After the last sector on a track

is read, the drive can switch or move the heads

and immediately find the first sector of the next

track. This means that the X value usually should

be 0.  If your system is slower, the head may

miss the first sector. Shifting lets slower

systems immediately find the first sector. A

shift value of 1 through 3 may speed up your disk

access.

Y SHIFT:          Similar to the X shift. If you are formatting a

disk using 20 sectors per track, set the X and Y

shift to 1. The result will be a very fast disk.

21 sectors uses an interleave of 2, so the

interleaved sector acts as extra gap and no extra

shifting us usually needed.

IMMEDIATE VERIFY: If set to yes, NFORMAT will format a track, and

then verify it. If set to no, NFORMAT will format

the entire disk, and then verify the entire disk.

Setting this option to "NO" is useful in

determining the maximum number of tracks on your

drive.

LABEL DISK:       When set to yes, NFORMAT will prompt for a volume

label after formatting the disk. When set to no,

NFORMAT will not ask for a volume label.

********************************************************************

Command line parameters:

Most of the parameters correspond with the definitions above. To get

a list of all command line parameters use the parameter /?

The normal syntax for NFORMAT command line parameters is:

NFORMAT <DRIVE:> <SWITCHES>

The first parameter is always the drive letter.

P: Specifies the physical drive unit (0 - 3)

T: Tracks on disk

H: Number of heads

S: Sectors per track

C: Sectors per DOS cluster (1 or 2)

R: Root directory entries

I: Interleave

F: Format ID for disk

G: Gap of bytes between sectors

B: Media discriptor

X: X sector shift

Y: Y sector shift

Q: Quick format, no verify (NOT RECOMMENDED!)

M: Use standard maximal density for disk

L: Format a low density disk in a high density drive

N: Do not ask for a volume label

K: Batch mode, skip "insert disk"

V: Non immediate verify

hexidecimal numbers must start with a "$"

Using command line parameters bypasses the menu.

**********************************************************************

The enhanced BOOT SECTOR:

You will no longer have to put up with the old DOS "Non DOS disk

or disk error" message if you have a hard disk drive. You will no

longer have to remove the disk from your drive and press a key.  The

NFORMAT boot sector checks for the presence of a hard disk drive.  If

one is present, it will boot the hard disk. It will seem as if no

disk was in the drive. If you do not have a hard drive you will get

an "insert system disk" message.

**********************************************************************

Placing PC/MS-DOS system files:

NFORMAT does not put system files on a floppy disk. This is

because different versions of DOS have their own way of placing

system files.  Your DOS should have a program called SYS.EXE or

SYS.COM. This program puts a DOS boot sector on the disk, transfers

the system file(s), and some versions transfer the COMMAND.COM

program.

A few computers may, because of BIOS or DOS, be capable of

booting from a dynamically formatted system disk. These computers

usually will not require the BIOSPTCH program. If you can boot

dynamic system disks then that is fine, but ALWAYS keep a standard

(360k, 720k, 1.20m, 1.44m) system disk. If you change machines, you

probably will not be able to boot you dynamic system disks on the new

machine.

**********************************************************************

Do you need the BIOSPTCH program?

If you want to use dynamic disks (1,74m, 830k, 430k) then

probably yes.  SOME machines do not (usually XT high density cards

with their own BIOS ROM). If you do not know and want to find out,

then try formatting a dynamic disk. Then, before installing BIOSPTCH,

try copying a large file (at least 32k) on to the disk and back.  If

no errors occur, you are OK.  Do not use a disk tester, because they

may bypass the DOS read/write. Other disk testers may go crazy

because they believe that all disks must be standard 360k, 720k,

1.20m, or 1.44m.

To install BIOSPTCH, copy it into your root directory and then

add the following line to your CONFIG.SYS file:

DEVICE=BIOSPTCH.SYS

or for DOS 5 use:

DEVICEHIGH=BIOSPTCH.SYS

If you wish to only use the standard disk formats (360k, 720k,

1.20m and 1.44m), you will not need BIOSPTCH.

**********************************************************************

What does NFORMAT do?

NFORMAT removes the frustration caused by trying to get old DOS

formatters to format a simple floppy disk which is slightly

different. Most disk formatters only allow formatting of several

different standard formats. Some dynamic disk formatters create slow

disks using poor error detection. The MS-DOS 5 formatter program is

so paranoid that the /U, unconditional format, switch must be added

for the program to actually low-level format a formatted disk.

WHY USE DYNAMIC CONFIGURATION?

Dynamic configuration allows you to determine the format placed

on the disk, so you may get the most usable space from your disk as

possible. This is rather technical, so feel free to skip to the next

section.

So, the label on your disk says that it has a capacity of 2

megabytes. When you format your disk you only get 1.44 megabytes.

Where do they get this 2.00 megs?

The result comes from:

SIZE:=((BAUD/8)/(RPM/60))*TRACKS*HEADS

BAUD: is the number of bits per second that the drive controller

reads/writes to the disk. This value is either 500k, 300k, or

250k, with k meaning one thousand. Dividing by eight gets the

number of bytes.

RPM:  The number of times per minute which your disk spins. The

normal value is 300 RPMs, but 1.20m drives and 8 inch drives

use 360 RPMs.

A standard "1.44 megabyte" drive would be baud=500k, rpm=300,

with 80 tracks and two heads.

((500000/8)/(300/60))*80*2

(62500/5)*80*2               {62500 BytesPS,5 RPS}

(12500)*80*2                 {12500 bytes per track}

2,000,000 bytes.             {total}

THE CATCH IS that floppy disks must store MORE than just your

data. Each track contains information about the track, and sectors in

the track. The track ends with a variable gap to compensate for

random speed fluctuations in the drive motor. Each sector contained

within the track has identification bytes, and includes a checksum.

The checksum ensures that the data in the sector is correct. Also,

between sectors there is a gap. This gap helps the hardware locate

the sector, and allows time to pass for the processing of the sector.

There are two gaps, one is constant, while the other is

variable. All of the standard PC formats spend 26% of the two

megabytes on non-user data. This is an unacceptably large amount of

wasted space. The low-density disks have small variable gaps, but

high density disks have extremely large variable gaps. In fact, the

default variable gap for a 1.44m disk is 108 bytes for each sector!

Because of this, only 18 sectors will fit on the track. By reducing

the gap from 108 to 12 bytes, 21 sectors will fit on the track. This

reduces the non-user space to 14%, a much better figure.

*********************************************************************

Determining the maximal format for your disk system:

Some disk drives are capable of accessing more disk space than

others. For example, most Toshiba 3.5" drives can access 84 tracks,

one more than the average. Most Teac drives only work with the normal

83 tracks. Here is a list of maximum tracks for a few drives:

TYPE                  TRACKS

----------------------------

IBM half height 360k      40

Tandon full height 360k   42

Copal half height 360k    42

Teac 1.20m                85

Kaypro 2000 3.5" 720k     80

Teac 1.44m                83

Toshiba 720k              84

Toshiba 1.44m             84

NOTE: Individual results may vary, despite the listed drive

brands.

If you have an unusual drive system, you may wish to experiment

with different configurations. NFORMAT should be able to handle up to

31 sectors per track, although most floppies do not have that many.

WARNING: Damage may occur to some old floppy drives if the

disk head attempts to go beyond the last track.

NFORMAT can not be held responsible for any damage

caused to your drive.

The easiest way to determine the maximum number of tracks

requires an error free disk and setting the VERIFY IMMEDIATE option

to NO. Try formatting the disk with the NFORMAT default maximum disk

parameters. NFORMAT will format the disk and then verify it. If the

number of tracks specified was too many, the drive head will remain

over the last track while attempting to format another track. The

track information that was already under the head is written over.

This will show up during verification as a completely unreadable

track. When lost tracks occur, you must decrease the number of

tracks.

Immediate verification can not be used for this process. The

immediate verify will format a track and verify it before moving the

head to the next track. When formatting too many tracks, immediate

verification can not detect if the track will be overwritten.

When trying a new maximum format, you should always make sure

that the new format is completely readable by your disk drive and

disk operating system.

Finally, keep in mind that your maximum formats may not be

readable by other machines. You should assume that your machine is

the only machine that can read your maximized disks. The main reason

for this is that maximized disks usually require a computer to have

BIOSPTCH.SYS. Also, many disk drives may not be able to access as

many extra tracks as your drive.

If you want to use your maximized disks on other computers, make

sure the format is compatible with the other disk drive(s).  You must

also have a normal disk with BIOSPTCH.SYS on it to install.

**********************************************************************

Disks-In-Drives:

Both the 1.20m and 1.44m high density drives offer support for

low density disks, but there are some compatibility problems.

360k disks in 1.20m drives:

Despite anything you may of heard, it is simply NOT possible to

format 360k disks in 1.20m drives. This is why NFORMAT does not

include the 360k format in its menu for the 1.20m drive. Any 360k

disk formatted in or written to a 1.20m drive can not reliably be

read by 360k drives. The reason for this is that 1.20m drives read

and write thinner tracks than 360k drives. If there is any data

between the tracks, a 360k drive will read and mix both the thin

track, and the edge of the data. The result will be an unreadable

track. Also, if a thick 360k track is written to by a 1.20m drive, the

thin track will be surrounded by the edges of the thick track. Again,

the 360k drive would read both signals resulting in an unreadable

track.

Since the low density disks can not be formatted high density,

the low density disks can take advantage of the thin tracks by

single stepping and doubling the number of tracks on the disk. A 360k

disk can be formatted reliably to 720k by doubling the tracks.

NFORMAT can take this a step further. By using 10 sectors per track,

and getting a few more tracks NFORMAT can format the disk to 850k.

NOTE: BIOSPTCH.SYS usually will need to be installed to read

720k and 850k disks in a 1.20m drive.

1.20m disks in 360k drives:

High density disks can not be formatted low density. The

magnetic material is usually designed to only store high frequency

data.

3.5" disks:

The high density 3.5 inch disks have a density notch in them to

automatically let the drive know what type of disk it is. This means

that it is impossible to format a 1.44m disk to 720k, and it is

impossible to format a 720k disk to 1.44m. (unless YOU put a notch in

the 720k disk, but that trick does not always work.)

NOTE: Some cheap 1.44m drives were, for some reason, not built

with density detection. Hopefully NFORMATs use of the media

discriptor byte will avoid any problems.

**********************************************************************

Unusual Environments:

If you are running NFORMAT on an old PC or XT class machine,

NFORMAT may receive incorrect information about your disk system. For

example, if NFORMAT is incorrectly told that all drives A: through D:

are 1.20m drives, you will have to manually edit the configuration.

If you are formatting using command line parameters, you will have to

specify the appropriate information for your drive.

If you have a high density disk system in an XT, the controller

card should have a BIOS ROM which may report the drive parameters to

NFORMAT like an AT would.

NFORMAT reports physical floppy drives 0 through 3 as A:,B:,X:,

and Y:, although DOS may assign drive letters anyway it feels like.

So, if you try to format drive C: or X:, NFORMAT will attempt to

format physical drive number 2.  Your hard disk is physical drive

number 128, therefore NFORMAT can not even get close to your hard

disk.  Physical drives 2 and 3 are listed by NFORMAT as X and Y to

prevent any confusion. The DOS drive letters may vary on different

machine, so NFORMAT refers to the drives by generic names X and Y.

Under versions of DOS previous to 3.2, the drive type reported

by DOS usually just returns a random number. If the machine you are

running is an AT, NFORMAT ignores whatever DOS says anyway, and gets

the info from the BIOS. HOWEVER, On a PC/XT machine, BIOS may not

report correct information.  If NFORMAT detects the bad parameters,

it will assume you have two 360k floppy drives, A: and B:. If they

are not 360k (perhaps 720k), you may edit the parameters.

Unusual disk device driver programs may interfere with the

operation of the BIOSPTCH.SYS program. These drivers will not allow

DOS to read more than the usual number of sectors per track from a

disk. This will result in errors such as "SECTOR NOT FOUND" while

attempting access to a maximized disks. If you get such errors and

your BIOSPTCH.SYS program is installed correctly, then you may need

to check for unusual disk device drivers. You may find such drivers

with exotic XT class floppy controllers. Sometimes this may be caused

by a DRIVPARM= statement in your CONFIG.SYS under some versions of

DOS. If you find such a driver, try removing it (but ALWAYS keep an

operational backup). Removing the driver may cause other formatters

and disk diagnostics programs to think your drive is a 360k drive. If

you are completely unable to access the drive after removing the

driver, then you will have to put the driver back.

If you get a NON-DOS DISK ERROR message while attempting to

access a maximized disk, chances are your DOS does not support

dynamic disk media. This error almost always occurs on versions of DOS

previous to 3.2 . If you are using an old version of DOS, then the

best solution is to upgrade. Some old versions of DOS have an

alterable disk-type look up table in the IO.SYS file.

If you have a monochrome system and the text is hard to read or

you get a blank screen, make sure that your have your video mode set

to mode number 2 (BW80). NFORMAT will detect the video mode and

display color only when color is set.

NFORMAT alters some of the drive parameters during operation. If

you exit normally, using the escape key, the parameters are reset to

what they were before program execution. If you use any other

method of exit, the drive may behave erratically and require that

your system be rebooted.

**********************************************************************

NFORMAT has been tested on a standard AT and XT system under

the following versions of DOS:

PC-DOS 2.10

MS-DOS 2.11

PC-DOS 3.10

PC-DOS 3.30

MS-DOS 3.30

MS-DOS 4.00

PC-DOS 5.00

MS-DOS 5.00

NFORMAT has also been tested under MS-WINDOWS 3.1 in 386

enhanced mode.

NOTE: Dynamic disks and Mouse support only works with DOS

version 3.2 and later.

**********************************************************************

DISCLAIMER: because of the nature of this program, this program

WILL write over any data on disks that it formats. Therefore, you are

to use this program at you OWN risk. This program can not be held

responsible for data loss or any problems caused by the loss.

<Remember, always keep backups!>

**********************************************************************

LICENSE:

This program is free for you to use and distribute. The only

condition is that no modifications are made to the NFORMAT.EXE or the

NFORMAT.DOC files.  This program is not to be sold. You may only

charge the price of the floppy disk you put the program on. Although

you are under no obligation to send money for the use of this

program, I would appriciate any contribution you may wish to make.

If you have any comments, questions, or suggestions about this

program, feel free to write me at:

Nathan Lineback

lineback@toastytech.com