README file for

ELSA WINNER Driver disk : WCFG.EXE

Copyright (c) 1993-94 ELSA AG, Aachen (Germany)

Version 1.01, 26.04.94, sd

***********************************************************************

Summary:

This file contains some hints for the use of the WINNER 1000

and WINNER 2000-2 utility WCFG.EXE

***********************************************************************

The program WCFG is used to fix a lapse of WINNER 2000-2 boards

with release steps C to E and WINNER 1000 1-2 MB (from SN:

804.024.001), if they are used in EISA bus. Depending on the

slot number the board is inserted and the BIOS version, the

board sends a different EISA identification number to the

system. Corresponding to that number the EISA configuration

program searches for a specific configuration file. If your

computer reports during the power up phase an unknown EISA card,

you can generate this configuration file with WCFG. To do this,

copy WCFG.EXE, !ELS0000.CFG and !ELS0001.CFG into an empty

directory and call WCFG. This generates all necessary configura-

tion files which have the extension CFG. Copy all these files on

your EISA configuration disk or into the corresponding directory

on your harddisk. Now you can start the EISA configuration

program, it's name mostly is CF or CFG, and install your WINNER

1000 or WINNER 2000-2.