Montest! The Ultimate in Video Card & Monitor testing!

����������������������������������������������������������������

Welcome to an initial public release of Montest. Montest was developed over a 5

year period by Merlin for Bayside PC Services for use in their repair workshops

for testing, aligning and configuring computer systems and monitors.

Montest is a very simple to use program, and supports both interactive and

stand alone testing, from a menu driven environment. Montest requires an IBM-PC

or compatable, ANY Video Card, 128k free RAM.

The type of video card installed greatly effects Montest's abilities, and the

appropriate card will need to be fitted to test any given monitor. It is

suggested that if VGA/SVGA monitor testing is required, a VESA compatible 1Mb

SVGA/XGA card be used (such as the Cirrus Logic CL-GD542x series or Trident

8900d) so that maximum testing modes are available. Montest can test all

resolutions up to XGA, and colour ranges up to 16.7M displayable colours.

These initial releases of Montest are for user evaluation and comment, as well

as to "open the market" for those who have been nagging me for a copy of

Montest for many months. The doccumentation is sketchy, and the code is

undergoing constant revision, so comments and feedback is much appreciated.

These initial test releases are easily identified by their version number of

1.xx.x�

VESA Support

�����������������

Montest makes heavy use of the VESA programming standard for SVGA, and as such,

SVGA test modes are unavailable without VESA. Montest automaticly identifies

and uses VESA if present. If your card doesn't support VESA in hardware,

consult your manuals & driver disks that came with your video card - even very

old 256k VGA cards usually come with software VESA drivers that Montest can

detect and make use of. There are also some generic shareware drivers that can

turn almost any card into a VESA compatible card. Montest can report all

available VESA modes along with the vital data on the modes themselves. Refer

to the online help in Montest for decoding the VESA info.

DAC Support

����������������

New to v1.59.x is proper Sierra (15 bit), HiColor (16 bit), and TrueColor (24

bit) colour mode testing. The tests use VESA video modes and display

appropriate pictures. The pictures are in the 3 .DAT files and are stored as

raw memory dumps of the video card's RAM. (Note: these modes do not require

pallette dumps as these modes don't use the pallette). To tell if you have a

working HiColor DAC. 1. Check the video card information screen - if it says

you've got a hicolor DAC, then go to the color output testing menu and try

options 5, 6 and 7. The 3 pictures are : Stonehenge, Porsche and Bush. The

colors should be bright and vibrant; if so, your chip works. If the colours are

mostly black, dull blues and greens, the DAC is not the right type. It is often

the case that video cards are made with HiColor compatable BIOS's, but at

assembly time standard DAC's are fitted; thus the CARD switches to hicolor mode

but you just get to look at mostly black or washed out colours.

In versions prior to 1.60.8, this feature was inoperable. the Card info screen

now definately shows if you have a "Standard" DAC or a programmable, hi color

compatable DAC, but it cannot tell if the DAC supports 15, 16 or 24 bit color;

so you still need to try to display these screens and look at the results

yourself to be sure. In addition, modes that are not available via the VESA

BIOS are no longer offered up for testing. Also, RGB Bar tests are now

available for these modes on the color menu in case you don't want/can't fit

the picture files on floppy or whatever.

Video Card Detection

��������������������������

Also new to v1.59 is video card detection. Montest now detects and reports on

most SVGA cards if found. note that SVGA test modes are still limited to VESA

compatable video cards.

Cards detected:

Ahead			V5000a/b

ATI			All

Chips & Technologies    All

Cirrus Logic            All CL510/610,CL-GD54xx,64xx,62xx

Genoa                   All

Oak                     OTI 037/057/067/077/087

Paradise                All

Trident                 All

UMC                     85c408

Video 7			All

Some information on using Montest follows ...

Note on Multitasking Environments

�������������������������������������

Due to the nature of operation of many multitasking environments, Montest may

not operate correctly withing these systems. If any multitaskers are present,

Montest displays a warning message before opening to remind you. Current

systems detected include Microsoft Windows (all variations), Dosswap (part of

MS-DOS's DOSSHELL), DR-DOS's TaskMax, and Quarterdeck's DesqView. For the most

part montest should still run OK, but doing a program swap or selecting the

video card speed tester may cause strange problems.

Note on Accuracy of System Information

������������������������������������������

The data displayed in the various information screens is accumulated by a

combination of BIOS calls, reading of registers and, in some cases, by the use

of undoccumented routines. Accuracy of the data is not always guaranteed. An

example is often found in the VESA data. Manufacturers often keep one table of

data in ROM, and use it regardless of the ammount of RAM or DAC type installed,

thus if, for example, a Cirrus Logic card is in use, it may report 2Mb RAM and

a 24 bit DAC, however it may only be fitted with 1Mb and a standard DAC. If the

information reported is incorrect (because you physicly know how much RAM is

onboard, or the DAC tests fail, for example) then this may be the source of

problems in other programs.

In the case of the display adapater speed tester, results read will generallly

be far different to what you would see with any other speed tester. this is

because the routines are in no way optimized, and represent a true test of

speed output from a high level program. The procedures tested for BIOS speed

are Write() with DirectVideo=false, and for direct access, the memw[] routine

(Turbo Pascal routines). This is to give a real world answer, not a highly

optimized result for one particular machine code optimized alogarythm.

Set Brightness Screen

��������������������������

This screen displays a greyscale of all possible values of colour, 0-63. Use

this screen to set the display cutoff pots and sub-brightness so that a

reasonable range of colour is visible, ie not too much black nor too much white

- the result should be a smooth transition of black-white that is even or

linear throughout the range.

Set R-G-B Balance

����������������������

This screen displays the RGB colour bars in intensities 0-63, and also an

alignment bar of r-g-b-r-g-b etc. Set the gun drive pots on the monitor or

colour balance pots so that the "height" of the RGB bars is even. This will

givew a correct spectral balance to the final picture and ensure that whites

are indeed WHITE. Ensure that no smearing or bleeding of colour occurs. If so,

reset the balance at a slightly lower bar height. This can be compensated for

later with the sub-brightness control.

Set Focus Screen (alignment menu, option 1, all systems)

������������������������������������������������������������

This screen displays randomly coloured blocks which are just right for doing

convergance and focus adjustments. The most important colours are yellow and

high intensity magenta, as these have the greatest offset of colours to align.

Check Purity Screen

�����������������������

This sequence of 3 screens displays solid red, green and blue for purity

testing. Purity problems are caused the disturbance of colour by stray magnetic

fields and usually manifest as blotches of wrong color onscreen. Examine the

screen for change of colour or shade. Purity is usually cleared by a mechanism

called the degausser on every COLD power-on, but strong fields sometimes won't

clear. Purity faults can also occur when moving the monitor with the power on

(magnetic orientation altered). To clear bad purity, turn monitor off, wait

until it gets cold (preferably overnight) and turn on again. DO NOT move

monitor during this sequence. If purity fault is still present, consult

professional repairer who has access to a degaussing wand.

Select VESA modes (UnAttended testing menu, option 4, VESA cards only)

��������������������������������������������������������������������������

Because not all monitors can handle all screen resolutions provided by some

better VESA cards, you can deselect certain resolutions so that the unattended

tests do not include these modes. By default, all VESA modes provided can be

tested.

For example: Most monitors cannot display XGA. Most 1mb cards provide an XGA

VESA mode. Repeatedly displaying this mode could be damaging to the monitor,

therefore you can deselect this test mode, and avoid damage. To test if the

monitor is capable of a given mode, use the graphics testing menu to manually

test each mode. Be sure to ESC out of any mode that does not display properly.

All test screens use the same "Grid" pattern, so detecting a mode you cannot

display is simple. Temporarily displaying a wrong mode should not be harmfull

to your monitor, but repeated display over a long term could be damaging. Some

monitors will 'die' when fed XGA resolutions as a result of their internal EHT

over-voltage shutdown: Simply turn the monitor off and on again to reset the

EHT shutdown circuit.

Test video card speed (Main menu, option 7, all cards)

����������������������������������������������������������

This routine tests the speed of the display adaptor in Characters per

Millisecond. Both direct hardware access and BIOS access is tested and reported

in real time. The effect of BIOS shadowing, Turbo modes, software drivers etc

may be assessed using this screen. Note that tests are performed in the default

screen mode (text screen) only. It is normal for speed in graphics modes to be

considerably slower. Montest does not, as yet, evaluate graphic mode

performance. This feature is planned for a later revision of Montest. The speed

tests are timed using the hardware timer and thus, are quite accurate and

independant of processor speed timing problems so commonly found in other test

software. The speed reductions noted when changing processor speeds is purely

the result of a slower host bus speed cycle time, etc and thus is a valid

benchmark of pure video speed.

Video Card Information (Main Menu, Option 6, All Cards)

�������������������������������������������������������������

The data accumulated when Montest first loads is displayed here. Most common

VGA/SVGA chipsets and cards are detected and displayed, along with their true

bus width/DAC type/RAM Size. Even Non-VESA cards are detected and in most

cases correct ram size is displayed. A more detailed log of this data will be

supplied when the detection routines are finalised in a future version.

Credits

�������������

Thanks to BCS, TCB, Bayside PC, Finn Thoegersen (VGADOC3.ZIP), Ralf Brown's

Interrupt list, SBCnet, FidoNet and ALL of you whom assisted in this project.

Montest is an ongoing project.  Please report all bugs, problems and wanted

features for future improvement. I am most anxious to hear from everyone

about this program.

I urge you to download VGADOCx.ZIP if you too require hardware information on

SVGA chisets, DAC's and SVGA info in general.

If you want the latest copy of Montest, or to contact Merlin, please call the

following:

Internet:	http://home.hyperlink.net.au/~chart

chart@hyperlink.net.au

Communications Barrier BBS:     +61-3-9585-1112    33.6k V34+

+61-3-9583-6119    28.8k Vfc

By Fidonet Netmail:   3:632/533 (FidoNet)

Bayside PC Services:  (Voice)     +61-3-9585-1645

(Fax)       +61-3-9585-1583

(Snail)     96 Nepean Hwy,

Mentone, 3194

Victoria, Australia.

<EOF>