DEC PATHWORKS installation with NE2000 Compatible NDIS driver

---------------------------------------------------------------

For

EtherPair/EtherCoax Family LAN Adapter Drivers

For Use in DEC Pathworks Environment

1. INSTALLATION INSTRUCTIONS

a. Insert the DEC PathWorks Client Setup diskette #1 into your

diskette drive (A: or B:)

b. type Netsetup at A:> or B: prompt, then press Enter to startup menu.

c. Unless you have an LK250 keyboard, Select "NO".

d. Select the DECNET transport option and press Enter.

e. Insert the DNET TRNSPRT diskette #1 into the diskette drive(A: or B:).

f. Specify the destination  drive letter (C or D)

g. Are you creating an initial workstation diskette? Select "NO".

h. Type in the node name and address(i.e node number).  Press Enter.

i. Type in the server node name and address(i.e node number), Press Enter.

j. Select "Ethernet" when prompt "Select the network adapter type":

k. Select "(Other NDIS)" when prompt "Select the network adapter":

l. Enter full DRIVER:\PATH\FILENAME for NDIS driver:

A:\ or B:\MSLANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

Located at  driver program diskette.

m. Enter full DRIVER:\PATH\ for the PROTOCOL.INI file:

A:\or B:\MSLANMAN.DOS\DRIVERS\ETHNE\ACCNE\

n. If they are all accurate, Select "WRITE KEY DISK" to start installation.

When prompt to copy files ETHNE.DOS and PROTOCOL.INI , Please insert

driver program diskette to diskette drive

(A: or B: drive dependent on path at step l. and m.)

o. Follow the prompts till completion.

p. At the Configuration screen, press F10 to exit to C:\

q. Using text editor to edit PROTOCOL.INI at DECNET sub-directory

Remove the semicolon (;) from the statement "NI_IRQ = 5"

under [DATALINK], Then check if the irq value in

the new [ETHNE.DOS] IRQ statement(maybe omitted) matches the NI_IRQ value

in the [DATALINK] statement and Adapter IRQ value when set.

Sample configuration files:

2.  CONFIG.SYS

shell=c:\command.com /P /E:526

device=c:\decnet\hiMEM.sys

device=C:\dos\EMM386.EXE ram 1024

dos=high, UMB

devicehigh=c:\decnet\protman.sys /I:D:\DECNET

devicehigh=c:\decnet\ethne.dos

rem device=c:\decnet\nddrv.sys

rem device=c:\decnet\npdrv.sys

FILES=50

BUFFERS=50

LASTDRIVE=v

STACKS=9,256

3.  PROTOCOL.INI file for Pathwork v 4.1

; PROTOCOL.INI file create by DNETOMO.V41 V4.1.060

; 			via DNETWIK.V41 V4.1.047

[protocol manager]

DRIVERNAME = PROTMAN$

[IPX4MAC]

DRIVER = IPX$MAC

BINDINGS	   = ETHNE.DOS

[DATALINK]

DRIVERNAME   = DLL$MAC

LG_BUFFERS   = 16

SM_BUFFERS   = 6

OUTSTANDING  = 32

HEURISTICS   = 0

BINDINGS	   = ETHNE.DOS

;Specify IRQ level used by workstations network adapter

NI_IRQ	   = 3

[ETHNE.DOS]

; PROTOCOL.INI for NE2000 Compatible Ethernet adapter

drivername = ETHNE$

; irq = NI_IRQ = Adapter IRQ Setting

irq = 3

iobase = 0x300

; End of this sample file

4.  AUTOEXEC.BAT

echo off

break on

cls

prompt $p$g

REM Insert any keyboard internationalization and character set information here.

if not exist \decnet\STARTNET.BAT goto nostartup

call \decnet\STARTNET

goto end

:nostartup

echo ** WARNING ** STARTNET.BAT file not found.  Network functions not performed

:end