SLICK 50 (& Others)

===================

Here is a compilation of articles on the Slick 50 articles I collected.

Slick 50, and other products such as T-Plus, contain an oil additive that

is based on PTFE, more commonly known as Teflon.

It is supposed to reduce engine wear at start up.

The first article is one I indirectly obtained from rec.boats and

rec.motorcycles (or something) which seems to be a transcript from a

magazine article from Road Rider on oil additives.

This is then followed by usenet testimonials, starting with an

article I transcribed from the LA times mixed with my own opinions.

Be aware of the differences between personal testimony, word of

mouth [most] and hard data [almost none].

The only thing that counts is hard data.

In summary, Slick 50 does not provide any long term benefits,

and in some instances may actually damage the engine by clogging

smaller oil passages.

jan@eds.ug.edu

----------------------------------------------------------------------------

Article: 23788 of rec.boats

Newsgroups: rec.boats

Reply-To: lmcjnwh@LMC.Ericsson.SE

Organization: Ericsson Communications Inc.

Date: Fri, 8 Oct 1993 23:38:36 GMT

X-Disclaimer: This article was posted by a user at Ericsson.

Any opinions expressed are strictly those of the

user and not necessarily those of Ericsson.

Not so long ago I got flamed on Rec.moto for being so foolish as to admit having

used Slick 50. Amidst the many flames I got a lot of useful info;

I am including the transcript of a magazine article (Road Rider)

investigating various oil additives, esp. the PTFE ones.

It makes interesting reading, especially if you have already used the

stuff!

-------------------------------------------------------------------------

Snake Oil!

Is That Additive Really A Negative?

Article and Photos by Fred Rau

Information for this article was compiled from reports and

studies by the University of Nevada Desert Research Center, DuPont

Chemical Company, Avco Lycoming (aircraft engine manufacturers),

North Dakota State University, Briggs and Stratton (engine

manufacturers), the University of Utah Engineering Experiment

Station, California State Polytechnic College and the National

Aeronautics and Space Administration's Lewis Research Center.

Road Rider does not claim to have all the answers. Nor do we

care to presume to tell you what to do. We have simply tried to

provide you with all the information we were able to dredge up on

this subject, in hopes it will help you in making your own, informed

decision.

You Can't Tell The Players Without A Program

On starting this project, we set out to find as many different

oil additives as we could buy. That turned out to be a mistake. There

were simply too many avail able! At the very first auto parts store

we visited, there were over two dozen different brand names

available. By the end of the day, we had identified over 40 different

oil additives for sale and realized we needed to rethink our strategy.

First of all, we found that if we checked the fine print on the

packages, quite a number of the additives came from the same

manufacturer. Also, we began to notice that the additives could be

separated into basic "groups" that seemed to carry approximately the

same ingredients and the same promises.

In the end, we divided our additives into four basic groups and

purchased at   least three brands from three different manufacturers

for each group. We defined our four groups this way:

1.)  Products that seemed to be nothing more than regular

50-rated engine oil (including standard additives) with PTFE (Teflon

TM) added.

2.)  Products that seemed to be nothing more than regular

50-rated engine oil (including standard additives) with zinc

dialkyldithiophosphate added.

3.)  Products containing (as near as we could determine) much

the same additives as are already found in most major brands of

engine oil, though in different quantities and combinations.

4.)  Products made up primarily of solvents and/or detergents.

There may be some differences in chemical makeup within groups,

but that is impossible to tell since the additive manufacturers

refuse to list the specific ingredients of their products. We will

discuss each group individually.

The PTFE Mystery

Currently, the most common and popular oil additives on the market

are those that contain PTFE powders suspended in a regular,

over-the-counter type, 50-rated petroleum or synthetic engine oil.

PTFE is the common abbreviation used for Polytetrafloeraethylene,

more commonly known by the tradename "Teflon," which is a registered

trademark of the DuPont Chemical Corporation.   Among those oil

additives we have identified as containing PTFE are: Slick 50, Liquid

Ring, Lubrilon, Microlon, Matrix, Petrolon (same company as Slick

50), QMl, and T-Plus (K-Mart). There are probably many more names in

use on many more products using PTFE. We have found that oil additive

makers like to market their products under a multitude of "private

brand" names.

While some of these products may contain other additives in

addition to PTFE, all seem to rely on the PTFE as their primary

active ingredient and all, without exception, do not list what other

ingredients they may contain.

Though they have gained rather wide acceptance among the

motoring public, oil additives containing PTFE have also garnered

their share of critics among experts in the field of lubrication. By

far the most damning testimonial against these products originally

came from the DuPont Chemical Corporation, inventor of PTFE and

holder of the patents and trademarks for Teflon. In a statement

issued about ten years ago, DuPont's Fluoropolymers   Division

Product Specialist, J.F. Imbalzano said, "Teflon is not useful as an

ingredient in oil additives or oils used for internal combustion

engines."

At the time, DuPont threatened legal action against anyone who

used the name "Teflon" on any oil product destined for use in an

internal combustion engine, and refused to sell its PTFE powders to

any one who intended to use them for such purposes.

After a flurry of lawsuits from oil additive makers, claiming

DuPont could not prove that PTFE was harmful to engines, DuPont was

forced to once again begin selling their PTFE to the additive

producers. The additive makers like to claim this is some kind of

"proof' that their products work, when in fact it is nothing more

than proof that the American legal ethic of "innocent until proven

guilty" is still alive and well. The decision against Dupont involved

what is called "restraint of trade." You can't refuse to sell a

product to someone just because there is a possibility they might use

it for a purpose other than what you intended it for.

It should be noted that DuPont's official position on the use of

PTFE in engine oils remains carefully aloof and non-commital, for

obvious legal reasons. DuPont states that though they sell PTFE to

oil additive producers, they have "no proof of the validity of the

additive makers' claims." They further state that they have "no

knowledge of any advantage gained through the use of PTFE in engine

oil."

Fear of potential lawsuits for possible misrepresentation of a

product seem to  run much higher among those with the most to lose.

After DuPont's decision and attempt to halt the use of PTFE in

engine oils, several of the oil additive companies simply went

elsewhere for their PTFE powders, such as purchasing them in other

countries. In some cases, they disguise or hype their PTFE as being

something different or special by listing it under one of their own

tradenames. That doesn't change the fact that it is still PTFE.

In addition, there is some evidence that certain supplies of

PTFE powders (from manufacturers other than DuPont) are of a cruder

version than the original, made with larger sized flakes that are

more likely to "settle out" in your oil or clog up your filters. One

fairly good indication that a product contains this kind of PTFE is

if the instructions for its use advise you to "shake well before

using." It only stands to reason that if the manufacturer knows the

solids in his product will settle to the bottom of a container while

sitting on a shelf, the same thing is going to hap pen inside your

engine when it is left idle for any period of time.

The problem with putting PTFE in your oil, as explained to us by

several industry experts, is that PTFE is a solid. The additive

makers claim this solid "coats" the moving parts in an engine (though

that is far from being scientifically proven).  Slick 50 is currently

both the most aggressive advertiser and the most popular seller, with

claims of over 14 million treatments sold.  However, such solids seem

even more inclined to coat non-moving parts, like oil passages and

filters. After all, if it can build up under the pressures and

friction exerted on a cylinder wall, then it stands to reason it

should build up even better in places with low pressures and

virtually no friction.

This conclusion seems to be borne out by tests on oil additives

containing PTFE conducted by the NASA Lewis Research Center, which

said in their report, "In the types of bearing surface contact we

have looked at, we have seen no benefit. In some cases we have seen

detrimental effect. The solids in the oil tend to accumulate at

inlets and act as a dam, which simply blocks the oil from entering.

Instead of helping, it is actually depriving parts of lubricant."

Remember, PTFE in oil additives is a suspended solid. Now think

about why you have an oil filter on your engine. To remove suspended

solids, right? Right. Therefore it would seem to follow that if your

oil filter is doing its job, it will collect as much of the PTFE as

possible, as quickly as possible. This can result in a clogged oil

filter and decreased oil pres sure throughout your engine.

In response to our inquiries about this sort of problem, several

of the PTFE pushers responded that their particulates were of a

sub-micron size, capable of passing through an ordinary oil filter

unrestricted. This certainly sounds good, and may in some cases

actually be true, but it makes little difference when you know the

rest of the story. You see, PTFE has other qualities besides being a

friction reducer:  It expands radically when exposed to heat. So even

if those particles are small enough to pass through your filter when

you purchase them, they very well may not be when your engine reaches

normal operating temperature.

Here again, the' scientific evidence seems to support this, as

in tests conducted by researchers at the University of Utah

Engineering Experiment Station involving Petrolon additive with PTFE.

The Petrolon test report states, "There was a pressure drop

across the oil filter resulting from possible clogging of small

passageways." In addition, oil analysis showed that iron

contamination doubled after using the treatment, indicating that

engine wear didn't go down - it appeared to shoot up.

This particular report was paid for by Petrolon (marketers of

Slick 50), and was not all bad news for their products. The tests,

conducted on a Chevrolet six-cylinder automobile engine, showed that

after treatment with the PTFE additive the test engine's friction was

reduced by 13.1 per- cent. Also, output horsepower increased from 5.3

percent to 8.1 percent, and fuel economy improved from 11.8 percent

under light load to 3.8 percent under heavy load.

These are the kind of results an aggressive marketing company

like Petrolon can really sink their teeth into. If we only reported

the results in the last paragraph to you, you'd be inclined to think

Slick 50 was indeed a magic engine elixir. What you have to keep in

mind is that often times the benefits (like increased horse power and

fuel economy) may be out weighed by some serious drawbacks (like the

indications of reduced oil pressure and increased wear rate).

The Plot Thickens

Just as we were about to go to press with this article, we were

contacted by the public relations firm of Trent and Company, an

outfit with a prestigious address in the Empire State Building, New

York. They advised us they were working for a company called QMI out

of Lakeland, Florida, that was marketing a "technological

breakthrough" product in oil additives. Naturally, we asked them to

send us all pertinent information,  including any testing and

research data.

What we got was pretty much what we expected. QMI's oil

additive, according to their press release, uses "ten times more PTFE

resins than its closest competitor." Using the "unique SX-6000

formula," they say they are the only company to use "aqueous

dispersion resin which means the microns (particle sizes) are

extensively smaller and can penetrate tight areas." This, they claim,

"completely eliminates the problem of clogged filters and oil

passages."

Intrigued by their press release, we set up a telephone

interview with their Vice- President of Technical Services, Mr. Owen

Heatwole. Mr. Heatwole's name was immediately recognized by us as one

that had popped in earlier research of this subject as a former

employee of Petrolon, a company whose name seems inextricably linked

in some fashion or another with virtually every PTFE-related additive

maker in the country.

Mr. Heatwole was a charming and persuasive talker with a knack

for avoiding direct answers as good as any seasoned politician. His

glib pitch for his product was the best we've ever heard, but when

dissected and pared down to the verifiable facts, it actually said

very little.

When we asked about the ingredients in QMI's treatments, we got

almost exactly the response we expected. Mr. Heatwole said he would

"have to avoid discussing specifics about the formula, for

proprietary reasons."

After telling us that QMI was being used by  "a  major oil

company,"  a "nuclear plant owned by a major corporation" and a

"major engine manufacturer," Mr. Heatwole followed up with,

"Naturally, I can't reveal their names- for proprietary reasons."

He further claimed to have extensive testing and research data

available from a "major laboratory," proving conclusively how

effective QMI was. When we asked for the name of the lab, can you

guess? Yup, "We can't give out that information, for proprietary

reasons."

What QMI did give us was the typical "testimonials," though we

must admit theirs came from more recognizable sources than usual.

They seem to have won over the likes of both Team Kawasaki and Bobby

Unser, who evidently endorse and use QMI in their racing engines. Mr.

Heatwole was very proud of the fact that  their product was being

used in engines that he himself admitted are "torn down and

completely inspected on a weekly basis." Of course, what he left out

is that those same engines are almost totally rebuilt every time

they're torn down. So what does that prove in terms of his product

reducing wear and promoting engine longevity? Virtually nothing.

Mr. Heatwole declined to name the source of QMI's PTFE supply

"for proprietary reasons." He bragged that their product is sold

under many different private labels, but refused to identify those

labels "for proprietary reasons." When asked about the actual size of

the PTFE particles used in QMI, he claimed they were measured as

"sub-micron in size" by a "major motor laboratory" which he couldn't

identify - you guessed it - for "proprietary reasons."

After about an hour of listening to "don't quote me on this,"

"I'll have to deny that if you print it," and "I can't reveal that,"

we asked Mr. Heatwole if there was something we could print.

"Certainly," he said, "Here's a good quote for you: 'The radical

growth in technology has overcome the problem areas associated with

PTFE in the I980s'"

"Not bad," we said. Then we asked to whom we might attribute

this gem of wisdom. DuPont Chemical, perhaps?

"Me," said Mr. Heatwole. "I said that."

QMI's press releases like to quote the Guinness Book Of Records

in saying that PTFE is "The slickest substance known to man." Far be

it from us to take exception to the Guinness Book, but we doubt that

PTFE is much slicker than some of the people marketing it.

The Zinc Question

The latest "miracle ingredient" in oil additives, attempting to

usurp PTFE's cure-all throne, is zinc dialkyldithiophosphate, which

we will refer to here after as simply "zinc."

Purveyors of the new zinc-related products claim they can prove

absolute superiority over the PTFE-related products. Naturally, the

PTFE crowd claim exactly the same, in reverse.

Zinc is contained as part of the standard additive package in

virtually every major brand of engine oil sold today, varying from a

low volume of 0.10 per cent in brands such as Valvoline All Climate

and Chevron l5W-50, to a high  volume of 0.20 percent in brands such

as Valvoline  Race  and  Pennzoil GT Performance.

Organic zinc compounds are used as extreme pressure, anti-wear

additives, and are therefore found in larger amounts in oils

specifically blended for high-revving, turbocharged or racing

applications. The zinc in your oil comes into play only when there is

actual metal-to-metal con tact within your engine, which should never

occur under normal operating conditions. However, if you race your

bike, or occasionally play tag with the redline on the tach, the zinc

is your last line of defense. Under extreme conditions, the zinc

compounds react with the metal to prevent scuffing, particularly

between cylinder bores and piston rings.

However - and this is the important part to remember - available

research shows that more zinc does not give you more protection, it

merely prolongs the protection if the rate of metal-to-metal contact

is abnormally high or extended. So unless you plan on spending a

couple of hours dragging your knee at Laguna Seca, adding extra zinc

compounds to your oil is usually a waste. Also, keep in mind that

high zinc content can lead to deposit formation on your valves, and

spark plug fouling.

Among the products we found containing zinc

dialkyldithiophosphate were Mechanics Brand Engine Tune Up, K Mart

Super Oil Treatment, and STP Engine Treatment With XEP2. The only

reason we can easily identify the additives with the new zinc

compounds is that they are required to carry a Federally mandated

warning label indicating they contain a hazardous substance. The zinc

phosphate they contain is a known eye irritant, capable of inflicting

severe harm if it comes in contact with your eyes. If you insist on

using one of these products, please wear protective goggles and

exercise extreme caution.

As we mentioned, organic zinc compounds are already found in

virtually every major brand of oil, both automotive and motorcycle.

However, in recent years the oil companies voluntarily reduced the

amount of zinc content in most of their products after research

indicated the zinc was responsible for premature deterioration and

damage to catalytic converters. Obviously this situation would not

affect 99 percent of all  the motorcycles on the road - however, it

could have been a factor with the newer BMW converter - equipped

bikes.

Since the reduction in zinc content was implemented solely for

the protection of catalytic converters, it is possible that some

motorcycles might benefit from a slight increase in zinc content in

their oils. This has been taken into account by at least one oil

company, Spectro, which offers 0.02 to 0.03 percent more zinc

compounds in its motorcycle oils than in its automotive oils.

Since Spectro (Golden 4 brand, in this case) is a synthetic

blend lubricant designed for extended drain intervals, this increase

seems to be wholly justified. Also, available research indicates that

Spectro has, in this case, achieved a sensible balance for extended

application without increasing the zinc content to the point that it

is likely to cause spark plug fouling or present a threat to

converter- equipped BMW models.

It would appear that someone at Spectro did their homework.

Increased Standard Additives

(More Is Not Necessarily Better)

Though some additives may not contain anything harmful to your

engine, and even some things that could be beneficial, most experts

still recommend that you avoid their use. The reason for this is that

your oil, as purchased from one of the major oil companies, already

contains a very extensive additive package.

This package is made up of numerous, specific additive

components, blended to achieve a specific formula that will meet the

requirements of your engine. Usually, at least several of these

additives will be synergistic. That is, they react mutually, in

groups of two or more, to create an effect that none of them could

attain individually. Changing or adding to this formula can upset the

balance and negate the protective effect the formula was meant to

achieve, even if you are only adding more of something that was

already included in the initial package.

If it helps, try to think of your oil like a cake recipe. Just

because the original recipe calls for two eggs (which makes for a

very moist and tasty cake), do you think adding four more eggs is

going to make the cake better? Of course not. You're going to upset

the carefully calculated balance of ingredients and magnify the

effect the eggs have on the recipe to the point that it ruins the

entire cake. Adding more of a specific additive already contained in

your oil is likely to produce similar results.

This information should also be taken into account when adding

to the oil already in your bike or when mixing oils for any reason,

such as synthetic with petroleum. In these cases, always make sure

the oils you are putting together have the same rating (SA, SE, SC,

etc.). This tells you their additive packages are basically the same,

or at least compatible, and are less likely to upset the balance or

counteract each other.

Detergents And Solvents

Many of the older, better-known oil treatments on the market do

not make claims nearly so lavish as the new upstarts. Old standbys

like Bardahl, Rislone and Marvel Mystery Oil, instead offer  things

like  "quieter  lifters," "reduced oil burning" and a "cleaner

engine."

Most of these products are made up of solvents and detergents

designed to dissolve sludge and carbon deposits inside your engine so

they can be flushed or burned out. Wynn's Friction Proofing Oil, for

example, is 83 percent kerosene. Other brands use naphthalene,

xylene,  acetone and isopropanol. Usually, these ingredients will be

found in a base of standard mineral oil.

In general, these products are designed to do just the opposite

of what the PTFE and zinc phosphate additives claim to do. Instead of

leaving behind a "coating" or a "plating" on your engine surfaces,

they are designed to strip away such things.

All of these products will strip sludge and deposits out and

clean up your engine, particularly if it is an older, abused one. The

problem is, unless you have some way of determining just how much is

needed to remove your deposits without going any further, such

solvents also can strip away the boundary lubrication layer provided

by your oil. Overuse of solvents is an easy trap to fall into, and

one which can promote harmful metal-to-metal contact within your

engine.

As a general rule of thumb these products had their place and

were at least moderately useful on older automobile and motorcycle

engines of the Fifties and Sixties, but are basically unneeded on the

more efficient engine designs of the past two decades.

The Infamous "No Oil" Demo

At at least three major motorcycle rallies this past year, we

have witnessed live demonstrations put on to demonstrate the

effectiveness of certain oil additives. The demonstrators  would

have  a bench- mounted engine which they would fill with oil and a

prescribed dose of their "miracle additive." After running the engine

for a while they would stop it, drain out the oil and start it up

again. Instant magic! The engine would run perfectly well for hours

on end, seemingly proving the effectiveness of the additive which had

supposedly "coated" the inside of the engine so well it didn't even

need the oil to run. In one case, we saw this done with an actual

motorcycle, which would be rid den around the parking lot after

having its oil drained. A pretty convincing demonstration - until you

know the facts.

Since some of these demonstrations were conducted using Briggs

and Stratton engines,  the  Briggs  and   Stratton Company itself

decided to run a similar, but somewhat more scientific, experiment.

Taking two brand-new, identical engines straight off their assembly

line, they set them up for bench-testing. The only difference was

that one had the special additive included with its oil and the other

did not. Both were operated for 20 hours before being shut down and

having the oil drained from them. Then both were started up again and

allowed to run for another 20 straight hours. Neither engine seemed

to have any problem performing this "minor miracle."

After the second 20-hour run, both engines were completely torn

down and inspected by the company's engineers. What they found was

that both engines suffered from scored crankpin bearings, but the

engine treated with the additive also suffered from heavy cylinder

bore damage that was not evident on the untreated engine.

This points out once again the inherent problem with particulate

oil additives: They can cause oil starvation. This is particularly

true in the area of piston rings, where there is a critical need for

adequate oil flow. In practically all of the reports and studies on

oil additives, and particularly those involving suspended solids like

PTFE, this has been reported as a major area of engine damage.

The Racing Perspective

Among the most convincing testimonials in favor of oil additives

are those that come from professional racers or racing teams. As

noted previously, some of the oil additive products actually are

capable of producing less engine friction, better gas mileage and

higher horsepower out put. In the world of professional racing, the

split-second advantage that might be gained from using such a product

could be the difference between victory and defeat.

Virtually all of the downside or detrimental effects attached to

these products are related to extended, long-term usage. For

short-life, high-revving, ultra-high performance engines designed to

last no longer than one racing season (or in some cases, one single

race), the long-term effects of oil additives need not even be

considered.

Racers also use special high-adhesion tires that give much

better traction and control than our normal street tires, but you

certainly wouldn't want to go touring on them, since they're designed

to wear out in several hundred (or less) miles. Just because certain

oil additives may be beneficial in a competitive context is no reason

to believe they would be equally beneficial in a touring context.

The Best of The Worst

Not all engine oil additives are as potentially harmful as some

of those we have described here. However, the best that can be said

of those that have not proved to be harmful is that they haven't been

proved to offer any real benefits, either. In some cases, introducing

an additive with a compatible package of components to your oil in

the right proportion and at the right time can conceivably extend the

life of your oil. However, in every case we have studied it proves

out that it would actually have been cheaper to simply change the

engine oil instead.

In addition, recent new evidence has come to light that makes

using almost any additive a game of Russian Roulette. Since the

additive distributors do not list the ingredients contained within

their products, you never know for sure just what you are putting in

your engine.

Recent tests have shown that even some of the most inoffensive

additives contain products which, though harmless in their initial

state, convert to hydrofluoric acid when exposed to the temperatures

inside a firing cylinder. This acid is formed as part of the exhaust

gases, and though it is instantly expelled from your engine and seems

to do it no harm, the gases collect inside your exhaust system and

eat away at your mufflers from the inside out.

Whatever The Market Will Bear

The pricing of oil additives seems to follow no particular

pattern whatsoever. Even among those products that seem to be almost

identical, chemically, retail prices covered an extremely wide range.

For example:

One 32-ounce bottle of Slick 50 (with PTFE) cost us $29.95 at a

discount house that listed the retail price as $59.95, while a

32-ounce bottle of T-Plus (which claims to carry twice as much PTFE

as the Slick 50) cost us only $15.88.

A 32-ounce bottle of STP Engine Treatment (containing what they

call XEP2), which they claim they can prove "outperforms leading PTFE

engine treatments," cost us $17.97. Yet a can of K Mart Super Oil

Treatment, which listed the same zinc-derivative ingredient as that

listed for the XEP2, cost us a paltry $2.67.

Industry experts estimate that the actual cost of producing most

oil additives is from one-tenth to one-twentieth of the  asking

retail price. Certainly no additive manufacturer has come forward

with any exotic, high-cost ingredient or list of ingredients to

dispute this claim. As an interesting note along with this, back

before there was so much competition in the field to drive prices

down, Petrolon (Slick 50) was selling their PTFE products for as much

as $400 per treatment! The words "buyer beware" seem to take on very

real significance when talking about oil additives.

The Psychological Placebo

You have to wonder, with the volume of evidence accumulating against

oil additives, why so many of us still buy them. That's the

million-dollar question, and it's just as difficult to answer as why

so many of us smoke cigarettes, drink hard liquor or engage in any

other number of questionable activities. We know they aren't good for

us - but we go ahead and do them anyway.

Part of the answer may lie in what some psychiatrists call the

"psychological placebo effect." Simply put, that means that many of

us hunger for that peace of mind that comes with believing we have

purchased the absolute best or most protection we can possibly get.

Even better, there's that wonderfully smug feeling that comes

with thinking we might be a step ahead of the pack, possessing

knowledge of something just a bit better than everyone else.

Then again, perhaps it comes from an ancient, deep-seated need

we all seem to have to believe in magic. There has never been any

shortage of unscrupulous types ready to cash in on our willingness to

believe that there's some magical mystery potion we can buy to help

us lose weight, grow hair, attract the opposite sex or make our

engines run longer and better. I doubt that there's a one of us who

hasn't fallen for one of these at least once in our lifetimes. We

just want it to be true so bad that we can't help ourselves.

Testimonial Hype vs. Scientific Analysis

In general, most producers of oil additives rely on personal

"testimonials" to advertise and promote their products. A typical

print advertisement will be one or more letters from a satisfied

customer stating something like, "1 have used Brand X in my engine

for 2 years and  50,000 miles and it runs smoother and gets better

gas mileage than ever before. I love this product and would recommend

it to anyone."

Such evidence is referred to as "anecdotal" and is most commonly

used to pro mote such things as miracle weight loss diets and

astrology.

Whenever I see one of these ads I am reminded of a stunt played

out several years ago by Allen Funt of "Candid Camera" that clearly

demonstrated the side of human nature that makes such advertising

possible.

With cameras in full view, fake "product demonstrators" would

offer people passing through a grocery store the opportunity to

taste-test a "new soft drink." What the victims didn't know was that

they were being given a horrendous concoction of castor oil, garlic

juice, tabasco sauce and several other foul-tasting ingredients.

After taking a nice, big swallow, as instructed by the demonstrators,

the unwitting victims provided huge laughs for the audience by

desperately trying to conceal their anguish and disgust. Some

literally turned away from the cameras and spit the offending potion

on the floor.

The fascinating part came when about one out of four of the

victims would actu ally turn back to the cameras and proclaim the new

drink was "Great" or "Unique" or, in several cases, "One of the best

things I've ever tasted!" Go figure.

The point is, compiling "personal testimonials" for a product is

one of the easiest things an advertising company can do - and one of

the safest, too. You see, as long as they are only expressing some

one else's personal opinion, they don't have to prove a thing! It's

just an opinion, and needs no basis in fact whatsoever.

On the other hand, there has been documented, careful

scientific analysis done on numerous oil additives by accredited

institutions and researchers.

For example:

Avco Lycoming, a major manufacturer of aircraft engines,

states, "We have tried every additive we could find on the market,

and they are all worthless."

Briggs and Stratton, renowned builders of some of the most

durable engines in the world, says in their report on engine oil

additives, "They do not appear to offer any benefits."

North Dakota State University conducted tests on oil

additives and said in their report, "The theory sounds good- the only

problem is that the products simply don't work."

And finally, Ed Hackett, chemist at the University of Nevada

Desert Research Center, says, "Oil additives should not be used. The

oil companies have gone to great lengths to develop an additive pack

age that meets the vehicle's requirements. If you add anything to

this oil you may upset the balance and prevent the oil from

performing to specification."

Petrolon, Inc., of Houston, Texas, makers of Petrolon and

producers of at least a dozen other lubrication products containing

PTFE, including Slick 50 and Slick 30 Motorcycle Formula, claim that,

"Multiple tests by independent laboratories have shown that when

properly applied to an automotive engine, Slick 50 Engine Formula

reduces wear on engine parts. Test results have shown that Slick 50

treated engines sustained 50 percent less wear than test engines run

with premium motor oil alone."

Sounds pretty convincing, doesn't it?

The problem is, Petrolon and the other oil additive companies

that claim "scientific evidence" from "independent laboratories,"

all refuse to identify the laboratories that conducted the tests or

the criteria under which the tests were conducted. They claim they

are "contractually bound" by the laboratories to not reveal their

identities.

In addition, the claim of "50 percent less wear" has never

been proven on anything approaching a long-term basis. Typical

examples used to support the additive makers' claims involve engines

run from 100 to 200 hours after treatment, during which time the

amount of wear  particles in the oil decreased. While this has proven

to be true in some cases, it has also been proven that after 400 to

500 hours of running the test engines invariably reverted to

producing just as many wear particles as before treatment, and in

some cases, even more.

No matter what the additive makers would like you to believe,

nothing has been proven to stop normal engine wear.

You will note that all of the research facilities quoted in this

article are clearly identified. They have no problem with making

their findings public. You will also note that virtually all of their

findings about oil additives are negative. That's not because we

wanted to give a biased report against oil additives - it's because

we couldn't find a single laboratory, engine manufacturer or

independent research facility who would make a public claim, with

their name attached to it, that any of the additives were actually

beneficial to an engine. The conclusion seems inescapable.

As a final note on advertising hype versus the real world, we

saw a television ad the other night for Slick 50 oil additive. The ad

encouraged people to buy their product on the basis of the fact that,

"Over 14 million Americans have tried Slick 50!" Great. We're sure

you could just as easily say, "Over 14 million Americans have smoked

cigarettes!"-but is that really any reason for you to try it? Of

course not, because you've seen the scientific evidence of the harm

it can do. The exact same principle applies here.

In Conclusion

The major oil companies are some of the richest, most powerful

and aggressive corporations in world. They own multi- million dollar

research facilities manned by some of the best chemical engineers

money can hire. It is probably safe to say that any one of them has

the capabilities and resources at hand in marketing, distribution,

advertising, research and product development equal to 20 times that

of any of the independent additive companies. It therefore stands to

reason that if any of these additive products were actu ally capable

of improving the capabilities of engine lubricants, the major oil

companies would have been able to determine that and to find some way

to cash in on it.

Yet of all the oil additives we found, none carried the name or

endorsement of  any of the major oil producers.

In addition, all of the major vehicle and engine manufacturers

spend millions of dollars each year trying to increase the longevity

of their products, and millions more paying off warranty claims when

their products fail. Again, it only stands to reason that if they

thought any of these additives would increase the life or improve the

performance of their engines, they would be actively using and

selling them - or at least endorsing their use.

Instead, many of them advise against the use of these additives

and, in some cases, threaten to void their warranty coverage if such

things are found to be used in their products.

In any story of this nature, absolute "facts" are virtually

impossible to come by.  Opinions abound.  Evidence that points one

direction or the other is avail able, but has to be carefully

ferreted out, and is not always totally reliable or completely

verifiable.

In this environment,  conclusions reached  by  known,

knowledgeable experts in the field must be given a certain amount of

weight. Conclusions reached by unknown, unidentifiable sources must

be discounted almost totally. That which is left must be weighed, one

side against the other, in an attempt to reach a "reasonable"

conclusion.

In the case of oil additives, there is a considerable volume of

evidence against their effectiveness. This evidence comes from

well-known and identifiable expert sources, including independent

research laboratories, state universities, major engine

manufacturers, and even NASA.

Against this rather formidable barrage of scientific research,

additive makers offer not much more than their own claims of

effectiveness, plus questionable and totally unscientific personal

testimonials. Though the purveyors of these products state they have

studies from other independent laboratories supporting their claims,

they refuse to identify the labs or provide copies of the research.

The only test results they will share are those from their own

testing departments, which must, by their very nature, be taken with

a rather large grain of salt.

---

+------------------------------------------------------+

| John Walsh : Ericsson Communications Inc.            |

| Internet   : lmcjnwh@lmc.ericsson.se                 |

|                                                      |

| [standard disclaimer]                                |

| --surely some mistake, officer!                      |

|                                                      |

+------------------------------------------------------+

Don Eilenberger

DISCLAIMER: I speak for me, and me alone!

----------------------------------------------------------------------------

From: jan@eds.ug.edu

The article on SLICK-50 in "Your Wheels" in the LA Times, 1992, claims that

Slick-50 does reduce engine wear.

The author had his oil tested at a lab before and after the treatment

(I believe he used it on a Honda with about 50kmi on it).

If I remember the article correctly, the test was "blind", i.e.

the lab did not know what had been done, but apparently figured it

out from the tests.

The results were a 2/3 decrease in metal particles indicating reduced

wear, which the lab claimed was typical for PTFE treatments.

The decrease was most prominent in iron particles (bearings) and

particles with molybdenum (rings).

However, the lab also mentioned that the treatment does not last

as long as claimed, from previous tests they had done.

Furthermore, Slick 50 only works on (cast) Iron parts, not Aluminum parts.

From another source I heard that DuPont does not endorse this product

for that very reason and because it is not permanent.

The author also recommended against similar products that require to

be shaken before usage (particles too large).

The author claimed improved gas mileage and reduced operating temperatures.

In my opinion, Slick 50 does exactly what it claims: Reduce engine friction

at start up.

Once the oil pressure is up, it will do little to reduce wear.

Results from the net indicate that it doesn't seem to harm the engine.

Other tests have however indicated that it may do engine damage because

of oil passage blockage.

I have used Slick-50 in two VW engines, noticed absolutely no difference

in either gas milage, oil temperature or performance (subjective).

Starting the car did seem a tad smoother, but that may have been purely

because of the oil change.

Conclusion IMHO: Slick-50 is of marginal, temporary benefit.

Use a good synthetic instead.

----------------------------------------------------------------------------

From: finnegan@navo.navy.mil

>A recent visitor to my house produced a copy of an article that

>apparently appeared in some sort of Automotive trade journal.  The

>article described the results of some testing that a German Auto

>club had done on the effects of Slick 50.  Does anyone have a copy

>of the article or have any idea of the source?  The test was

>performed using a VW 1.8 litre Golf.  The gist of the results were

>that horsepower was slightly increased, operating temperature was

>significantly decrease, but the treatment didn't last the full

>50,000 miles, so they recommended reapplication at 30,000 mi.

>

>Thanks,

>

>Kenneth

>finnegan@navo.navy.mil

From: blu@cellar.org (Blu-Max) writes:

>anh@cbnewsm.att.com (christopher.j.hapeman) writes:

>

>> recommended 'slicking' my '87 Golf GL's engine.

>>

>> Christopher Hapeman

>

>Well, perhaps, *I* call Slick 50 'slicking'.  Then again, he could just be

>

>Slick 50.  Ive used it in my two cars, a 89 Jeep YJ 4 cyd, and my 90 Golf.

>The Golf seemed to get more from it than the Jeep, my milage went up by about

>1/2 Gal, and it *did* seem to run smoother.  I could only guess that this

>also keep engine wear at a minimum.  However, the car did not seem to have

>any more power.  Over all, I'd say get the stuff, as it really can't HARM

>your engine.

>

>[  Dan Reed : An Extraordinary AnalCad Geek : SysOp of the THYRATRON BBS  ]

>[    USENET : blu@cellar.org                :       1 (609) 582-5410      ]

Subject: Re: Slick Gear oil treatment

From: welty@nunki.crd.ge.com (richard welty)

Date: 21 Mar 91 22:10:46 GMT

Sender: news@crdgw1.crd.ge.com

References: <1991Mar20.122939.1@dev7d.mdcbbs.com>

Organization: New York State Institute for Sebastian Cabot Studies

Lines: 18

In article <1991Mar20.122939.1@dev7d.mdcbbs.com> jan@dev7d.mdcbbs.com writes:

>And now something completely different...

>What about Slick 50 as GEAR OIL Treatment opposed to ENGINE Oil

>treatment (now sold in the local stores).

>Would seem helpfull for bearings, but the synchros? How will

>it affect changing gears?

quite frankly, i'll wager that you're better off going with

a quality synthetic gear lube like Redline, Amsoil, or

Agip SINT (all better than Mobil 1 gear lubes.)  i'm not

yet a convert to synthetic motor oils, but by god, i love

synthetic gear lubes.

richard

--

richard welty                                       welty@sol.crd.ge.com

518-393-7228, Infologic, 1400 Balltown Road, Niskayuna, New York

Path: isc-br!olivea!spool.mu.edu!yale.edu!qt.cs.utexas.edu!cs.utexas.edu!sun-barr!lll-winken!addvax.llnl.gov!lim

From: lim@addvax.llnl.gov (Doug Lim)

Newsgroups: misc.consumers

Subject: Slick 50 Summary

Keywords: Slick 50, Oil Additives

Date: 14 Dec 92 19:29:00 GMT

Sender: usenet@lll-winken.LLNL.GOV

Reply-To: DougLim@llnl.gov

Organization: LLNL

Lines: 223

S L I C K    5 0   A N D   S I M I L A R

P R O D U C T S   S U M M A R Y

Thanks to all those who responed to my posting about Slick 50 and similar

types of products on the market! This appears to be quite a controversial

subject. The total tally was 10 votes for these types of products and

19 votes against them. I have tried to compile an objective summary of the

results, but you may find it useful in evaluating the summary to know that

I decided AGAINST use of any of these types of products.

For those who don't know what these products claim to do - they are basically

products that are occasionally added with your oil when you change oil,

once every 50,000 miles. The products supposedly coat your engine parts to

reduce the wear during cold starts.

ARGUMENTS FOR

- "Not only did slick 50 protect my engine, but I'm getting better gas mileage,

better acceleration, and more power overall from my engine."

>Rebutal: "The ONLY thing you will hear about Slick 50 are personal

testimonials, which are next to worthless."

- "The most amazing I've seen is one called Durabond.  I went up and talked to

the guys in Buffalo (who did the unrehearsed info-mercial in their

high-performance engine shop) and they swear up and down that the stuff is as

good as the info-mercial says it is."

- "I am using the engine and manual transmission formulas and have been pleased

with the results....The transmission version I tried first.  It got me easier

shifting, lower noise (maybe psychological) and at least 2mpg more in the

city...."

>Rebutal: [Another personal testimonial.]

- "i often put molybdenum disuplhide based ``molyslip'' into the engine and

gearbox when i change the oil.  that's not snake oil, so far as i can

see: it's a genuine lubricant."

>Rebutal: [Another personal testimonial.]

- "While I believe in placebo's (and I put Slick 50 in my truck and noticed

no difference.  I don't use it anymore) there is a product that I've

used that I do think works, and I don't think that it's the placebo

effect at work.

The product that I'm talking about is called "Engine Restore".  It's

not supposed to be like Slick 50, but it is supposed to reduce

emmisions (oil burning)...I added the Engine Restore, and after about a week,

the puff of oil smoke (and the smell of oil) was gone...I don't believe that

I'm imagining the effect, either you smell oil, or you don't."

>Rebutal: [Another personal testimonial.]

"Some similar products use moly sulfide instead of teflon, and

I once saved an engine having had the forsight to poor in a can :-)...

I doubt that would be the case with moly sulfide, but haven't ever seen a

report - what's in the major (Valvoline?) oil that's being touted on tv

these days - you know, the adv with dozens of stand-mounted engines, which

all expire except the one using the Right Stuff :-)"

- "My roommate is a mechanical engineer for Carrier Corp. and at the

Univeristy last year, the Mechanical group did some testing with Slick 50 and

were very impressed with the results.  What the results are, and whether or not

one person can notice a difference, I do not know.  I would assume the

friction losses were significantly less than using regular oil alone."

- "I put some in my 1981 Pontiac wagon with 130,000 miles.  I

had the problem of running extremely hot during the summer and burning oil so

bad that I had to change it every 800 miles using 20 w50 and oil additive and

adding 2 quarts of oil in the time between changes.  After I did add it, my

engine ran cooler and now I can go 1300 miles between oil changes and I don't

have to add any extra oil.  It also seems to have improved my gas efficiency

slightly."

>Rebutal: [Another personal testimonial.]

ARGUMENTS AGAINST

- "I know of 2 mechanics that swear by it, and one that swears that you should

not put it in your engine. To be honest, I would rather believe the one that

told me not to because he has a lot more knowledge, and much much more

experience than the other 2, but I talked to him after I had already

put it in."

>Rebutal: [Another personal testimonial.]

- "As a long time Car&Driver reader, I can tell you this is sort of

a FAQ in the magazine. I don't remember their exact reasoning

but C&D does not suggest the use of such products. It has to

do something with the fact that if these prodcuts lived up

to their claims, than a major oil company would have latched

onto it and they would be advertised up to the hilt."

>Rebutal:

"I have heard people respond to this by saying that this argument is similar

to one that says that high-efficiency engines must be a joke because if

they weren't, some car maker would latch onto them and advertise them to

the hilt...Seems to me that a major Oil Company would make more money

assuring that every 3 months you put 4 or more quarts of oil (at $1.50 a

quart or so) in your car than saying spend $30 (or whatever Slick 50 costs)

for a one-time protection for 50,000 miles :). One thing for sure, the

advertising for Slick 50 is very slick :)"

- "'Road Rider' magazine, a 'Consumer Reports' type of magazine for

motorcycles (product reviews, accepts no advertising) did an extensive

investigation of Slick 50 and its cousins.  Short answer:  do you

engine a favor, stay away from the stuff...Quoting from the "Road Rider"

article:

The major oil companies are some of the richest, most powerful and

aggressive corporations in the world.  They own multi-million dollar

research facilities manned by some of the best chemical engineers money

can hire.  It is probably safe to say that any one of them has the

capabilities and resources at hand in marketing, distribution,

advertising, research and product development to equal to 20 times that

of any of the independant additive companies.  It therefore stands to

reason that if any of these additive products were actually capable of

improving the capabilities of engine lubricants, the major oil

companies would have been able to determine that and to find some way

to cash in on it.

Yet of all the oil additives we found, none carried the name or

endorsement of any of the major oil producers...

In the case of oil additives, there is a considerable volume of

evidence against their effectiveness.  This evidence comes from

well-known and identifiable expert sources, including independent

research laboratories, state universities, major engine manufacturers,

and even NASA."

- Slick 50 contains powdered PTFE (polytetrafloeraethylene), more

commonly known as "Teflon," a trademark of DuPont Corp.  The theory is

that the teflon coats the metal parts in your engine, reducing wear

during start-up, before the oil has circulated through the engine.

That's the theory.  In reality, one of the favorite places the teflon

ends up coating is the inside of the oil passages, reducing their

diameter and restricting oil flow to the engine.  The result is oil

starvation and *excessive* wear of the metal in the engine, exactly the

opposite of what the addative claims to do!"

- "DuPont has claimed it is useless in engine oil.  Briggs & Stratton,

after a thorough, controlled, engineering analysis of Slick 50, found

that it caused excessive engine wear, compared to an identical engine

run with regular oil.  PTFE is freely available (ie, the makers of

Slick 50 cannot claim it is some "miracle new technology" they have

exclusive rights to), yet NO major oil manufacturers use it or

recommend its use as an addative.  NO major automobile, motorcycle, or

lawn mower motor manufacturer recommends its use (which, if it does

what it claims, should save them millions in warrantee repair work)."

- "Independant research laboratories, state universities, major engine

manufacturers, and NASA have concluded that at *best*, PTFE in engine

oil does no harm.  Slick 50's makers, while claiming to have extensive

test data from independant laboratories, refuse to release the data, or

even name the labs.  DuPont Corp., a multinational, billion dollar

chemical company, who invented the stuff,  and trademarked the name

"teflon" for PTFE, has concluded that it does no good in engine oil,

and tried to refuse to sell it to the makers of Slick 50, which a court

disallowed on "freedom of trade" grounds."

>Rebutal:

"I find this odd considering that I believe DuPont actually backs one of

this Telflon oil products... Is it T-Base ?

I'll have to check the next time I'm in a parts store..."

- "Briggs & Stratton took two identical engines off their assembly line,

and set them up for bench testing.  After break-in, one was run on

Slick 50, the other on straight motor oil for 20 hours.  They were both

shut down and the oil drained, then both were started up and run for

another 20 hours.  Both engines were then completely torn down and

inspected by the company's engineers.  Both suffered from scored

crankpin bearings, but the Slick 50 engine also suffered from heavy

cylinder bore damage that was NOT evident on the untreated engine!  The

engineers attributed this to oil starvation due to restricted oil flow

passages in the treated engine (the PTFE is *supposed* to adhere to the

metal, and it does... in the interior of oil passages, reducing their

diameter).  Once you block the passages, all future oil flow is

restricted."

>Rebutal:

"Wow! We've got data on two Briggs & Stratton engines that show severe

wear after only 20 hours of running on Slick 50. I expected

something of more statiscal significance from a man who takes his

laboratory data so seriously.

This Slick 50 scare...[if real]...would result in a multi-million dollar

class-action suit against its manufacturer."

- "The University of Nevada Desert Research Center, DuPont Chemical

Company, Avco Lycoming (aircraft engine manufacturers), North Dakota

State University, Briggs and Stratton, the University of Utah

Engineering Experimental Station, California State Polytechnic College,

and NASA's Lewis Research Center have ALL studied Slick 50, and NONE of

them have found ANY evidence of its effectiveness."

"Also, a good arguement is made for not mixing the stuff with oil that is

already carefully blended with additives, etc."

- '...By far the most damning testimonial against these products

originally came from the DuPont Chemical Corporation, inventor of PTFE and

holder of the patents and trademarks for Teflon.  In a statement issued

about ten years ago, DuPont's Fluoropolymers Division Product Specialist,

J.F. Imbalzano said, "Teflon is not useful as an ingredient in oil additives

or oils used for internal combustion engines."'

- "...Additive manufacturers will claim that they use "sub-micron" sized

particles that will pass through your oil filter.  The only problem is

that, "PTFE expands radically when exposed to heat."  So when your engine

reaches normal operating temparatures, the particles may not longer pass

through your filter, but instead will clog it!"

| | | |     +----------------------------------------+

| | | |     |      Doug Lim - DougLim@llnl.gov       |

| | | |__   | Lawrence Livermore National Laboratory |

| | |___/   |        Computer Scientist - ASD        |

\ \____/    |    (510) 423-6019 FAX: (510)423-2419   |

\____/     +----------------------------------------+

--

Carl Paukstis, RRR&RSG  S.I. 2.27     | "If we were directed from Washington

Olivetti North America / Spokane, WA |  when to sow and when to reap, we would

Mail: carlp@mail.spk.olivetti.com    |  soon want for bread."

or: ...uunet!oliveb!isc-br!carlp   |     - Thomas Jefferson

Article 11551 of rec.autos.vw:

Relay-Version: EDS Unigraphics News Server 14/03/90 VAX/VMS V5.5; site ug.eds.com

Path: ug!uunet!wupost!howland.reston.ans.net!newsserver.jvnc.net!erenj.com!bdboyle

Newsgroups: rec.autos.vw

Subject: Re: [A] Slick 50 synthetic oil

Message-ID: <1993Mar3.134543.9765@erenj.com>

From: bdboyle@erenj.com (Bryan D. Boyle)

Date: Wed, 3 Mar 1993 13:45:43 GMT

Sender: news@erenj.com (ERNewsE News Gateway Facility)

References: <2MAR199313230372@cc.utah.edu>

Organization: Exxon Research and Engineering Co., NJ

Nntp-Posting-Host: maverick1.erenj.com

Lines: 30

In article <2MAR199313230372@cc.utah.edu> mgb5528@cc.utah.edu (MICHAEL G. BECK) writes:

>  Hi there. I have a '67 bug and was wondering what the advantages are of

>putting in that SLICK 50 stuff into my engine.  Have there been any reports

>that it is bad for air cooled engines, or is it just a snake oil scam?

>

>Thanks

>Michael Beck

>mgb5528@cc.utah.edu

>

NIST (national institute of science and technology) did an

investigation about slick50 (which is oil basestock with

teflon powder added...the materials will cost you about $1.50

if you want to mix it yourself in a colloidal mill...) and found

that there is no difference between the lubricating properties

of a good mineral-based oil formulated the way most are today and

the claims made for slick50.  NIST additionally found that when the

slick50-contaminated oil was drained, there was no teflon left inside

of the crankcase or on journal bearings or lifter surfaces.

My opinion (and I know I work for one of the "seven sisters", but I'm

a data jock full time...and this is MY OPINION now...): buy a good

semisynthetic (mobil1, castrol, etc.) and change it regularly.  The

detergents, viscosity improvers, anti-friction modifiers will do more

to enhance your motor's longevity than slick50.

--

Bryan D. Boyle              |Physical: Exxon Research, Annandale, NJ 08801

Senior Engineer             |Logical: bdboyle@erenj.com

#include <disclaimer>       |Virtual: (908) 730-3338

**********The Clinton Presidency, day 39: America Under Siege.**********

From usc!cs.utexas.edu!uunet!lhdsy1!nntpserver.chevron.com!news Tue Jun 29 18:46:55 PDT 1993

Article: 58435 of rec.autos.tech

Path: usc!cs.utexas.edu!uunet!lhdsy1!nntpserver.chevron.com!news

From: SJJE@CHEVRON.COM (Scott Jessen)

Newsgroups: rec.autos.tech,rec.autos

Subject: Re: SLICK 50 / PTFE OIL ADDITIVES

Message-ID: <1993Jun29.141711.21866@nntpserver.chevron.com>

Date: 29 Jun 93 14:17:11 GMT

References: <1viqg5$7ul@news.cerf.net> <NV.740157375@neckweed> <6404@prcrs.prc.com>

Sender: news@nntpserver.chevron.com (USENET News System)

Organization: Chevron Information Technology Co.

Lines: 49

Xref: usc rec.autos.tech:58435 rec.autos:111448

In article <6404@prcrs.prc.com>, jkupiec@prcrs.prc.com (John Kupiec) says:

>

>In article <NV.740157375@neckweed> jdc@ideas.com (J. David Cover) writes:

>>pnc@nic.cerf.net (Pacific Numerix Corporation) writes on the subject of

>PTFE additives:

>>

>

>>(material deleted)

>>  I have used TufOil, which is basically the same thing as

>>Slick-50, and cannot give it high marks.  I just realized that I am having

>>problems with my valve cover gasket leaking....

>>(more deleted)

>> I do not want to use any PTFE

>>additive ever again in any of my cars.  I also did not notice any benefit

>>to adding it....

My thoughts:

I have seen a lot of pros and cons written and attributed to PTFE additives.

I myself have only used the Slick 50 brand additive.  I can report NO

negative side effects in the 5 vehicles I have used Slick 50 in (one is a race

car). Another vehicle has had no drop in compression in 50K miles -- I can't say

that using a good oil is or isn't helping (I've used several brands)....

I can report that a friend has had some rather convincing pro Slick 50 instances:

One is that his wife drove his Porsche 356 for 20 miles ignoring "the little

green light" (no oil pressure), he expected to find a ton of metal in the oil

screen -- there was none, compression is fine and the car is still running.

My friends next door neighbor runs a refrigerated trucking company.  He was

having frequent motor failures on his compressor drive motors (they run

continually).  He "Slick 50'd" his entire fleet 3 years ago and has not had

a single failure to date (I find this particularly impressive).  Another friend

had a leak in an oil/water cooler in his boat and lost all the engine oil.  He

did not know how long he had been running at near full throttle when he

finally noticed the oil pressure was zero.  Since he was miles from home,

with no tow in sight he limped his boat at near idle all the way home (1+

hour).  After fixing the problem and filling up with oil the boat motor checked

out 100% (compression, speed, etc.).

I know these are just testimonials, but they are people I know.  I'm convinced

and will continue to use the product.  By the way -- if this stuff can cause so

much damage (supposedly), why don't we read of major law suits or

independent labs claiming fraud??

Can't wait to read the responses.....

Of course....  The opinions I've expressed are mine and mine alone........

========================================================================

From cc.usu.edu!news.cs.utah.edu!dog.ee.lbl.gov!agate!hilbert.dnai.com!news.csufresno.edu!rsharp Wed Sep 28 12:26:30 PDT 1994

Article: 341 of rec.autos.sport.tech

Relay-Version: ANU News - V6.1 08/24/93 VAX/VMS V1.5; site cc.usu.edu

Path: cc.usu.edu!news.cs.utah.edu!dog.ee.lbl.gov!agate!hilbert.dnai.com!news.csufresno.edu!rsharp

Newsgroups: rec.autos.tech,rec.autos.rod-n-custom,rec.autos.sport.tech

Subject: **PTFE**  What Independent Researchers Say about  Slick 50 et al

Message-ID: <rsharp.38.0006AD81@hopf.dnai.com>

From: rsharp@hopf.dnai.com (<<BOBCHIP>>)

Date: Sat, 24 Sep 1994 06:40:37

Organization: New Age Technology Group  (Focus on Education)

Keywords: PTFE Slick50 DuPont Teflon

Summary: Million dollar study which may shed more light on prolonging engines

NNTP-Posting-Host: dynamic-203.dnai.com

X-Newsreader: Trumpet for Windows [Version 1.0 Rev A]

Lines: 252

Xref: cc.usu.edu rec.autos.tech:34752 rec.autos.rod-n-custom:5062 rec.autos.sport.tech:341

I posted this article in another group last week.  I received so many

responses (mainly asking me to repost the article because of a formatting

error of mine which cut some lines short), that I thought I should spread it

around a little more.

Enjoy!

__

/      )              /                   /

/ - -< _  _  __/_ __  _ .     /_    o   _

/___/   ( _ )   / _  )  ( __   /    /_<_ /_ )_

/

****************************************************

*****************************************************

Car  Care

by Rik Paul

(Motor Trend magazine October 1994.  page 148.)

Navigating the Maze of Friction-Reducing Formulas

One of the biggest phenomenons to hit the engine-care

scene over the last few years has been the wave of

friction-reducing anti-wear engine treatments.  Over the

years we've received more reader questions about these

products than any other car-care subject.  It also seems

as though we receive notices of new entries in this

category almost weekly.  The main claim of these products

is the ability to reduce internal engine wear ...

particularly during the critical cold-start period ...

resulting in longer service life.  Some also make

additional claims regarding increased performance and

fuel economy resulting from the reduced friction.

To the average consumer all of the hype claims and

counter-claims can seems as confusing to navigate as a

bayou.  Part of the problem is due to the fact that

objectively evaluating these products in order to

substantiate their anti-wear claims is almost impossible

without prohibitively expensive lab tests.  Which of

course provides fertile ground for exaggerated assertions

and inflated statistics.  Meanwhile  public perception of

these friction-reducers ranges from miracle treatments to

modern-day snake oils.  The truth however probably rests

somewhere in between.

One of the common additives found in many formulas is a

polymer called polytetrafluoroethylene or PTFE.

Originally discovered by DuPont in the late '30s it's

commonly known by that manufacturer's trade name Teflon

and is considered to be the most slippery solid substance

known to man.  As an engine treatment it's added to

engine oil as microscopic particles.  The particles

purportedly bond to internal engine surfaces  reducing

the friction of moving parts such as bearings and rings

thereby decreasing wear and improving efficiency.

PTFE's history as an engine oil additive though is spotty

due largely to the fact that not all formulas are created

equal.  It was first used for this application in the

'70s but by 1980 DuPont had decided to discontinue Teflon

sales to oil-additive manufacturers a ban that remained

in place for about a decade.  One problem was that anyone

could .. and still can ... buy the polymer in bulk add it

to carrier oil and package it as their own formula

without sufficient technical expertise to make it work.

In some products the PTFE reportedly separated from the

oil and settled in the oil pan.  Plus there was concern

over particle build-up clogging oil filters and passages.

Such problems with these early versions has left a stigma

that still lingers.

Petrolon got the current bandwaon rolling about five

years ago when it introduced a reformulated Slick 50

Engine Treatment.  The product has been heavily marketed

and is currently the category's undisputed sales leader.

In fact in '92 it held a 90-percent share of the

engine-treatment market.  Petrolon claims that Slick 50

reduces engine wear at start-up and during operation for

over 50,000 miles.

A few years ago the company also made claims regarding

improvements in performance and fuel economy  but in 1992

the National Advertising Division of the Council of

Better Business Bureaus asked Petrolon to discontinue

such assertions due to lack of substantiating evidence.

Although Petrolon was targeted for this review largely

because of its size the same scrutiny should be applied

to any of the products in this category.

As in most current products the PTFE particles in Slick

50 are maintained in a colloidal suspension in which they

are electrically charged to repel each other.  This keeps

them from bonding or coagulating which not only keeps the

PTFE from setting out of the oil but also prevents the

clogging of oil filters.

This charge is gradually lost and sources vary on how

long PTFE remains effective.  Although Slick 50 claims it

reduces wear over 50,000 miles others say the real value

of PTFE is more short-term with its effectiveness

possibly beginning to erode after 10,000 to 20,000 miles.

Hilton Oil Corporation the company that markets T-Plus

another compound containing PTFE says that over a period

of time and mileage, PTFE levels in the engine are

gradually reduced.  Subsequently the company offers a

T-Plus Booster to replenish the PTFE.

Until recently DuPont has maintained a neutral stance on

the use of Teflon as an oil additive neither confirming

nor promoting any claimed benefits.  Last year however

Petrolon and DuPont signed a technology-sharing agreement

to cooperate in development of new lubricant formulas and

other products incorporating PTFE as well as the possible

development of new polymers and co-polymers for this

purpose.

What about the problem of clogged oil filters?  The Fram

Filter Division of Allied-Signal Aftermarket Group

conducted a study in 1992 to determine just this point

specifically targeting Slick 50 and a competitor named

Tufoil.  The study found that the PTFE particles in those

formulas freely flow through typical oil filters used on

passenger-car engines with no danger of clogging.  The

average size of a PTFE particle is about 0.2 to 0.3

microns  which is well below the 35-micron pore size of

typical passenger-car oil filters.  Petrolon however

advises that use of Slick 50 isn't recommended with

systems that filter particles 5 microns or smaller.

Slick 50 shares the scene with numerous other products

each with its own formula and claims.  T-Plus for

instance claims it has 50 percent more PTFE while QMI

claims 10 times more PTFE than its nearest competitor.

Petrolon counters that an engine can only use so much

PTFE  just as a human body can only absorb so much

vitamin C. Products such as Tufoil and OEM combine PTFE

with molybdenum a soluble heavy-metal compound which

their manufacturers claim provides increased anti-wear

characteristics.  Meanwhile a company called Engine-slick

advertises that it uses a unique interface bonding

technique instead of the more common colloidal suspension

which interlocks Teflon particles together to provide 99

to 100 percent coverage on internal engine surfaces.

Other companies take non-PTFE approaches to the problem.

For instance First Brands' STP Engine Treatment uses a

formula it calls XEP2 composed of various agents which is

also claimed to bond to internal engine surfaces.

Castrol's Syntec FSX contains a negatively charged

chemical ester that is said to bond molecularly with

positively charged engine parts and uses a 5W-50

synthetic carrier oil. Power Up employs NNL-690 a

lubricant that reportedly changes from a liquid to a

solid (in the form of minute particles) as the conditions

change from light to heavy loads.

So how can you tell what works and what doesn't?  In most

cases buyers need to sift through the literature and try

to read between the lines; not exactly a reliable method.

So it was with interest that we met with representatives

from Petrolon who brought stacks of hard numbers from an

extensive series of tests conducted last year.  Here are

the details in a nutshell.

The tests were done by Southwest Research Institute in

San Antonio Texas an independent facility monitored by

the American Society for Testing and Materials (ASTM).

Two different tests were conducted at a cost of about $1

million: a start/stop test and a continuous operation

test each designed to simulate 50,000 miles worth of

operation.  The tests were extended versions of a

Sequence IIIE test a widely accepted automotive and

petroleum procedure for motor oils that simulates

stressful highway driving.  Six industry-standard Buick

V-6 engines were used for each phase;  three were treated

with Slick 50 and three were not to serve as controls.  A

5W-30 SG premium-quality oil was used in all engines.

The parts to be examined including piston rings rocker

arms and connecting rod and main bearings were weighed

both before and after the tests to determine the amount

of wear.

In the start/stop test the engines were started idled for

one minute accelerated to the equivalent of 50 mph for 10

minutes to simulate a short trip and then turned off.  A

total of 330 such repetitions were performed.  Then the

oil and filter were changed (no new Slick 50 was added)

and the engines run for four hours at the equivalent of

70 mph to flush them.  The oil was drained and the filter

removed.  Then a series of 500 start/stop sequences were

conducted without oil to simulate dry starts in which the

engine was started  revved to 1000 rpm and shut off.

In the continuous-operation test  the engines were run

for 480 hours at 3000 rpm  which is estimated to simulate

50 000 miles.  The oil and filter were changed

every 3000 miles with Slick 50 added to the appropriate

engines during only the first oil change.

After both test were completed  all engines were

disassembled and measured for wear.  The following results

show the average reduction of wear in the parts

used in the Slick 50-treated engines:

Start/Stop

Rod bearings .................... 55 percent

Main bearings ................... 34 percent

Piston rings .................... 43 percent

Rocker arms ..................... 22 percent

Average of all parts ............ 39 percent

Continuous Operation

Rod bearings .................... 44 percent

Main bearings ...................  8 percent

Piston rings .................... 25 percent

Rocker arms ..................... 18 percent

Average of all parts ............ 24 percent

Petrolon noted that anti-wear protection was best between

10 000 and 50 000 miles   though there was no way to

project within this testing format whether more frequent

PTFE treatments would've given better results.

According to these tests  Slick 50 provides significant

anti-wear benefits.  Many other companies however lack

the funds to verify their products' claims to

conclusively.  Therefore common sense should be used when

shopping.

* Approach claims of life-time treatments and specific

percentage   increases in mileage or performance with a

healthy skepticism.  Even  if better gas mileage or

performance is technically true the  improvements will

likely be subtle.

* PTFE is inert won't melt at engine temperatures and has

virtually  no expansion rate.  Beware of claims that

contradict these properties.

* If a PTFE product requires you to shake the can  the

particles likely will separate and settle into your oil

pan.  Avoid these compounds.

* Remember that the anti-wear benefits of these engine

treatments won't  provide a payoff until many miles down

the road.  If you're the type that gets a new car every

five years  then it likely will be future  owners who'll

thank you for using an anti-wear product.  However  if

you keep vehicles for a long time then a

friction-reducing formula may be worth the investment.

From cc.usu.edu!news.cs.utah.edu!cs.utexas.edu!howland.reston.ans.net!spool.mu.edu!news.nd.edu!bsuvc.bsu.edu!01keanderson Wed Sep 28 12:28:18 PDT 1994

Article: 346 of rec.autos.sport.tech

Relay-Version: ANU News - V6.1 08/24/93 VAX/VMS V1.5; site cc.usu.edu

Path: cc.usu.edu!news.cs.utah.edu!cs.utexas.edu!howland.reston.ans.net!spool.mu.edu!news.nd.edu!bsuvc.bsu.edu!01keanderson

Newsgroups: rec.autos.tech,rec.autos.sport.tech

Subject: Re: **PTFE** What Independent Researchers Say about Slick 50 et al

Message-ID: <1994Sep25.105304.34628@bsuvc.bsu.edu>

From: 01keanderson@bsuvc.bsu.edu

Date: 25 Sep 94 10:53:03 -0500

References: <rsharp.38.0006AD81@hopf.dnai.com> <Pine.SOL.3.90.940924205819.3410K-100000@gladstone>

Organization: Ball State University

Lines: 29

Xref: cc.usu.edu rec.autos.tech:34852 rec.autos.sport.tech:346

In article <Pine.SOL.3.90.940924205819.3410K-100000@gladstone>, Daniel Joshua Stern <dastern@gladstone> writes:

> That "independent research article" looks strikingly like one I saw

> around 1987 or 88-- can you say "PAID ADVERTISING," boys and girls?  Na,

> I'm much more inclined to believe major, established engine companies

> like Briggs and Stratton, Lycoming and Chrysler, all of whom tested these

> PTFE products and found WORSE wear in the treated engines.

> By the way, simply "accelerating the engine to a speed equivalent to 50

> miles per hour" is NOT analogous to all the friction dynamics involved in

> a short trip--not by a LONG shot!  This is not a flame at the poster who

> posted this message--it is a disgusted rebuttal at the misleading

> articles these companies pay to have printed.

>

> SL6 Dan

This article says the data from this so-called independent research

was brought by Petrolon to be used in the article.  That doesn't sound

very independent to me.  It also states that DuPont has made no

statements about PTFE engine treatments until recently.  This is not

true.  DuPont attempted to prevent the use of the Teflon name in oil

additives because of it's well known ill effects.  DuPont even took

this issue to court.  Slick50 is garbage, and should never be added to

engine oil.  If this stuff was so great oil manufacturers would

include it in their oil regularly.  The fact is any oil additive will

dilute oil and reduce it's effectiveness.  PTFE is the worst since it

will clog oil passages, and will damage the engine.

Kriss Anderson

01keanderson@bsu.edu