Documentation for Parallel.exe

==============================

It is possible to closely approximate any arbitrary value of resistance from two standard values placed in parallel. This has some advantages when the formula you have used to design a filter or other electronic circuit gives you a non standard value. You either have to use the nearest standard value and accept the inaccuracy, or order expensive high spec high accuracy values from the E96 range,even though you don't need 0.1% accuracy, or sit down with a calculator working out parallel combinations.

The purpose of this program is to work out the values of resistors which when placed in parallel will give the resistance value that you want.

When the program starts it asks you for a target resistance and then tries to match it to a sorted list of pairs stored in the program, because it uses a binary search to locate the values it usually ends up very close but may not be on the closest pair, this usually doesn't matter as it is likely to be well within the resistor tolerance. Having found the value you may nudge it up or down to get different combinations, more accuracy or just to suit the resistors you happen to have.

K,k,M,m,R or r may be appended to the value as a multiplier i.e. 34k5  34500  34500R  34.5K  0M0345 are all valid inputs. The letters k or K multiplies the value by 1000. The letters M or m multiplies the value by 1000000, yes I know lower case m should represent milli ( divide by 1000 ) but there isnt much need for accurate parallel combinations in this range and so for convenience the program interprets m as 1000000. R and r are simply interpreted as decimal points.

The program must be used with some common sense, it assumes that you have perfect resistors. If you have 5% resistors then all you can gaurantee is that the combination will be within 5%, there is little point in fine tuning the combination to 0.01%.

You may select between the E12 range and the E24 range. The E12 range has 12 values per decade and is usually 5%, the E24 range has 24 values per decade, they are usually 2% or 1% tolerance.

The E12 values are 1.0  1.2  1.5  1.8  2.2  2.7  3.3  3.9  4.7  5.6  6.8 and 8.2

The E24 values are 1.0  1.1  1.2  1.3  1.5  1.6  1.8  2.0  2.2  2.4  2.7  3.0  3.3  3.6  3.9  4.3  4.7  5.1  5.6  6.2  6.8  7.5   8.2 and 9.1

Keyboard Commands

-----------------

Nudge Up   = U, up arrow or F5

Nudge Down = D, down arrow or F6

New Value  = N or F3

E12 series = F7

E24 series = F8

Quit       = ESC, Q or F10

I wrote this program about two years ago to save myself time. It had a simple Scanf Printf interface. Recently I got my hands on a HP200LX and found that the program was much more usefull on a palmtop than on a large PC. Then PAL came along and I added all the bells an whistles. I have not supplied the source code because this is a classic quick and dirty program, one long main(), very few supporting functions and little regard to structure or style, it's only redeeming feature is that it works!

The latest version of this program was written using the Palmtop Application Library, (c)1994 The PAL Group. Thank you to all the PAL contributors for all the work you have put into creating this excellent tool.

I Paul J Miller disclaim all warranties, expressed or implied, including without limitation, any warranties of fitness for any purpose.

I Paul J Miller assume no liability for damages, direct or consequential, which may result from the use of this program.

All I can say is that it seems to work reliably on my machine.

This program is copyrighted freeware. You are not required to pay for this program but if you find it usefull then please send me an E-mail to let me know ( also if you REALLY do want the source code, but be warned..... ).

My CompuServe address is 100527,3576.

Paul J. Miller