==============================================================

Beep MIDI Player LXMIDI ver. 1.40

Copyright (C) 1996 HARUYA

==============================================================

1.Introduction

LXMIDI is a software MIDI synthesizer designed for

HP100LX/200LX.  It conducts real-time conversion from MIDI data

(in Standard MIDI File Format 0) into 8- to 16-part polyphonic

multitimbral square-wave PCM data and plays it with the

internal speaker of 100LX/200LX.

Although wave form and the number of parts (timbres) are

inferior to normal MIDI sound source, implementation of

important MIDI events such as percussion set (GM standard),

pitch modulator, pitch bend, portamento and others, enables

high fidelity to the original music, if it has relatively low

number of voices.  Reverb effect is applied to help adding a

depth to the sound.

For songs with large number of voices, LXMIDI tried to

make a compromise between the parts, so the atmosphere

of the originals should not be deteriorated.

It can be used also on IBM-PC compatibles, with '-N2' option,

or on NEC PC-98, with '-N9' option.

This program makes full utilization of high speed hardware

interrupts for creation and execution of PCM data.  Therefore,

collision with TSR programs, illegal playback data or small

bugs in the program can easily make the system hang.  I

recommend you to back-up C: drive and to make other precautions

before trying to run this program.

Normal speed 100/200LX can be used.  However my recommendation

is to use a machine with double-speed crystal upgrade (or faster)

to eliminate high pitch noise and to enable largest polyphonic

notes, for higher quality sound.

2. Quick Start

Please prepare MIDI data in Standard MIDI File Format 0.

Most of the Standard MIDI File (SMF) data available online

are in Format 1.  You should convert it to format 0.

(Conversion utilities are listed below, in Section 5.)

From DOS prompt, enter:

LXMIDI -N1 -X SAMPLE.MID            (double speed machine)

or

LXMIDI -N1 -X -O8 -C150  SAMPLE.MID      (normal machine)

(SAMPLE.MID is assumed to be the name of your MIDI file).  Now

your 100/200LX is reborn as a palmtop MIDI sound module.

Note: Value '150' in '-C' option needs fine tuning.  See

below for detail.

Your should have at least 140 KB of conventional memory to run

LXMIDI.  If your sound data is large, you need more.  If you

use LXMIDI on DOS process shelled out from System Manager, you

need to increase memory size on Setup - System menu.  Your will

have no problems if you terminate system manager and run LXMIDI

from DOS prompt.

3. Command reference

Usage of LXMIDI is as follows:

LXMIDI [option(s)] <filename>[.MID]

<filename>

The name of the MIDI data file to play. Default

extension is '.MID'.

option -Nn

n = 0  (default)  HP100/200LX with 36 MHz crystal (so called

2.27-times speed upgrade)

n = 1  HP100LX/200LX with normal speed or double speed

crystal.   '-C' option is necessary for normal speed.

n = 2  for IBM-PC compatible.

n = 3  HP100/200LX with 32.0 MHz crystal

n = 4  HP100/200LX with 40.0 MHz crystal and 31/32 MHz driver

n = 9  for NEC PC-98 and its compatible.

Option -On

This option specifies maximum polyphonic notes. The

parameter n should be 8, 12 or 16.  Default is 12.

Larger value requires larger CPU power.  If your

crystal is normal speed, setting of '-O8' together

with '-X' option (see below) is recommended.

Option -X

This option disables LCD display while the music is

played.  Its effect is as follows.

o CPU power is increased by more than 7 %, since LCD

controller is stopped and CPU gains control of the

bus all the time.  You can specify better value to

-O and -C.

o With the same reason, PCM interrupts by CPU is more

stable, which leads to less jittering noise while

playback.

o Power consumption is decreased.

I recommend to use this option if possible, This

option does not work with -N2 or -N9.

Option -Mccc...ccc       c:{0123456789ABCDEF}

The parameter specifies MIDI channels to be muted

(masked).  For example:

-M128AC

Normal MIDI sound source (GM or GS compliant) has 16

to 64 channel multi-timbral/polyphonic capability.

Therefore some of the sound data requires voices far

exceeding 16 notes available on LXMIDI.

Even if the data requires parts exceeding this number,

LXMIDI tries to conserve the original music, but may

not success, if the number of voices are too many. In

that case, masking of some MIDI channels and reducing

polyphonic sounds may help.

Percussion sound source implemented by LXMIDI does not

have very good quality, due to the limitation of the

machine power.  If you do not like this percussion,

simply mask its part by '-M9' option.

Option -P

This option ignores the instrument given by MIDI data.

All the sounds are then made with flat envelope.

LXMIDI is a synthesizer with square-wave sound source

and cannot change wave form to recreate the type of

sound for an instrument.  Instead, LXMIDI uses the

change of envelope to recreate the sound.  (according

to instruments as assigned by GM standard)  This option

suppress this function.

Option -Vn

This option sets sound volume (0 to 100).  If it is

set to '0', LXMIDI automatically sets the volume to

optimum value.  The default setting is '0' (automatic

volume control).  Value too high will distort the

sound if MIDI data contain high-volume part.  This

setting is independent from the system volume set by

Setup menu of 100/200LX.

Option -An

This option sets sound volume under automatic volume

control (10 to 200 %).  If you use automatic volume

control, LXMIDI selects the optimum volume level, a

maximum value with no distortion.  If you want to have

larger volume and tolerate the distortion at laud

parts, then you may set this option to a value

exceeding 100.

Option -Cn

Sets cycle duration of PCM (40 to 255).  Default is 64

for-N1 and -N2 options; 95 for -N0 option.  Low value

eliminates the high-pitch noise but requires higher

CPU power.  Too low a value (which exceeds the CPU

power) will make the sound skip.

You should increase this setting, if you use normal

speed 100/200LX.  First set a large amount (around

150) and gradually decrease it.  The setting should be

a little larger than the value where the playback

starts to suffer from skipping sound.

Option -K

With this option, LXMIDI will wait for you to press

any key before it starts playing.

Option -Tn

This option set tempo (10 to 1000) in per cent unit.

Examples: -T103 sets tempo as 103% of the original

-T50  sets tempo as 50% of the original

Option -Rn

Transpose (pitch shift) value is specified (-24 to 24)

in the unit of half step (a twelfth octave).

Examples: -R1   raise by half step.

-R12  raise by an octave.

-R-12 lower by an octave.

Option -Dn

This option sets volume of percussion part relative to

other parts (in per cent unit).

Examples: -D100  default.

-D200  volume of percussion is twice as large.

-D50   volume of percussion is reduced to half.

Option -En

Reverb effect is adjusted by this option (1 to 6).

Examples: -E1   -6 dB (maximum)

-E2   -12 dB (default)

-E6   -36 dB (minimum)

Notes:

(1) The reason that -N0 is default is because -N1 will hang the

36 MHz machine.

(2) Under normal condition, temporary patches are applied to

interrupt routines specific to LX for better sound

quality.  -N2 option disables this patch so LXMIDI can

run on IBM-PC compatibles.

(3) Option -N1 also works for 32 MHz crystal. However Tempo

and pitch would be higher by approximate 1 %.  Option -N3

is recommended.

4. Key input during playback

LXMIDI first analyses the input MIDI file and creates internal

data.  During this period, pressing any key can stop the

execution.

After the analysis, the music starts (If -K option is used, LXMIDI

waits for a key before starting).  If -X option is specified, the

LCD display will be turned off.

After the music starts, [SHIFT] key will pause the music

temporarily. You can power off or on during the pause.  Press

any other key to resume the playback.

[CTRL] or [ALT] key can interrupt the playback.

5. MIDI Data playable by LXMIDI

LXMIDI supports MIDI data in SMF (Standard MIDI File) Format 0,

with a file extension of .MID.  Data in SMF format 1 can be converted

to SMF format 0, by using following utilities.

MIDI1TO0.EXE     SMF(.MID) format 1 to format 0 Conversion

MD1T0V04.LZH  NIFTY-Serve FMIDITOL forum LIB.8 #15

SMF1TO0.EXE      SMF(.MID) format 1 to format 0 Conversion

SMF10V10.LZH  NIFTY-Serve FMIDITOL forum LIB.8 #25 (SMF10V11.LZH)

File extension .STD is also used for SMF format.

Popular formats such as RCP, MCP, MM2, MMC, SEQ, or SNG can be

converted to SMF by following utilities.

CVS.EXE          .MCP/.RCP      to SMF(.MID) format 1

CVS506.LZH    NIFTY-Serve FMIDITOL forum LIB.8 #69

MM2S.EXE         .MM2/.MMC/.SEQ to SMF(.MID) format 0/1

MM2S31.LZH    NIFTY-Serve FMIDITOL forum LIB.8 #30

SNG2S.EXE        .SNG           to SMF(.MID) format 0/1

SNG2S33.LZH   NIFTY-Serve FMIDITOL forum LIB.8 #31

I would like to express my thanks to the authors of these

valuable utility programs.

( Translator's Note:

These software are all available on NIFTY-Serve, a Japanese

information service, a licensee of CompuServe, and meant

to be for Japanese users.  I suppose similar conversion

utilities can be found on MIDI forums of Compuserve.

MIDI1TO0.EXE, one of the above utilities,  can be downloaded

from Compuserve, HPHAND forum, LIB 11, as MIDI1TO0.ZIP. )

If you need, other software are also available at following

web sites:

ftp://www.vector.co.jp/common/dos/music/conv/

http://www.vector.co.jp/common/dos/music/conv/00_index.html  )

6. MIDI events supported by LXMIDI

Following is the MIDI events supported by LXMIDI, described

in MIDI Implementation Chart format.

You can skip this section, if this format in not familiar

to you.  You can use LXMIDI without the knowledge described

in this section.

FUNCTION           RECEIVE          NOTES

------------------------------------------------------------------

BASIC CHANNEL      1-16

MODE               X 3              16-POLYPHONIC 16-MULTI TIMBRE

NOTE NUMBER        5-126

VELOCITY NOTE ON   O 9n,V=1-127

NOTE OFF  X

AFTER TOUCH        X

PITCH BENDER       O                PRECISION:25CENT.(1/4 HALFTONE)

CONTROL CHANGE  1  O                PITCH MODULATION DEPTH

5  O                PORTAMENTO TIME

6  O                DATA ENTRY

7  O                VOLUME

11  O                EXPRESSION

64  O                HOLD 1 (DUMPER PEDAL)

65  O                PORTAMENTO ON/OFF

100,101 O                RPN LSB,MSB *

120 O                ALL NOTE OFF

121 O                CHANNEL RESET

122-127 O                ( = ALL NOTE OFF)

PROGRAM CHANGE     O 0-121          ENVELOPE EMULATION ONLY **

EXCLUSIVE          X

COMMON(SONG,TUNE)  X

REALTIME

(CLOCK,COMMAND) X

------------------------------------------------------------------

*  MSB=0,LSB=0 (BENDER RANGE) ONLY.

** PROGRAM NUMBER IS BASED ON GENERAL MIDI STANDARD.

CH.10:SUPPORT GM STANDARD PERCUSSION SET ONLY.

7. Revision History

Ver 1.10

o First Release

Ver 1.20 (not released)

o Percussion set implemented.

o Resolving process for polyphonic sound was improved for

higher fidelity.

o Reverb effect introduced.

o Playback routine optimized, with 15% gain in speed.

o Default extension of '.MID' is applied.

o Removal of a lot of small bugs.

Ver 1.21

o Removed a bug that expression and pitch bender/portamento

events cannot run simultaneously.

o '-T' and '-R' options added.

Ver 1.22

o Percussion data set was tuned and improved (I hope).

o Resolving process for exceeding polyphonic sound are

modified for (probably) better reproduction of the

original.

o '-D' and '-E' options added.

o Several bugs removed.

Ver 1.23 (not released)

o Resolving process for exceeding polyphonic sound are

improved.

o Removed a bug that running status in not adequately

handled after the execution of a meta event.

Ver 1.30

o Removed a bug that caused hangup after the completion of

playback.  (I would like to thank to Chikara and all the

other people who reported or commented on this program.)

o Added automatic volume control, which set the volume to

the maximum without distortion of the sound.

o [Shift] key pauses the playback.

o PITCH MODULATOR implemented.  Vibrato is enabled.

o Removed plural bugs.

Ver 1.40

o Expanded polyphonic notes to max 16.

o Added '-X' option, which disables LCD display while

playback for increased CPU power and better sound quality.

o Optimization of playback routines.  Introduction of self

modification method for higher speed (increase by 18%).

12- or 16-note polyphony with no '-X' option is now

practical on machines with double-speed crystal or

higher.  Smaller value in -C option is enabled on normal

speed machine with 8-note polyphony.

o Changed algorithm for automatic volume control.

o Now runs on NEC PC-98.

o Volume balance is adjusted on each timbre.

o Removed a bug by which LXMIDI did not work on IBM-PC.

o Added '-A' option.

o Compiling model is changed to tiny model (.COM format)

to reduce the memory necessary for execution by 30 KB.

Ver 1.41

o Removed a bug that the date is not renewed if a new date

arrives during playback.

o Removed a bug that the time is not correctly adjusted when

playback is done at 19 to 20 o'clock.

7. Copyright and disclaimers

This program is a free software.  Its copyright is owned by its

author, HARUYA <pxw03430@niftyserve.or.jp>.

Distribution is permitted, if it is not for commercial

purposes.  When you repost it to the other network, please

notify the author (prior notification is not necessary).  If

you repost this software, you are obliged to update it, when a

new version of this software is released.

Prior notification is necessary, it this software are to be

bundled to publication - a book or a periodical.

No part of the attached source code shall be used for other

software without permission from the author.   Distribution

of this software which are modified from the original is

not permitted unless allowed by the author.

The author will be very happy, if he is informed of any bugs

of this software.  He is more happy, if the cause of the bugs

is also reported.

The author has no responsibility for any damages, which may

be caused by use of, or inability to use, this program.  Use it

at your own risk.

1996.09.21

HARUYA

pxw03430@niftyserve.or.jp

[  Translated from Japanese by:  ]

[  Toshiki Sasabe (Janbe)        ]

[  73560.524@compuserve.com      ]