SoundFont 2.0 Example Document

Version 1.1, MG (mikeg@emu.com)

Use with sf2exam.sf2 version 1.1.

See Bank name for version information

BASIC.SF2

This SoundFont bank is designed to test individual parameters and basic functions of a SoundFont bank.BASIC.SF2 contains all required sample data. The bank is split up into a number of PRESETS each lying under MIDI Bank index 0. Each PRESET is designed to test individual parameters or groups of parameters. The presets get more complex as their indices increase.

This SoundFont bank does not use Layers for any reason other than to point at instruments. Individual samples are shared between the presets.

Preset index 0: LFO to Filter

Here is a rough diagram of the preset:

This is the simplest form of a SoundFont Object or Preset. The Preset points to a single instrument, which in turn points to a single sample. The instrument has a few generator parameters and points to a single sample.

When the Soundfont compatible driver navigates this preset with a keynumber of 60 and a velocity of 100, the output will be a single vector with the following parameters:

--------- Vector # 0 --------------

dwStart                  4888		samples

dwEnd                    7025		samples

dwStartLoop              6883		samples

dwEndloop                7017		samples

dwSampleRate             44100	Hz

OriginalKey              64

Correction               60		cents

shModLfoToPitch          0		cents/max excursion

shVibLfoToPitch          0		cents/max excursion

shModEnvToPitch          0		cents/max excursion

shInitialFilterFc	 8246	cents above 8.176 Hz

shInitialFilterQ         180		cB above DC level

shModLfoToFilterFc   -1200	cents/max excursion

shModEnvToFilterFc       0		cents/max excursion

shModLfoToVolume         0		cB/max excursion

shChorusEffectsSend      0		units of 0.1%

shReverbEffectsSend      0		units of 0.1%

shPanEffectsSend         0		units of 0.1% full right

shDelayModLfo            -12000	TimeCents above 1 second

shFreqModLfo         -1908	cents above 8.176 Hz

shDelayVibLfo            -12000	TimeCents above 1 second

shFreqVibLfo             -725		cents above 8.176 Hz

shDelayModEnv            -12000	TimeCents above 1 second

shAttackModEnv           -12000	TimeCents above 1 second

shHoldModEnv             -12000	TimeCents above 1 second

shDecayModEnv            768		TimeCents above 1 second

shSustainModEnv          0		cB 	Attenuation

shReleaseModEnv          -12000	TimeCents above 1 second

shAutoHoldModEnv         0		TimeCents / KeyNum

shAutoDecayModEnv        0		TimeCents / KeyNum

shDelayVolEnv            -12000	TimeCents above 1 second

shAttackVolEnv           -12000	TimeCents above 1 second

shHoldVolEnv             -12000	TimeCents above 1 second

shDecayVolEnv            -12000	TimeCents above 1 second

shSustainVolEnv          0		cB Attenuation

shReleaseVolEnv          -12000	TimeCents above 1 second

shAutoHoldVolEnv         0		TimeCents / KeyNum

shAutoDecayVolEnv        0		TimeCents / KeyNum

shKeynum                 60

shVelocity               100

shInstVol                82		cB of Attenuation

shCoarseTune             0		Semitones

shFineTune               7		cents

shSampleModes            1

shKeyExclusiveClass      0

This bolded elements of this vector are the emphesis of this preset. The values indicate that the sample should be played with an LFO modulating Filter Cutoff. The frequency of the LFO should be about 3 Hz. At note-on, the Filter Cutoff should be about 1 kHz, then DECREASE down one octave below the initial filter cutoff amount (to 500 Hz), then increase up 1 octave ABOVE the initial filter cutoff amount (to 2 kHz).

Note from the italicized shSampleModes  parameter listing that the sample should be played from the start point into the loop points then stay in the loop throughout the release portion. The samples from the loop end to the sample end are never heard.

Also note that pitch calculation occurs with a combination of the following fields:

dwSampleRate

shOrigKeyAndCorr

shCoarseTune

shFineTune

shScaleTune

Preset Index 1: Envelope to Filter

This preset has the same form as Preset Index 0.

When the Soundfont compatible driver navigates this preset with a keynumber of 60 and a velocity of 100, the output will be a single vector with the following parameters:

--------- Vector # 0 --------------

dwStart                  4888		samples

dwEnd                    7025		samples

dwStartLoop              6883		samples

dwEndloop                7017		samples

dwSampleRate             44100	Hz

OriginalKey              64

Correction               60		cents

shModLfoToPitch          0		cents/max excursion

shVibLfoToPitch          0		cents/max excursion

shModEnvToPitch          0		cents/max excursion

shInitialFilterFc     5192	cents above 8.176 Hz

shInitialFilterQ         126		cB above DC level

shModLfoToFilterFc       0		cents/max excursion

shModEnvToFilterFc    3600 	cents/max excursion

shModLfoToVolume         0		cB/max excursion

shChorusEffectsSend      0		units of 0.1%

shReverbEffectsSend      0		units of 0.1%

shPanEffectsSend         0		units of 0.1% full right

shDelayModLfo            -12000	TimeCents above 1 second

shFreqModLfo             -1925	cents above 8.176 Hz

shDelayVibLfo            -12000	TimeCents above 1 second

shFreqVibLfo             -725		cents above 8.176 Hz

shDelayModEnv            -12000	TimeCents above 1 second

shAttackModEnv        -4021	TimeCents above 1 second

shHoldModEnv          -12000	TimeCents above 1 second

shDecayModEnv          1094	TimeCents above 1 second

shSustainModEnv        1000	cB Attenuation

shReleaseModEnv          -12000	TimeCents above 1 second

shAutoHoldModEnv         0		TimeCents / KeyNum

shAutoDecayModEnv        0		TimeCents / KeyNum

shDelayVolEnv            -12000	TimeCents above 1 second

shAttackVolEnv           -12000	TimeCents above 1 second

shHoldVolEnv             -12000	TimeCents above 1 second

shDecayVolEnv            -12000	TimeCents above 1 second

shSustainVolEnv          0		cB Attenuation

shReleaseVolEnv          -12000	TimeCents above 1 second

shAutoHoldVolEnv         0		TimeCents / KeyNum

shAutoDecayVolEnv        0		TimeCents / KeyNum

shKeynum                 60

shVelocity               100

shInstVol                82		cB of Attenuation

shCoarseTune             0		Semitones

shFineTune               7		cents

shSampleModes            1

shKeyExclusiveClass      0

The vector looks very much like Preset Index 0, except the parameter emphasized here is Modulation Envelope to Filter Cutoff. That Envelope should have a 0.1 second attack, a minimal hold (-12000 TimeCents), and a 2 second decay to a fully attenuated sustain level. The Modulation Envelope to Filter Cutoff amount is +3600 cents, the Initial Filter Cutoff is around 171 Hz. This means the filter cutoff should start at 171 Hz, increase to about 520 Hz, then back down to 171 Hz.

Also note from the Sample Loop Points that it uses the SAME SAMPLE as Preset Index 0. The bank itself uses a DIFFERENT INSTRUMENT pointing to that sample. This is what allows the difference in articulation data you see above.

Looking at Preset Index 0 AND Preset Index 1:

Note that other instruments used by other presets also point to this sample.

Preset Index 2: Volume and Filter Envelope

The output vector is similar to Preset Index 2. In this preset, both the Volume and Modulation Envelopes affect the sound. The Volume Envelope has an increased attack time. So you should get the same sounds as Preset Index 1 except the volume of the sound should gradually  increase to the Initial Attenuation level.

Preset Index 3: Envelope to Pitch

Similar to Preset Index 3. In this preset, the Modulation Envelope to Pitch is emphasized. That Envelope has an instantaneous attack, a 1/2 second hold time, and a 1 second decay time. The Modulation Amount is +1200 cents/full excursion. The note should play immediately at one octave higher than the keynumber and sample data indicates, hold that pitch for 1/2 second, then gradually decay to the indicated pitch.

Preset Index 4: Modulation Lfo to Pitch

Similar again. This time the Modulation LFO is set to a frequency of 1 Hz with a pitch modulation amount of +1200 cents/full excursion. The pitch should start at the original pitch then INCREASE to one octave higher than the original pitch, then back down to one octave lower than the original pitch.

Preset Index 5: VibLfo to Pitch ( fast Lfo)

Similar to Preset Index 4, except the Vibrato LFO is used, the frequency is higher, and the amount is -1200 thus the LFO causes the pitch to first decrease then increase.

Preset Index 6: Warm Synth String Pad

With this preset, we are no longer using the SawBassC3 sample. The preset has the same form as the above presets. The generators produced indicate a 7% reverb amount, and an active Filter AND Volume Envelope Modulation. The Filter modulation happens using the Modulation Envelope. That envelope has an instantaneous attack, full sustain level, and a 28 second release time. That envelope modulates the filter by about 3800 cents/full excursion. The Volume Modulation happens using the Volume Envelope. That envelope has a ~0.8 second attack, full sustain level, and a 2.2 second release time.

Preset Index 7: Warm Synth String Pad with Chorus

This is the same as Preset Index 7 with an additional instrument parameter to give the sound a 100% chorus amount.

Preset Index 8: Warm Synth String Pad ( with Envelope to pitch and filter)

This is similar to Preset Index 6, except the Modulation Envelope is used to modulate both pitch AND filter. The pitch amount is negative, whereas the filter modulation amount is positive.

Preset Index 9: Synth Drum

This preset introduces the concept of keyboard SPLITS. The Preset has the following form:

Note the generator list in the instrument now contains more than one key range. The split generators indicate the differences. The First key split has a Volume Envelope attack time of about 1/4 second. The Second key split that parameter at 0.025 seconds. The third has an instantaneous attack time.

Preset Index 10: 5th Saw Wave

This preset has the following form:

This bank also uses multiple key splits. However some of those splits OVERLAP other splits. This is accomplished with the existence of TWO key splits with the SAME range. This preset also takes advantage of using DIFFERENT samples BETWEEN some of the key splits.

In this preset, each key range resolves into the same sample twice, with one of those key ranges adding a +5 semitone coarse tuning parameter, thus the 5th chorded sawtooth wave sound.

Preset Index 11: Test Scale Tuning

In this preset, the instrument has multiple key splits pointing to the same sample. The scale tuning parameter is set differently in each of the key splits.

From key 12 to 47, the scale tuning parameter is 0. This means that the pitch value by 0 semitones from the original sample pitch as you increase the keynumber. Every key sounds at the original sample pitch.

From key 48 to 71, the scale tuning parameter is 50. This means that the pitch value increases by 50 semitones from the original sample pitch as you increase the keynumber. This gives a quarter toned key range.

From key 72 to 83, the scale tuning parameter is 100, thus a half toned key range. This is the default and typical value for scale tuning.

From key 84 to 127, the scale tuning parameter is 200, thus a full toned key range. Each keynumber increases the pitch by a full tone instead of a semitone.

As a final note, scale tuning is based on deviations from the ORIGINAL sample pitch AT the ORIGINAL key. An instrument which uses multiple samples layed out across the keyboard to work as a half toned instrument WILL NOT BECOME a quater toned piano by simply changing all of the scale tunings to 50!

Preset Index 12: Global Tremelo

This preset introduces the concept of the GLOBAL SPLIT. A global split of generators contains generators which apply to ALL splits UNLESS they are RE-DEFINED in the individual split.

The bank has this format:

The fact that a group of generators appears BEFORE the first keyrange tells you that those generators in the global split. In this bank, there are two generators in the global split, one indicating a Modulation LFO frequency of 1 Hz, the other indicating a Modulation LFO to Volume amount of +120 cB/max excursion. However the second split has a Modulation LFO frequency of 2 Hz. That is why they are moving at different frequencies.

The advantage of Global Splits is not demonstrated here, since only a limited number of generators are used. Global Splits are most beneficial when a preset contains many splits which all would require the same articulation data. That articulation data would appear only once within the instrument, as opposed to once per split.

Preset Index 13: Velocity Splits

Velocity can also specify a split in SoundFont bank. This preset has the following form:

In this bank, a velocity value of 0 through 69 triggers the Saw Bass Wave. A velocity value of 90 through 127 triggers the String Sample wave. A velocity value of 70 through 89 triggers both samples. Each split has its own set of additional generators.

LAYERED.SF2

This SoundFont bank is designed to test the layering functions of a SoundFont bank.LAYERED.SF2 contains all of the samples necessary to play back correctly. The bank is split up into a number of PRESETS each lying under MIDI Bank index 0. Each PRESET is designed to test individual parameters or groups of parameters. Basically, the presets get more complicated as their indices increase.

The structure for all the banks in BASIC.SF2 was the one used with SoundFont version 1, where preset level of the SoundFont bank was not used for any other purpose than to point to an instrument. All presets in that bank (with the exception of Preset Indices 11, 12, and 13) was created with Creative Technologies Vienna SoundFont Studio then converted to SoundFont 2.0 using Creative Technologies SoundFont 1.0 to 2.0 Converter.

When a SoundFont bank created with Vienna is converted into SoundFont 2.0 format, that bank is converted from a collection of Presets into a collection of Instruments. Then with a SoundFont 2.0 Editor such as E-mu Systems Preditor using preset level editing, the developer may quickly and easily create variations or combinations of those instruments in their entirety.

Preset Index 0: Organ 1

This demonstrates the simplest use of layering. This preset shows the notion of multi-layereing a preset occuring at the SoundFont Preset level rather than at the SoundFont instrument level. Its format is:

Two different organs are layered to give a more full organ effect. No Layer generators are used for tweaking of the data.

Preset Index 1: Bass/ Piano

This preset uses the same concept as Preset Index 0, except there are keyrange fields at the Preset level. With this preset, you have a fretless bass instrument from keynumbers 0 to 47 and you have a piano instrument from keynumbers 48 to 127.

Preset Index 2: Major 7th Chord

BASIC.SF2 contains a "5th saw wave" preset created by duplicating all of the splits and adding additional coarse tune generators to each of the copies. A Piano instrument typically contains many key splits (sometimes as many as  30 splits).  To create a chorded piano using the techniques demonstrated above, all of those splits would need to be duplicated and tweaked. Using that technique, creating a Major 7th chorded piano (where each key plays 4 samples) within an instrument would be a nightmare.

Here we demonstrate a much easier way of accomplishing the same task using Layering.

The only generators at the Preset level are coarse tune adjustments, one for +4 semitones, another for +7 semitones, and a third for +11 semitones. To turn this into a minor seventh, change the coarse tuning parameter in the layer with +4 semitone to +3 semitones.

In this preset, a single keynumber and velocity results in 4 output SoundFont vectors.

Preset Index 3: Velocity/Key Layers

This preset uses both velocity and keynumber splits to define layers. It also has layer generators to add -36 semitones to the pitch of the Organ1 instrument.

Preset Index 4: Layered Violins

This preset introduces the concept of the Global Layer. A Global Layer of generators contains generators which apply to ALL layers UNLESS they are RE-DEFINED in the individual layer. Those layer parameters are then added to the instrument generators in the final output vector.

In this preset, the Global Layer contains a single generator to add 10% reverb to all layers. There are then 3 layers of the 'strings' instrument. Those layers have benign parameters with the exception of the attack time parameter. 1000% (12000 TimeCents) attack time is added to the first layer, defined at keynumbers 0 to 39. 500% (6000 TimeCents) attack time is added to the second layer, defined at keynumbers 40 to 79. Finally, -200% (-2400 TimeCents) attack time is added to the third layer, defined at keynumbers 80 to 127.

Preset Index 5: Layered Mixed Bag

This preset is a mixed bag of some of the various layering functions demonstrated with the previous presets. Here we have a preset with three layers, each pointing to a single instrument. That instrument has two splits, each pointing to a single sample.

This preset can be viewed as follows:

STEREO.SF2

This SoundFont bank is designed to test the SoundFont stereo implementation,  STEREO.SF2 contains all the samples necessary to play the sounds. The bank is split into a number of PRESETS within MIDI Bank index 0. Each PRESET is designed to test the stereo implementation of a SoundFont compatible wavetable synthesizer.

The instruments in this bank were constructed using 4 samples, namely:

s_16_44 [R]

s_16_44 [L]

s_8_22 [R]

s_8_22 [L]

The presets in this bank were constructed using 6 instruments, namely:

Stereo CarStart

Mono CarStart

Other Mono CarStart

Stereo Noise

Mono Noise

CarStart and Noise

Preset Index 0: Stereo 1

This preset uses the sample 16 bit 44 kHz stereo waveform which shipped with the AWE-32 (s_16_44.wav). That WAV file sounds like 2 cars starting. This preset has the following form:

The samples pointing at each other indicate two distinct facts: 1. The sample in question claims it is a Right sample, a Left sample, or a Linked sample, (in the sfSampleTypes field of the Sample Header data structure) and 2. The sample they point (in the sfSampleTypes field of the Sample Header data structure) to is contained within the closure of the splits.

Navigating this preset with keynumber 60 and velocity 100 produces the following two output vectors:

--------- Vector # 0 --------------

dwStart                  0		samples

dwEnd                    137025	samples

dwStartLoop              91350	samples

dwEndloop                137018	samples

dwSampleRate             44100	Hz

OriginalKey              0

Correction               60		cents

shModLfoToPitch          0		cents/max excursion

shVibLfoToPitch          0		cents/max excursion

shModEnvToPitch          0		cents/max excursion

shInitialFilterFc        13500	cents above 8.176 Hz

shInitialFilterQ         0		cB above DC level

shModLfoToFilterFc       0		cents/max excursion

shModEnvToFilterFc       0		cents/max excursion

shModLfoToVolume         0		cB/max excursion

shChorusEffectsSend      0		units of 0.1%

shReverbEffectsSend      0		units of 0.1%

shPanEffectsSend         -500		units of 0.1% full right

shDelayModLfo            -12000	TimeCents above 1 second

shFreqModLfo             0		cents above 8.176 Hz

shDelayVibLfo            -12000	TimeCents above 1 second

shFreqVibLfo             0		cents above 8.176 Hz

shDelayModEnv            -12000	TimeCents above 1 second

shAttackModEnv           -12000	TimeCents above 1 second

shHoldModEnv             -12000	TimeCents above 1 second

shDecayModEnv            -12000	TimeCents above 1 second

shSustainModEnv          0		cB Attenuation

shReleaseModEnv          -12000	TimeCents above 1 second

shAutoHoldModEnv         0		TimeCents / KeyNum

shAutoDecayModEnv        0		TimeCents / KeyNum

shDelayVolEnv            -12000	TimeCents above 1 second

shAttackVolEnv           -12000	TimeCents above 1 second

shHoldVolEnv             4000		TimeCents above 1 second

shDecayVolEnv            -12000	TimeCents above 1 second

shSustainVolEnv          0		cB Attenuation

shReleaseVolEnv          -12000	TimeCents above 1 second

shAutoHoldVolEnv         0		TimeCents / KeyNum

shAutoDecayVolEnv        0		TimeCents / KeyNum

shKeynum                 60

shVelocity               100

shInstVol                0		cB of Attenuation

shCoarseTune             0		Semitones

shFineTune               0		cents

shSampleModes         12

shSampleLink          1

shKeyExclusiveClass      0

--------- Vector # 1 --------------

dwStart                  137071	samples

dwEnd                    274096	samples

dwStartLoop              228421	samples

dwEndloop                274089	samples

dwSampleRate             44100	Hz

OriginalKey              0

Correction               60		cents

shModLfoToPitch          0		cents/max excursion

shVibLfoToPitch          0		cents/max excursion

shModEnvToPitch          0		cents/max excursion

shInitialFilterFc        13500	cents above 8.176 Hz

shInitialFilterQ         0		cB above DC level

shModLfoToFilterFc       0		cents/max excursion

shModEnvToFilterFc       0		cents/max excursion

shModLfoToVolume         0		cB/max excursion

shChorusEffectsSend      0		units of 0.1%

shReverbEffectsSend      0		units of 0.1%

shPanEffectsSend         500		units of 0.1% full right

shDelayModLfo            -12000	TimeCents above 1 second

shFreqModLfo             0		cents above 8.176 Hz

shDelayVibLfo            -12000	TimeCents above 1 second

shFreqVibLfo             0		cents above 8.176 Hz

shDelayModEnv            -12000	TimeCents above 1 second

shAttackModEnv           -12000	TimeCents above 1 second

shHoldModEnv             -12000	TimeCents above 1 second

shDecayModEnv            -12000	TimeCents above 1 second

shSustainModEnv          0		cB Attenuation

shReleaseModEnv          -12000	TimeCents above 1 second

shAutoHoldModEnv         0		TimeCents / KeyNum

shAutoDecayModEnv        0		TimeCents / KeyNum

shDelayVolEnv            -12000	TimeCents above 1 second

shAttackVolEnv           -12000	TimeCents above 1 second

shHoldVolEnv             4000		TimeCents above 1 second

shDecayVolEnv            -12000	TimeCents above 1 second

shSustainVolEnv          0		cB Attenuation

shReleaseVolEnv          -12000	TimeCents above 1 second

shAutoHoldVolEnv         0		TimeCents / KeyNum

shAutoDecayVolEnv        0		TimeCents / KeyNum

shKeynum                 60

shVelocity               100

shInstVol                0		cB of Attenuation

shCoarseTune             0		Semitones

shFineTune               0		cents

shSampleModes         4

shSampleLink          0

shKeyExclusiveClass      0

Note the vector has its own way of representing sample links within a vector group. The sample link status is contained in the shSampleModes field and the vector link indices in the shSampleLinks field. SampleModes with the 4th  (FIRST_LINK) bit set says the vector is the head of a group of linked samples. Sample Modes with the 3rd (LINKED) bit set says the vector contains a linked sample. The shSampleLinks field tells the index of the  VECTOR the current vector is linked to.

These samples should be played with the pitch phase locked and the RIGHT sample (the vector with the 4th or FIRST_LINK bit set) as the pitch master. The pan positions are contained within the generator splits (TYPICALLY left panned -500, right panned +500) and SHOULD NOT BE ASSUMED.

Preset Index 1: Stereo 2

This preset has the same archetecture as Preset Index 0, except a different stereo pair of samples is used. It uses samples originally contained in the s_8_22.wav file which shipped with the AWE-32. That WAV file sounds like an arcade game noise. The corresponding samples within this SoundFont bank are now 16 bit samples with the bottom bytes of each sample set to 0.

Preset Index 2: Not Stereo 1

This preset has the following form:

Note that this preset should NOT be treated as a stereo preset because the LINKED samples were not found in the individual Instrument Closures. This is a pair of INSTRUMENTS each with ONE MONO Sample, where the samples happen to be LINKED to each other within a DIFFERENT INSTRUMENT than is used here.

Preset Index 3: Not Stereo 2

This preset has the following form:

The samples pointing into outer space indicate two distinct facts: 1. The sample in question claims it is a Right sample, a Left sample, or a Linked sample, and 2. The sample they point to is NOT contained within the closure of the splits.

Even though there is a left and a right sample, they are not each other's LINKS. This preset should not be played back with the samples pitch phase locked. In other words, this preset plays two independent mono samples which happen to be elements of two different stereo pairs.

Preset Index 4: Not Stereo 3

This preset has the following form:

Because only ONE of the sample pair was in the instrument closure, this sample was intended to be played as a MONO sample, not as a stereo sample with its partner.

Preset Index 4: Stereo -n- Mono

This preset has the following form:

Note that this preset layers the s_16_44 stereo instruments with the NotStereo2 instrument used above. This should be played with the stereo samples pitch phase locked and the third sample played as a mono sample.

Preset Index 5: Stereo -n- Mono 2

This preset is similar to Preset Index 4, except the mono instrument chosen was the Mono CarStart, not the Mono Noise. Since the stereo instrument is the Stereo CarStart,  the result of the initial navigation would tell you that there are TWO samples in the Preset Closure which are pointing to the Left Sample. Only one of them may do so in a stereo pair, the second should be treated as a mono sample.

Preset Index 6: Stereo Vel Splits

This preset takes adatage of the unique qualities associated with pitch control of stereo samples and careful split determination:

Note that there are two NON-OVERLAPPING splits pointing to the RIGHT sample. NON-OVERLAPPING means that for all possible key and velocity values only one of those SPLITS will be active.

The first split pointing to the RIGHT sample has a coarse tuning of -12 semitones, a Modulation Envelope to Pitch amount of 1200 cents/max excursion and a Modulation Envelope attack time of 5 seconds. The effect is if you hit the key soft, the ENTIRE STEREO SAMPLE (including the LEFT sample) gradually increases in pitch.

The second split pointing to the RIGHT sample has a coarse tuning of +12 semitones, a Vibrato LFO to Pitch amount of -1200 cents/max excursion and a Vibrato LFO frequency of 0.5 Hz. The effect is if you hit the key hard, the ENTIRE STEREO SAMPLE gradually decreases in pitch then increases.

Those two splits are distinguished by velocity range. So effectively, you have velocity to stereo sample pitch control.

One may also note that it is quite easy to change the splits from being NON-OVERLAPPING to being OVERLAPPING by increasing the first split's maximum velocity without increasing the second split's minimum velocity. In this case, they have created a sound which is NOT defined according to the SoundFont 2.0 specification in the overlapping split range. This should be avoided.