By Don R. Booher

Dept X39  Glendale Lab

Endicott, NY

02/27/88

*   With minor modifications you could install a larger       *

*   static ram chip or multiple chips and use it as extended  *

*   RAM without the fixed disk installed. By installing a     *

*   dip switch on the address lines you could map the memory  *

*   into any of the unused memory segments. If you are going  *

*   to use this for cartridge development all addresses above *

*   CFFFF use a two byte CRC instead of a one byte checksum.  *

** NOTE: BE SURE TO READ UPDATE AT END OF THIS FILE BEFORE PROCEEDING **

My installation was using a JR/PC bus extension that allowed 2 PC slots.

I extended the bus via 60 pin plug soldered to a ribbon cable and the

other end soldered to two 62 pin pc slots. The simulator was wired on

a Jameco prototype card and installed in one of the PC slots. The other

slot has the Western Digital controller with the ROM  removed.

- The setup looks like this -

___________          ____  Crumley'ed 640k

/            /i       /      memory sidecar

/            / i      /

/            /  i     /    ____ printer sidecar

_______________/            /   i__ _ _   /     with external power

/               /___________ /   /   / / /i/      and TOD clock

/   Crumley'ed  i C drive    i   /   / / / i

/    three drive i____________i  /   / / /  i

/     change      i B drive    i /   / / /   i       ____

/                  i____________i/   / / /  ;--------/ / /i

/____________________________________/_/_/  ;-ribbon / / / /

i                   i A drive     i  i i i ;-cable  / / / /

i  iiiiiiii  :speed i_____________i  i i i;--------/_/_/ /

i  iiiiiiii  o sw   [[[[[[   [[[[[[  i i i /       i_i_i/

i____________________________________i_i_i/      2-62 pin connectors

4.77/7.33 mH                       installed in external

driving a V20                      20 meg drive enclosure

with external power for

the disk drive

I have run bench marks on this setup and the PCMON overall index

is 1.70 of the PC1. With the V20 HFBIOS the drive access is

faster then the XT286.

-----------------------------------------------------------------------------

Wiring diagram for ROM simulator

8-K

(  With Battery backup  )

*** Parts List ***

IC1 74LS04  Hex Inverter

IC2 74LS30  Single 8-input NAND Gate

IC3 6264LP-15 CMOS

D1 and D2 1N4148 Silicon Diodes

R1  27K  resistor  1/4 watt

R2  33K  resistor  1/4 watt

C1  .1 MFD capacitor ( non-electrolytic )

B1   3.2V lithium battery ( I used two C cells as I was snowed in )

SW1 SPDT mini-switch

- ADDRESS DECODE CIRCUIT -

+5V

IC1 LS04   Vcc�������14 � IC2 LS30

__ __ __ __ __ |__          �

|                14 |         �

1 ������Ŀ

A19����|�������������������|��������Ĵ      �

2 �      �

A18����|�������������������|��������Ĵ      �

|\                    3 �      �

A17����|���1 |�o 2���������|��������Ĵ      �                chip select

|/    |\              4 �      � 8              C000 for 8K

A16����|���������3 |�o 4���|��������Ĵ      ����������Ŀ

|\    |/              5 �      �          �

A15����|���5 |�o 6���������|��������Ĵ      �          to pin 20 of IC3

|/    |\              6 �      �

A14����|���������9 |�o 8���|��������Ĵ      �

|\    |/              11�      �

A13����|���11|�o10���������|��������Ĵ      �

|/                      ��������

| __ __ __ __ __ _7_|       7 �

|����������Ĵ

gnd          �

- ENABLE/DISABLE WRITE CIRCUIT -

SW1

enable write

-MEMW ������������o

����o����������������> to pin 27 of IC3

���o

�    disable write

Vcc

+5V

- BATTERY BACKUP CIRCUIT -

Vcc +5V

^

�

�                ���������������> to pin 26 of IC3

�                �

�        R2      �      R1

o������\/\/\/\���o����\/\/\/\��Ŀ

�                               �

�                               �

�                              gnd

�

�  D1   �

�������>�����Ŀ

�     �

�

B1 - � � +        D2   �     �

��������ĳ����������������>������o�������> to pin 28 of IC3

�        ���� 3.2V         �     �

�        � �                     �

�                                �

�                              �����  C1

�                              �����

�                                �

o���������������������������������

�

�

�

�

gnd

6264LP-15  8192-word x 8bit CMOS RAM pin compatable with 2764 EPROM

(Chip can be used as a pro-printer buffer)

�������������������������Ŀ

��Ĵ          �   �          ��Ŀ

NC   �1 �          �����          �28�  Vcc * to backup circuit

��Ĵ                         ����

��Ĵ                         ��Ŀ  ��

A12  �2 �                         �27�  WE     * to SW1

��Ĵ                         ����

��Ĵ          IC3            ��Ŀ

A7   �3 �                         �26�  CS2  * to backup circuit

��Ĵ                         ����

��Ĵ                         ��Ŀ

A6   �4 �                         �25�  A8

��Ĵ                         ����

��Ĵ                         ��Ŀ

A5   �5 �                         �24�  A9

��Ĵ                         ����

��Ĵ                         ��Ŀ

A4   �6 �                         �23�  A11

��Ĵ                         ����

��Ĵ                         ��Ŀ  ��

A3   �7 �           T             �22�  OE     * -MEMR

��Ĵ           O             ����

��Ĵ           P             ��Ŀ

A2   �8 �                         �21�  A10

��Ĵ                         ����

��Ĵ                         ��Ŀ  ���

A1   �9 �                         �20�  CS1    * pin 8 of LS30

��Ĵ                         ����

��Ĵ                         ��Ŀ

A0   �10�                         �19�  I/O 8  * D7

��Ĵ                         ����

��Ĵ                         ��Ŀ

* D0    I/O 1 �11�                         �18�  I/O 7  * D6

��Ĵ                         ����

��Ĵ                         ��Ŀ

* D1    I/O 2 �12�                         �17�  I/O 6  * D5

��Ĵ                         ����

��Ĵ                         ��Ŀ

* D2    I/O 3 �13�                         �16�  I/O 5  * D4

��Ĵ                         ����

��Ĵ                         ��Ŀ

GND   �14�                         �15�  I/O 4  * D3

��Ĵ                         ����

���������������������������             Standby .01mW

TRUTH TABLE                    Operating 200mW

���������������������������������������������������������������������������Ŀ

� �� � �� �    � �� �                         �             �               �

� WE �CS1 �CS2 � OE �     MODE                � I/O PIN     � Vcc Current   �

���������������������������������������������������������������������������Ĵ

� x  � H  � x  � x  �  NOT SELECTED           �   High Z    � Isb,Isb1      �

�������������������Ĵ                         �����������������������������Ĵ

� x  � x  � L  � x  � (POWER DOWN)            �   High Z    � Isb,Isb2      �

���������������������������������������������������������������������������Ĵ

� H  � L  � H  � H  �  OUTPUT DISABLED        �   High Z    � Icc,Icc1      �

���������������������������������������������������������������������������Ĵ

� H  � L  � H  � L  �  READ                   �   Dout      � Icc,Icc1      �

���������������������������������������������������������������������������Ĵ

� L  � L  � H  � H  �  WRITE  CYCLE(1)        �   Din       � Icc,Icc1      �

���������������������������������������������������������������������������Ĵ

� L  � L  � H  � L  �  WRITE  CYCLE(2)        �   Din       � Icc,Icc1      �

�����������������������������������������������������������������������������

Wire into PCJR bus as noted -  Keep wires as short as possible

JRBUS           SIGNAL       PCBUS      SIMULATOR

B06  ---------  A0  --------- A31 ------ 10  IC3

A07  ---------  A1  --------- A30 ------ 09  IC3

BO7  ---------  A2  --------- A29 ------ 08  IC3

A08  ---------  A3  --------- A28 ------ 07  IC3

A09  ---------  A4  --------- A27 ------ 06  IC3

B09  ---------  A5  --------- A26 ------ 05  IC3

B10  ---------  A6  --------- A25 ------ 04  IC3

A11  ---------  A7  --------- A24 ------ 03  IC3

B11  ---------  A8  --------- A23 ------ 25  IC3

A12  ---------  A9  --------- A22 ------ 24  IC3

A13  ---------  A10 --------- A21 ------ 21  IC3

B13  ---------  A11 --------- A20 ------ 23  IC3

B14  ---------  A12 --------- A19 ------ 02  IC3

A15  ---------  A13 --------- A18 ------ 11  IC1

A16  ---------  A14 --------- A17 ------ 09  IC1

B16  ---------  A15 --------- A16 ------ 05  IC1

A17  ---------  A16 --------- A15 ------ 03  IC1

B18  ---------  A17 --------- A14 ------ 01  IC1

A19  ---------  A18 --------- A13 ------ 02  IC2

B19  ---------  A19 --------- A12 ------ 01  IC2

A01  ---------  D0  --------- A09 ------ 11  IC3

B01  ---------  D1  --------- A08 ------ 12  IC3

B02  ---------  D2  --------- A07 ------ 13  IC3

A03  ---------  D3  --------- A06 ------ 15  IC3

B03  ---------  D4  --------- A05 ------ 16  IC3

A04  ---------  D5  --------- A04 ------ 17  IC3

A05  ---------  D6  --------- A03 ------ 18  IC3

B05  ---------  D7  --------- A02 ------ 19  IC3

B21  ---------  -MEMR ------- B12 ------ 22  IC3

B22  ---------  -MEMW ------- B11 ------ SW1

A10  ---------  GND --------- B01 ------ GND

A18  ---------  GND --------- B10 ------ GND

A22  ---------  GND --------- B31 ------ GND

A06  ---------  +5V --------- B03 ------ +5V

A26  ---------  +5V --------- B29 ------ +5V

TOP

jr bus pins

A1 .............................. A30

B1 .............................. B30

OPERATING INSTRUCTIONS

1.Switch the ROM simulator to write enable

2.Using debug load the HFBIOS of your choice(see sample below)

3.Switch the Rom simulator to write disable and BOOT from harddisk

- SAMPLE -

debug a:hfv2002.bin

-R DS

DS 0F04

:C000     ** set up register DS for segment C000

** enable write on simulator

-M CS:100 2100 0         *** move the code to c000:0 for a length of x'2000'

-D C000:0                *** verify that the move took place

C000:0000  55 AA 06 E9 D1 00 E9 FE-09 0D 0A 50 43 4A 52 20   U..........PCJR

C000:0010  46 69 78 65 64 20 44 69-73 6B 20 42 49 4F 53 20   Fixed Disk BIOS

C000:0020  56 65 72 20 33 2E 31 31-0D 0A 28 43 29 20 43 4F   Ver 3.11..(C) CO

C000:0030  50 59 52 49 47 48 54 20-49 42 4D 20 20 43 4F 52   PYRIGHT IBM  COR

C000:0040  50 2E 2C 31 39 38 32 2C-31 39 38 35 2E 0D 0A 28   P.,1982,1985...(

C000:0050  43 29 20 43 4F 50 59 52-49 47 48 54 20 4A 6F 65   C) COPYRIGHT Joe

C000:0060  6C 20 53 6D 69 74 68 20-31 39 38 36 0D 0A 28 43   l Smith 1986..(C

C000:0070  29 20 43 4F 50 59 52 49-47 48 54 20 52 69 63 68   ) COPYRIGHT Rich

-d

C000:0080  61 72 64 20 54 6F 64 64-20 31 39 38 36 0D 0A 28   ard Todd 1986..(

C000:0090  43 29 20 43 4F 50 59 52-49 47 48 54 20 44 6F 6E   C) COPYRIGHT Don

C000:00A0  20 42 6F 6F 68 65 72 20-31 39 38 38 0D 0A 20 68    Booher 1988.. h

C000:00B0  61 72 64 20 63 6F 64 65-64 20 66 6F 72 20 64 72   ard coded for dr

C000:00C0  69 76 65 20 74 79 70 65-20 30 32 0D 0A 30 32 2F   ive type 02..02/

C000:00D0  31 30 2F 38 38 40 00 2E-8E 1E D5 00 2E 8E 1E D5   10/88@..........

C000:00E0  00 C6 06 40 00 FF B8 03-00 CD 10 B9 C4 00 BE 09   ...@............

C000:00F0  00 B3 00 B4 0E 2E 8A 04-CD 10 46 E2 F6 2B C0 8E   ..........F..+..

-Q

* disable write on simulator

* with the battery backup this is now the same as a EPROM

NOTE: The ROM at C800 on the hard disk adapter must be disabled by removal

of the ROM or disable the read to the chip as the JRBIOS will detect

the ROM signature and initialize the wrong HFBIOS on the adapter.

If you get ERROR during initialization - this is a checksum error at segment

F  C0                         C000 and the code in the simulator

should be reloaded with debug.

DUE TO BEING IN A HURRY... I DID NOT TEST THE BATTERY BACKUP CIRCUIT THEROM00010

WAY I SHOULD HAVE. I USED A TOD CLOCK CIRCUIT AND FOR SOME CHIPS THIS DOROM00020

NOT HAVE ENOUGH DRIVE ON THE CHIP SELECT LINE FOR RELIABLE OPERATION. IFROM00030

YOU ARE HAVING PROBLEMS WHILE RUNNING.. IE:HANGS, WRITE ERRORS, LOOPS.. ROM00040

THEN THE TEMPORARY FIX IS AS FOLLOWS.                                   ROM00050

ROM00060

R1 SHOULD BE CHANGED TO 33K                                             ROM00070

R2 SHOULD BE CHANGED TO 10K                                             ROM00080

ROM00090

IF THE SIMULATOR WILL NOT HOLD THE MEMORY ON POWER DOWN, ADD A 1.5V     ROM00100

AAA OR AA BATTERY IN SERIES WITH THE 3.2V BATTERY. THIS IS A TEMP FIX,  ROM00110

THE REAL FIX WILL BE A COUPLE OF TRANSISTORS ADDED TO THE CIRCUIT       ROM00120

SIMULAR TO THE PCAT CMOS. THE TEMP FIX WILL WORK BUT IS NOT AS NEAT.    ROM00130

ROM00140

SORRY IF THIS HAS CAUSED ANY PROBLEMS...                                ROM00150

DON BOOHER                                                              ROM00160

ROM00170

#