Aug 30, 1994

Enclosed are the following files for Windows NT 3.5 with an

ARTIST Graphics 1000 series cards version 1.5.

art.sys   - ARTIST Graphics miniport driver version 1.5

ART.dll   - ARTIST Graphics video driver version 1.5

DISK1	   - id indicator for which disk to use

TXTSETUP.OEM   - setup installation script file with no vga

OEMSETUP.INF   - setup installation script file with vga

8514SYS.FON   - 120 DPI sized sys font

8514FIX.FON   - 120 DPI sized fix font

8514OEM.FON   - 120 DPI sized oem font

WSSYS150.FON   - 150 DPI sized sys font

WSSYS192.FON   - 192 DPI sized sys font

120DPI.INI   - sets up 120 DPI along with 120 DPI sized fonts - large

96DPI.INI   - sets up 96 DPI along with 96 DPI sized fonts - small

192DPI.INI   - sets up 192 DPI along with 192 DPI sized fonts

150DPI.INI   - sets up 150 DPI along with 150 DPI sized fonts

REGINI.EXE   - program that sets up the specific DPI size

Hardware Requirements:

- An 80x86 based computer with an ARTIST Xj1000, Xj1000i, Xj1000 Lite

Xj1000 GrayScale (EISA, ISA, MC, or Local Bus) installed at I/O

address of 3E0.

-  A graphic display monitor

-  Windows NT 3.5

To install starting with a VGA display:

-  Select Control Panel

-  Select Change Display Type

-  Select Other

-  Put the ARTIST Display Driver disk in a diskette drive

-  Select the resolution and test it

-  Shut down the system and power up again.

To change the system Display Font Size:

-  You can select 96, 120, 150, or 192 DPI system display fonts.

To use a different size font, first copy these five font files

from the ARTIST display drivers disk to the \WINNT35\SYSTEM

directory.

8514SYS.FON

8514FIX.FON

8514OEM.FON

WSSYS150.FON

WSSYS192.FON

-  To install a font and DPI setting.

[d:]REGINI (filename)

where [d:] is the drive where the ARTIST display driver disk

is located.  Replace (filename) with the name of the *.INI file

you want to install:

120DPI.INI

96DPI.INI

192DPI.INI

150DPI.INI

The default font file selected at installation is 96DPI.INI which

VGA uses.  For example, to install 192 DPI fonts from the ARTIST

disk located in the A: drive, enter this command.

A:REGINI 192DPI.INI

Note that larger fonts like 150 DPI and 192 DPI are usually used

with higher resolution drivers (1280x1024 or 1600x1200).  If you

select a larger font and later return to a lower resolution

display, IMPORTANT - remember to install a smaller font.