FM - 3812 Resident Driver

Substitute for "sound.com"

Syntax

fm [-iXX] [-m] [-h] [insfile]

Options

-iXX	- XX= interrupt number to use (hex) (default: 65h)

-m	- install 80x86 machine-specific code

-h	- use high memory for buffer

insfile	- file containing instrument definitions

Note: these options are planned, not implemented

Operation

This program is a TSR that installs into interrupt 65h and handles

the function calls as specified in the 1987 Ad Lib Synthesizer Card

Programmer's Manual.

Run the program fm.com to load the driver.  The program will not load

more than once unless interrupt 65h is intercepted by another program

that does not have a valid signature.

Auxiliary Programs

A program can be written to load the queue and terminate

before turning the driver on.

The "pmode.com" and "mmode.com" programs switch the card into

percussive and melodic modes, respectively.

The programs "qon.com" and "qofff.com" turn on and off the driver.

If you wish to remove the TSR from memory, first run "qoff"

to turn the driver off in order to reset the clock timer to

its normal speed;  then, release the memory.

Disassembly of these programs will show you that they

are simply function calls to interrupt 65h.

Defects

The main defect with the program is that the buffer size is not

large enough.

The program can not handle a very fast tempo;  the "pinkpan1.rol"

song will demonstrate this.

Internal

Interrupt Service Functions - * indicates currently implemented

F#	Definition

0*	Initialize; set volume to 100%, set pitch to 0,

set melodic mode, set timbre for all voices

to piano, empty queues, and set tempo to 90 bpm.

2	Set Relative Time Start;

3	Set State; either 1, active, or 0, stopped.

4	Get State; either 1, playing, or 0, complete or stopped.

5	Empty queues; silence all voices, stay active.

6	Set Mode; set melodic or percussive, set volume to 100%,

set timbre for all voices to piano, set pitch to 0.

7	Get Mode; return 0 for melodic, 1 for percussive.

8	Set Volume at Time.

9	Set Tempo at Time.

10	Set Transpose;  immediately affect all voices by specified

number of semitones.

11	Get Transpose.

12	Set Active Voice.

13	Get Active Voice.

14	Play Note with Delay; play note of pitch for length, set delay.

15	Play Note without Delay; play note of pitch for length.

16	Set Voice Timbre at Time.

17	Set Pitch at Time; affect all voices.

18	Set Ticks per Beat;  all notes should fall on multiple of

1/TickBeat, and obey: 18.2 <= (TickBeat * Tempo / 60).

19*	Direct Note On;  play voice at pitch.

20*	Direct Note Off;  stop voice.

21*	Direct Set Timbre;  set timbre for voice.

Development Notes

01/23/92 - Corrected delta pitch effect.

01/22/92 - Beta version 3.0 released.  Fully functional.  Queue size

too small.

01/09/92 - A defect with the combination of the timer and music interrupt

routines caused the computer to enter an endless loop, with

interrupts disabled: lock-up.  Version 2.1 corrects this.

01/07/92 - Additional functions 8000h, 8001h return pointers to internal

data structures and the count of voices;  8002h and 8003h start

and stop background fm sounds, as with the fmfun.exe program.

mov si, 8002h		; start background sound

int 65h

mov si, 8003h		; stop background sound

int 65h

12/30/91 - Only the Reset Chip and Direct Note On/Off and Set Timbre

functions are implemented.  All other functions simply return 0.

Otherwise, fully resident with proper signature placement and

checking.

Supplied test program fmfun.exe allows manipulation of voice

parameters and pitch;  the tab key causes the program to begin

reading memory at 0:0, determining a pitch and length for each

voice.