STANDARD MICROSYSTEMS CORPORATION

DOS ODI Driver

For EtherCard PLUS Family LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

INSTALLATION PROCEDURE

----------------------

Use the following procedure to enable a workstation to communicate

with both Novell NetWare 386 and IBM AS400 networks concurrently.

1. Create a directory on your hard drive called \NOVELL:

mkdir C:\NOVELL

2. Copy the following files to the \NOVELL subdirectory:

LSL.COM

ODINSUP.COM

IPXODI.COM

NETX.COM

SMC8000.COM

NET.CFG

For example:

copy a:\dos_odi\lsl.com c:\novell

copy a:\dos_odi\ipxodi.com c:\novell

copy a:\netx.com c:\novell

copy a:\dos_odi\smc8000.com c:\novell

copy a:\dos_odi\as400\net.cfg c:\novell

LSL.COM, IPXODI.COM and NETX.COM are provided on the SuperDisk,

ODINSUP.COM may be aquired from Novell Technical Support or from

the Novell section on CompuServe(type GO NOVELL at the CompuServe

prompt).

3. Install the SMC(or IBM) LAN Support Program Version 1.25 or later on your

workstation.

4. Copy the ODI driver, SMC8000.COM, from your SMC SuperDisk to your

workstation. This file is located in the DOS_ODI subdirectory on

the SuperDisk.

copy a:\dos_odi\smc8000.com c:\novell

5. Using a text editor, open the AUTOEXEC.BAT file on your workstation

and edit until it looks like this:

lsl

SMC8000

ipxodi

odinsup

netbind

netx

f:

6. Reboot the workstation.

The workstation displays the F: prompt.

Your workstation can now communicate with both Novell NetWare 386

and AS400 networks.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard

Microsystems Corporation. Other trademarks mentioned herein belong to

their respective companies.