INSTALLATION & USAGE INSTRUCTIONS

S3 86C801/805 DOS and CAD Drivers

Revision:		1.7

Release Date:	April 7, 1993

This document contains the installation instructions and, where applicable,

the usage and performance tips for the following 86C801/805 drivers:

*   CadKey v5.0/DataCad v4.06

*	Cadvance v3.51

*	Generic CADD v6.0

*	Autodesk ADI 4.2 Protected mode:

- AutoCAD 11/12

- 3D Studio 1.0/2.0

- AutoShade 2.1

*	Microstation PC:

- Protected Mode v4.0

- Real Mode v3.3

*	VersaCAD Designer 7.0 & VersaCAD 386 7.0

*	Lotus 1-2-3 2.3

*	Microsoft Word 5.0

*	Word Perfect 5.1

CADKEY 5.0/DATACAD 4.06 DRIVER

The following resolutions are supported:

640 x 480	256 colors	0.5M

800 x 600	16 colors	0.5M

800 x 600	256 colors	1M

1024 x 768	16 colors	0.5M

1024 x 768	256 colors	1M

1280 x 1024	16 colors	1M

This driver is a TSR (Terminate and Stay Resident) program that must be

invoked BEFORE Cadkey (or DataCAD) will run.  The driver will accept

certain command line options that will change the behavior of the driver.

These options will allow you to remove a previous copy of the driver from

memory, or to install the driver at a different interrupt vector.

To remove a previous copy of the driver, use the '-r' option.

Examples:

CK_911 -r	This will remove the driver (if installed) from system

memory.

This driver will install itself on interrupt 60h by default.  To install

the driver at a different interrupt vector, use the '-f##' option.  Where

'##' is replaced by a number between 60 and 67.

Examples:

CK_911		Will install the driver at interrupt 60h.

CK_911 -f62	Will install the driver at interrupt 62h.

Cadkey Driver Installation:

1.	Insert the S3 86C801/805 DOS and CAD Drivers Diskette in drive A:

2.	Change the current drive/directory path to the one which contains the

Cadkey application.

3. 	Copy the Cadkey driver from the diskette into the sub-directory which

contains Cadkey.  For example, if Cadkey is located on drive C: in the

\CADKEY sub-directory, type the following, "COPY A:\CADKEY\CK_911.EXE

C:\CADKEY".

4.	In the Cadkey sub-directory on the hard-disk, create a backup of the

GRDEV.DAT file by rename the file GRDEV.DAT to GRDEV.BAK.

5.	Create a new GRDEV.DAT  by appending the GRDEV.DAT file on the

diskette to the GRDEV.BAK file.  To do this, type the following, "COPY

GRDEV.BAK+A:\CADKEY\GRDEV.DAT GRDEV.DAT"

7.	Run the Cadkey CONFIG program and make the following selections:

A.	Select "Set graphics options".

B.	Select "Select graphics device" option.

C.	Select "S3 86C911 (86C911)".

D.	Select the appropriate video resolution for the monitor

installed.

E.	Select the rest of the graphics options, depending upon user

preferences.

F.	Exit, saving the new configuration.

DataCAD Driver Installation:

1.	Insert the S3 86C801/805 Driver Diskette in drive A:.

2.	Change the current drive/directory to the one which contains the

DataCAD drivers, typically \MTEC\DRV.

Without EMS Memory:

3.	Copy the Cadkey driver from the diskette into the sub-directory that

contains the DataCAD drivers.  For example, if DataCAD is located on

drive C:, type the following "COPY A:\CADKEY\CK_911.EXE C:\MTEC\DRV"

With EMS Memory:

3.	Copy the Cadkey driver from the diskette into the sub-directory that

contains the DataCAD drivers.  For example, if DataCAD is located on

drive C:, type the following "COPY A:\CADKEY\DL_911.EXE C:\MTEC\DRV"

4.	In the MTEC\DRV sub-directory, rename the two drivers files,

DRIVERS.GD2 and DRIVERS.GD4 to DRIVERS2.BAK and DRIVERS4.BAK,

respectively.

5.	Create a new DRIVERS.GD2 and DRIVERS.GD4 by appending the DRIVERS.GD2

and DRIVERS.GD4 files on the diskette to the DRIVERS2.BAK and

DRIVERS4.BAK.  To do this type in the following command lines:

COPY DRIVERS2.BAK+A:\DRIVERS.GD2 DRIVERS.GD2

COPY DRIVERS4.BAK+A:\DRIVERS.GD4 DRIVERS.GD4

6.	Change to the \MTEC sub-directory on the hard disk.

7.	Run DataCAD CONFIG program, selecting the "S3 86C911 (86C911)" as the

display driver.

A.	Select the appropriate video resolution for the monitor

installed.

B.	Select the rest of the graphics options, depending upon user

preferences.

C.	Exit, saving the new configuration.

To Uninstall Cadkey:

1.	Change to the \CADKEY sub-directory.

2.	Enter the following command:

COPY GRDEV.BAK GRDEV.DAT

3.	Delete the file CK_911.EXE.

4.	Run the Config program, selecting a new display driver.

To Uninstall DataCAD:

1.	Change to the \MTEC\DRV directory.

2.	Enter the following commands:

COPY DRIVERS2.BAK DRIVERS.GD2

COPY DRIVERS4.BAK DRIVERS.GD4

3.	Delete the files CK_911.EXE and DL_911.EXE.

4.	Change to the \MTEC directory.

5.	Run the CONFIG program and select a different video driver.

CADVANCE 3.51 DRIVER

The following resolutions are supported:

This driver is loaded into memory by Cadvance, and requires no intervention

on the part of the user.  Cadvance will also remove the driver from memory

upon completion, thus conserving DOS memory.

This driver will work as a rendering device, if sufficient memory is

installed and the driver is using a 256 color mode.  The resolution will be

the same as the resolution used in the 2D and 3D modes.

The driver will default to the highest resolution possible.  If this is not

what you desire, the resolution may be overridden by setting an environment

variable as follows:

SET S31_911=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors (911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

Cadvance Driver Installation:

1.	Insert the S3 86C801/805  Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains the

Cadvance application.

3. 	Copy the Cadvance driver from the diskette into the sub-directory

which contains Cadvance.  For example, if Cadvance is located on drive

C: in the \CAD sub-directory, type the following, "COPY

A:\CADVANCE\CDV911.DRV C:\CAD\GS.DRV"

GENERIC CADD 6.0 DRIVER

The driver is loaded and unloaded in memory by CADD, and has no TSR

(Terminate and Stay Resident) considerations.  If there is no need for the

screen save function in Generic CADD, it is recommended that the smaller

driver be used.  This will free up an additional 11KBytes of drawing space

if no EMS is installed, or around 88KBytes if there is EMS memory.

The driver will default to the highest resolution possible.  If this is not

what you desire, the resolution may be overridden by setting an environment

variable as follows:

SET CADD_INIT=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors (911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolutions may be available.

Generic CADD Driver Installation:

1.	Insert the S3 86C801/805  Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains the

Generic software product (eg. Level 2, Level 3, 3D Drafting, or CADD

6.0)

3.	Copy the driver you wish to use from the \GENCAD sub-directory on the

driver disk to the sub-directory which contains the application.  For

example, type "COPY A:\GENCAD\LGEN911.VGD C:\LEVEL2" ,to copy the

driver with screen save function for use with CADD level 2.

4.	After the desired driver is copied into the appropriate sub-directory,

run the CONFIG, or 3DCONFIG if 3D Drafting is used, to configure the

application.  Configure the Generic product for "S3 86C911" as the

display driver.  If CADD 6.0 is used, first select the "Other VGD's"

option, then select "S3 86C911."

AUTODESK ADI 4.2

The Aquila driver is a professional quality PADI (Protected mode Autodesk

Driver Interface) driver to be used with AutoCAD Release 11/12, 3D Studio

1.0/2.0, and AutoShade 2.1.

Features

*	Protected mode operation, PADI 4.2 specification.

*	32 bit display list for "no regen" operation.

*	Display list operation with 16 active viewports, including pspace

viewports.

*	Birds-Eye and Edge-Pan features for convenient zooms and pans.

*	Partial redraws on overlapping vports.

*	Redraw list for maximum redraw speed.

*	Intelligent clean-up (Erase Buffer* ) to eliminate "flashing".

*	Display list memory meter.

*	AutoCAD memory usage statistics.

*	Paging Alarm* - disables display list usage when paging become

excessive.

*	Reliable dual screen operation.

*	Variable font support- allows fonts up to 16 by 30 in size.

*	International font support.

*	User configurable screen colors.

*	15 and 16 bit "Band-Free* " rendering support.

*	24 bit Rendering support.

*	Text screen emulation window for Release 12.

The following resolutions are supported:

640 x 480	256 colors		0.5M

800 x 600	16 colors 		0.5M

800 x 600	256 colors 		1M

1024 x 768	16 colors		0.5M

1024 x 768	256 colors		1M

1280 x 1024	16 colors 		1M

1280 x 1024	256 colors		2M

640 x 480	32768 colors		1M		(Rendering Mode)

640 x 480	65535 colors		1M		(Rendering Mode)

640x480	16.7M colors		1M		(Rendering Mode)

800X600	65535 colors		1M		(Rendering Mode)

1024x768	65535 colors		2M		(Rendering Mode)

Installation

1.	Create a directory on your hard drive for these files - "MD \AQUILA".

Move to that directory by entering "CD \AQUILA".

2.	Copy all files from the DOS and CAD drivers diskette to this

directory.  Type "COPY A:*.*".

Basic configuration

1.	Add the following 3 lines to your AUTOEXEC.BAT:

"SET DSPADI=C:\AQUILA\RCPATC.EXP"

"SET RCPADI=%DSPADI%"

"SET RDPADI=%DSPADI%"

This will cover all Autodesk applications except AutoCAD Release 12.

Simplified configuration

1.	Aquila will detect the Autodesk application that it is being

configured for, and will provide correct defaults for all the various

applications.  By accepting all the default parameters, your driver

will be configured correctly. This will work with AutoCAD, AutoShade,

or 3D Studio* .

2.	This driver will use the 8 bit international font in the VGA BIOS

(8x16), or an Aquila optional font file if present.  Font files are

available in sizes  up to 16 x 30.

3.	The driver searches for the file "AQUILA.FNT" in the current

directory.  If not found, it searches for the file specified in the

DOS environment variable "AQUILA_FONT."

For example, "SET AQUILA_FONT=C:\AQUILA\9X15.FNT".  If the font file

is readable, it will be used, otherwise Aquila will borrow font data

from the VGA BIOS.

Please note that font files can only be used with AutoCAD and

AutoShade, while 3D Studio may only use the internal 8x16 VGA font.

If Aquila font files are used with 3D Studio, characters such as

arrowheads, may look strange or different.

AutoCAD Release 11 configuration

1.	Set the environment variables as described above under Basic

configuration.

2.	Set the font variable as described above under Simplified

configuration.

Rerun your batch file if needed to initialize the variables.

3.	Start AutoCAD by moving to the "\ACAD" directory and typing "ACAD".

4.	Select option #5 "Configure AutoCAD".

5.	Select option #3 "Configure video display".

6.	Select option #1 "ADI P386 v4.0/4.1 display" which will display the

Aquila configuration menu.

7.	Choose 1 to select a display resolution mode.  If your system is

capable of dual monitor operation, you will be asked if you wish to

use this option.

8.	Choose 2 to start the color menu to customize colors used for the

AutoCAD graphics screen.

9.	The Graphics Flip-screen (selection 3) is available for Release 12

only.

10.	Choose 4 to adjust or disable the Display List options.

11.	Choose 5 to disable or adjust the Erase-buffer size (default is

enabled for "flash-less" redrawing during editing.

12.	Choose 6 to enable or adjust the sensitivity setting for the Page-

Alarm (default is off).

13.	Choose 7 to adjust or enable the Edge-pan option (default is off).

14.	Choose 8 to disable or change the size of the Birds-eye (default is

25% of the total screen size).

15.	Choose 9 to configure the digitizer or mouse button to use for

entering commands.  The transparent Birds-eye command "AQBIRD" is the

recommended double-click command as this will allow you to bring up

the Birds-eye anywhere on the graphics screen while in the middle of

an AutoCAD command.

16.	Exit Video Display Configuration menu.

17.	Configure pointer, printer and plotter.

18.	Return to the main AutoCAD menu, saving the configuration results.

AutoCAD Release 12 configuration

AutoCAD Release 12 uses a much different method of locating device drivers

than Release 11.  Release 12 will search directories for device files, and

ignores the "DSPADI=" environment variable.  If you are installing or

configuring Release 12 for the first time, please pay careful attention to

the following setup instructions.

1.	Set the environment variable in the ACADR12.BAT and AUTOEXEC.BAT files

as follows:  "SET ACADDRV=C:\AQUILA;C:\ACAD\DRV".

2.	Set the font variable as described above under Simplified

configuration.  Rerun your batch files if needed to initialize the

environment variables.

3.	Start AutoCAD by moving to the "\ACAD" directory.  Enter "ACAD" if you

are configuring for the first time or "ACAD -r" if you are

reconfiguring.

4.	Select the Video Display driver menu. From available drivers, select

"S3 ADI v4.2 Display and Rendering by ATC Graphics".

5.	Choose 1 to select the AutoCAD S3 Display resolution mode.  If your

system is capable of dual monitor operation, you will be asked if you

wish to use this option.

6.	Choose 2 to start the color menu to customize colors used for the

AutoCAD graphics screen.

7.	Choose 3 to enable or disable the "Flip-screen" feature.  If enabled,

this will allow the AutoCAD text screen to appear in the center of the

graphics screen, and will eliminate the mode switching delay common to

most multi-frequency monitors.  If you are using an after-market TSR

product that pops up into the screen during AutoCAD sessions

(calculator, plot spooler, etc.) then you should not use this option.

Flip-screen is automatically disabled if you selected dual monitor

operation.

8.	Choose 4 to adjust or disable the Display List options.

9.	Choose 5 to disable or adjust the Erase-buffer size (default is

enabled for "flash-less" redrawing during editing.

10.	Choose 6 to enable or adjust the sensitivity setting for the Page-

Alarm (default is off).

11.	Choose 7 to enable or adjust the Edge-Pan feature.  Later changes to

the Edge-Pan setting can be made within AutoCAD.  (Default is off.)

12.	Choose 8 to disable or change the size of the Birds-Eye (default is

25% of the total screen size).

13.	Choose 9 to configure the digitizer or mouse button to use for double-

click entering of your selected command.  It is recommend that the

transparent Birds-eye command "'AQBIRD" be used, as this will allow

you to bring up the Birds-eye while in the middle of any AutoCAD

command.

14.	Exit the Aquila Display configuration menu.

15.	Configure pointer (mouse or tablet), printer and plotter as needed.

16.	Return to the main menu and save the configuration results.

AutoCAD 12 AVE configuration

AutoCAD's AVE Rendering must also be configured for the Aquila driver.

1.	If you are reconfiguring AutoCAD, delete your current AVE.CFG file.

2.	Once in AutoCAD, type "RCONFIG" at the AutoCAD Command prompt, or

select from the Pulldown menus, RENDER/PREFERENCES/Reconfigure.

3.	When you configure the rendering display driver for AutoCAD Render,

select option 1, AutoCAD's configured P386 combined display/rendering

driver.  (Do not select option 2).

4.	The rendering resolution mode selected will apply only when rendering

to the separate rendering screen.  When rendering to a viewport,

AutoCAD's display driver controls the rendering resolution.

Display List configuration for AutoCAD 11 and 12

This section covers the configuration of the display list features found in

Aquila.  Please pay careful attention to the information presented here,

since this will affect the performance of Aquila under AutoCAD.

1.	During configuring, you must choose a method of memory management for

the display list information.  Which method you use will depend upon

the type and number of drawing you work with during an AutoCAD

session.

Dynamic - Memory keeps on growing as it is needed.  This requires the

least amount of operator interaction, and is the most carefree.

Static - Memory is a fixed amount that you will specify.  When the

display list grows bigger than memory limits, the display list will be

shut off.  The static option is a little bit faster, but requires a

knowledge of how much memory your drawings will consume.

Some experimentation will be necessary in order to make Aquila work

optimally for your particular environment and needs.

2.	The first option is "Enable Display List?".  Answering "No" will end

the display list configuration choices.

3.	If you are configuring for Release 12, you will then be asked "Enable

32 bit regens?".  Enabling this feature will greatly reduce regens,

providing faster operation.  The down side of this choice is that

Aquila will use more memory for the display list.

4.	Select a memory management scheme.  "Dynamic dlist (automatic)?"

Answering "Yes" will select the dynamic scheme.  You would then be

prompted to select a performance level, "Miserly (1), Normal (2) or

High Performance (3)".  If you have very little memory in your machine

then select Miserly, if your have lots of memory, select High

Performance.  All others select Normal.  If you selected Static

management (by answering "No"), then you will be asked to provide a

memory size to use.  If this is the first time being configured,

Aquila will make a suggestion based upon the amount of memory in your

system.

5.	"Enable redraw list?" allows you to enable a faster redraw method.

This will consume some additional memory, but it does improve redraw

speeds.  If you selected "Static" memory management earlier, then you

will be asked to select a memory size for the redraw list.  Usually

this can be 1/2 to 1/4 of the amount you specified for display list

usage.

6.	Select erase buffer operation.  Answering Yes to "Enable erase

buffer?" brings up a second question "Erase buffer size?".  This will

determine the number of erased objects it takes before Aquila will

start an automatic "cleaning" cycle to remove them from the display

list.  Select a higher value if you plan on doing much large drawing

editing.

7.	Configure the Page-Alarm option, "Enable Page-Alarm?".  Answering

"Yes" will bring up a second prompt about the sensitivity setting for

the Page-Alarm.

AutoShade 2.1 Configuration

1.	Set the environment and font variables as described above under Basic

and Simplified configuration.  Rerun your batch file if needed to

initialize the variables.

2.	Start up AutoShade 2.1, with the reconfiguration command "SHADE /R".

3.	Select the correct pointing device.

4.	Select option #2 "P386 Autodesk Device Interface display driver".

5.	Choose a display resolution mode for the interactive screen.

6.	Choose the colors to use for Menus, Pop-ups, Background, etc.

7.	Save and exit the colors menu.

8.	Select option #2 "P386 Autodesk Device Interface rendering driver".

9.	Choose a resolution mode for the rendering (shading and RenderMan)

screen.

10.	If you select the 32768 or 65535 color modes, you will be asked if you

wish to enable "Band-Free" mode which allows the best color quality

possible in a 15 or 16 bit device. Select Low, Medium or High.

11.	Answer the next 3 questions with a "YES", since only one screen is

used, and must be redrawn upon switching.

12.	Select a hard copy device option.

13.	Configuration is complete and the interactive screen is displayed.

3D Studio 1.0 Configuration

Verify that the environment variables listed under Basic configuration,

above, are initialized before proceeding with your 3D Studio configuration.

1.	Go to your 3D Studio 1.0 subdirectory, e.g.,"\3DS" directory, and edit

the file "3DS.SET" file.

2.	Change the line that starts with "DEFAULT-DISPLAY = VGA320X200" to

read "DEFAULT-DISPLAY = RCPADI".

3.	Change the line that begins with "MAIN-DISPLAY =" to read "MAIN-

DISPLAY = RCPADI".  Delete any leading semicolon (;) that may be

present.

4.	Change the line that begins with "MATERIAL-DISPLAY = VGA320X200" to

read "MATERIAL-DISPLAY = RCPADI".

5.	Save the file and exit the editor.

6.	Delete the file "3DADI.CFG", if present.

7.	Start 3D Studio with the command "3DS".

8.	Select a 256-color resolution  for the Main display.  This is the mode

that will be used for the 2D Shaper, 3D Lofter, 3D Editor, and

Keyframer.

9.	Select a 256-color resolution for the Materials Editor.  Use of a

resolution greater than 640 x 480 will not increase the usefulness of

the Materials Editor at all.

10.	You should now find yourself inside the 3D Editor of 3D Studio.

11.	The first time that you try to render a drawing or view an image, you

will be asked to select a resolution for Rendering.  If you select the

32768 or 65535 color modes, you will be asked if you wish to enable

"Band-Free" mode.  This allows the best color quality possible in a 15

or 16 bit device. Select Low, Medium or High.  Configuration is now

complete.

3D Studio 2.0 Configuration

Verify that the environment variables listed under Basic configuration,

above, are initialized before proceeding with your 3D Studio configuration.

1.	Go to your 3D Studio 2.0 subdirectory, e.g., the "\3DS" directory, and

edit the file "3DS.SET" file.

2.	Change the line that starts with "DEFAULT-DISPLAY = VGA320X200" to

read "DEFAULT-DISPLAY = RCPADI".

3.	Change the line that begins with "MAIN-DISPLAY =" to read "MAIN-

DISPLAY = RCPADI".  Delete any leading semicolon (;) that may be

present.

4.	Change the line that begins with "MATERIAL-DISPLAY = VGA320X200" to

read "MATERIAL-DISPLAY = RCPADI".

5.	Save the file and exit the editor.

6.	Delete the file "3DADI.CFG", if present.

7.	Start 3D Studio with the command "3DS".  Answer the question about

having a VGA or SVGA.

8.	Select a 256-color resolution mode for the "ADI main display".  This

is the mode that will be used for the 2D Shaper, 3D Lofter, 3D Editor,

and Keyframer.  Select a 256-color resolution for the "ADI MEDIT",

Materials Editor.  Use of a resolution greater than 640 x 480 will not

increase the usefulness of the Materials Editor at all.

9.	Answer the question about "Will RDPADI render to a separate screen?".

The answer is always "no".

10.	Select a resolution mode for the Rendering "RDPADI" Device.  If you

select the 32768 or 65535 color modes, you will be asked if you wish

to enable "Band-Free" mode.  This allows the best color quality

possible in a 15 or 16 bit device. Select Low, Medium or High.

11.	Answer the question about "Will RCPADI render to a separate screen?".

If using Aquila for an 8514/A, the answer is "yes", for all others,

the answer is "no".

12.	Select a resolution mode for the Rendering "RCPADI" Device.  If you

select the 32768 or 65535 color modes, you will be asked if you wish

to enable "Band-Free" mode.  This allows the best color quality

possible in a 15 or 16 bit device. Select Low, Medium or High.

13.	Configuration is now complete.

Special Features

AutoCAD Screen layout

1.	Current Color Display - The rectangle in the top left of the screen

displays the current drawing color.

2.	Logo - This is used to display the manufacturers logo near the lower

right corner of the screen.

3.	Display Status Area - This area is located below the Logo Area and is

present only when the display list has been enabled.

Memory Status - This provides a percentage of total system memory that

is being used by AutoCAD.  AutoCAD may start to slow down when this

display exceeds 100%.  After this figure exceeds 150%, "paging" will

occur.

Display/Screen Lists Memory Status - Amount of memory being used by

the display list and screen list.  When this line reads "Empty", this

means that there is no drawing in memory (new drawing).

Display List Status abbreviations -

*	D	32 bit display list data is in use.

*	d	16 bit display list data has been selected.

*	Z	Zooming from the display list is allowed.

*	z	Pspace and perspective zooms by AutoCAD.  Birds-eye will not work

in viewport.

*	S	Redraw list is enabled.

*	s	Redraw list is turned off.

*	P	Page alarm enabled and paging occurred recently.

*	p	Page alarm enabled and no paging has occurred.

*	E	Edge-pan enabled.  Edge pan is impending.

*	e	Edge-pan enabled but none impending.

Graphics Flip-screen -

When enabled the graphics Flip-screen will emulate the VGA text window.

Use the F1 key to toggle between modes.  If you wish to execute any DOS

commands, Aquila will switch the screen to a full-screen text mode allowing

for full DOS compatibility.  If you are using a plot spooler or other TSR

that requires a compatible "pop-up" window, then this option must be

disabled.

Birds-eye View -

The Birds-eye feature allows fast transparent zooms and pans.  The Birds-

eye can be activated by typing "'AQBIRD" or by double-clicking on the mouse

or digitizer button specified during configuration.

When activated, the Birds-eye window will appear under the current cursor

location.  The window will show a small overall view of the drawing that

AutoCAD has currently placed in Aquila's display list.  Within the Birds-

eye window a highlighted rectangle indicates the currently viewed area, and

an outline rectangle indicates the area that will be shown if a Birds-eye

zoom is completed.  If this rectangle contains an "X" in the middle, then

the Birds-eye is in pan mode.  If this rectangle contains an arrow, then

the Birds-eye is in zoom mode.

Pressing the pick button on your mouse or digitizer (usually the left

button for a mouse) allows you to toggle between zoom and pan modes in the

Birds-eye window.  Pressing the selection button on your digitizer causes

the Birds-eye to disappear and the selected view to be used in the

currently active viewport.  A new zoom will be executed if necessary.

Moving the cursor outside the Birds-eye window without making a selection

will cancel the function.  This Birds-eye will work in multiple viewport

drawings, and will appear in the aspect ratio of the currently active

viewport.

Edge Pan -

Edge-pan is used to move around while zoomed into a drawing.  By leaving

the cursor near the border of the currently active viewport, Aquila will

automatically begin to pan in that direction.  This only begins if the

cursor is left idle for a predetermined length of time.  The default is

Edge-pan disabled.  Edge-pan settings may be adjusted either in the

configuration menu or from within AutoCAD.  Experimentation will yield the

best results for your needs.

Paging Alarm -

When AutoCAD's memory requirements exceed the amount of physical memory

available in your machine, it begins "paging" in order to gain extra

memory.  Paging means that certain memory areas are traded back and forth

with a "swap file" on the hard drive.  Due to the difference in access

speed between main memory, and this "virtual" memory that is on the hard

drive, AutoCAD begins to slow down, often to the point of being unusable.

To combat this, memory is used very sparingly, and only when absolutely

necessary.  As a second line of defense against paging, when the amount of

memory paged out to disk exceeds several preset thresholds, Aquila begins

shutting down extra functions, and returns the memory to AutoCAD.

When paging activity exceeds the first threshold, the screen-list feature

is disabled and all associated memory is returned to AutoCAD.  At this

point, the screen list status line will disappear from the status window.

Should paging activity continue, Aquila will disable the display list

feature and return all of that memory to AutoCAD.  When that has been done,

the second line of the status window will say "Disabled", to indicate that

all display list functionality has been terminated.

The driver will continue to function as normal, but redraws will be slower.

However, AutoCAD itself will now be much faster than it was when the paging

was occurring.  This allows you to complete your drawing session in much

less time.

Erase buffer -

When AutoCAD erases objects from the drawing database, it erases them from

the screen as well.  Sometimes this action causes other objects to appear

erased as well.  The "redraw" command has no effect, but a "regen" causes

the objects to appear correctly.

This problem can be solved by using the erase buffer to correct the display

corruption that occurs when objects are erased from the drawing.  Normally,

AutoCAD will request that the driver erase a line from the screen by

drawing it in black.  This causes the erased object to flash on and off the

screen during redraws.

Another side effect is that objects near the erased one may appear to be

incomplete.  To correct this in the fastest manner possible, Aquila will

keep a list of erased vectors, and erase them only when the buffer fills.

This speeds up block erasures considerably.

There are certain after market software packages for AutoCAD, such as CAD

Overlay, which rely upon the ability to draw objects in black (background

color).  If you are using one of these packages and the screen images

appear distorted, then you need to disable the erase buffer.

Band-Free Rendering -

When using the 15 or 16 bit rendering modes, often times the images appear

"banded".  This banding or staircasing occurs most often on objects with

large sloping or gently curving surfaces.  Band-Free rendering improves the

image quality in these modes.

The "Band-Free" effect can be controlled as well.  It may be set to "Low",

"Medium", or "High", depending upon how much of the banding you want to

eliminate.  This feature will be of limited use to users of AutoShade,  but

is beneficial to users of Autodesk RenderMan and 3D Studio.

Aquila  commands for AutoCAD

*	AQHLP	The help screen function displays a list of all the driver

local commands, and gives a brief description of each.

*	AQDLQ	Displays on a text-or flip-screen all the viewports currently

in use, along with some useful statistics about each one.

*	AQDRQ 	Displays current display resolution and memory allocation

statistics (CTRL-L provides the same function).

*	AQCLN	Removes objects from display list.

*	AQBIRD	Displays Birds-eye window underneath current cursor location.

*	AQEPAN	Toggles on and off the Edge-pan feature.

*	AQPL	Pan to the left by 25 percent.

*	AQPR	Pan to the right by 25 percent.

*	AQPU	Pan up by 25 percent.

*	AQPD	Pan down by 25 percent.

*	AQZI	Zoom in by 50 percent.

*	AQZO	Zoom out by 50 percent.

*	AQZWI	"Zoom way in" while using the same scale factor.  Zooms in as

much as possible before another regen will occur.  (AutoCAD

12 only)

*	AQZWO	"Zoom way out".  Zooms out as far as possible before another

regen occurs.  This command is useful when making large

additions to the drawing.  (AutoCAD 12 only)

*	AQBESZ=xx	Resize the Birds-eye window.  Valid number is between 10 and

50, and it is a percentage of the graphics screen size.

*	AQPANSCR=xx	Amount of the viewport scrolled by the edge-pan feature.

Valid number must be between 10 and 80, and it is a

percentage of the viewport size.

*	AQPANBRD=xx	Width of the edge-pan border given in number of pixels.

Valid number must be between 5 and 50.

*	AQPANTM1=xx	Amount of initial delay given in number of "clicks" or

fractions of a second used before the edge-pan begins.  Valid

range is 10 to 64.

*	AQPANTM2=xx	Amount of time before another edge-pan begins.  Valid range

is 1 to 18.

*	AQEBUF=xx	Size of the erase buffer.  Using 0 turns off the buffer.

MICROSTATION PC DRIVER (PROTECTED MODE 4.0)

The first time this driver is used, it will ask the user which video mode

to use for MicroStation.  It will present the user with a menu of all

available resolution modes.  One must be selected, and then a configuration

file will be created that the driver will use for subsequent MicroStation

sessions.

To re-configure the driver, erase the file \USTATION\DRIVERS\MGL911.CFG and

run MicroStation again.

To temporarily change the resolution mode used, you may set the environment

variable MGL911 to one of the following numbers:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 1024, 16 colors

6	1280 x 1024, 256 colors

If the driver finds a legal value in the environment, it will use it to

override any variable found in the configuration file.

MicroStation PC Driver Installation:

1.	Insert the S3 86C801/805  Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains the

MicroStation Drivers.

3.	Copy the following files to the \USTATION\DRIVERS directory:

MGL911.EXP		The executable driver file.

MGL911.MA		The MDL application that loads the driver.

MGL911.MGL	Used by the configuration program to locate the driver

files.

4.	Change the current directory to the \USTATION sub-directory, or the

one which contains the MicroStation application.

5.	Run the USCONFIG program to select the new device driver.

A.	Select "Display Adapters" entry.

B.	Select "VENDOR SUPPLIED DRIVER" option.

C.	Select "PMGL 86C911 DRIVER" option.

D.	Select second adapter options as needed.

E.	Select "Exit and Save".

F.	Reboot system for installation to take affect.

This driver will work with a Hercules-compatible MDA card as the second

graphics device.  This can be configured as either the "Primary" (right

screen) or "Secondary" (left side) graphics device.

SWAP screen capabilities are supported if a 256 color mode is configured.

If the initialization code is unable to determine which mode to use, it

will always default to the 640 x 480 x 256 color mode.  The driver will

exit if it is not able to find an S3-based graphics card.

MICROSTATION PC DRIVER (REAL MODE 3.3)

The driver is a TSR (Terminate and Stay Resident) driver that must be

invoked before MicroStation PC will run.  The driver will accept certain

command line options that will change the behavior of the driver.

If your 86C911 has at least 1 megabyte of VRAM, this driver will support

either 2 pages ("swap screen" capability) or will support 256 colors.  To

select the 256 color option, use the "/noswap" option.

MicroStation 4.0 will run faster (redraws 50% faster) if you have a VCPI

memory manager (QEMM or 386MAX) present and correctly configured.  To

optimize your VCPI memory for use with 4.0, all memory outside the DOS 640K

range should be configured as EMS memory.

The driver will default to the highest resolution possible.  If this is not

what you desire, the resolution may be overridden by setting an environment

variable as follows:

SET S31_911=<MODE>

Where <MODE> is:

0	640 x 480, 256 colors

1	800 x 600, 16 colors

2	800 x 600, 256 colors

3	1024 x 768, 16 colors

4	1024 x 768, 256 colors

5	1280 x 960, 16 colors	(911/924 only)

6	--> reserved <--

7	1280 x 1024, 16 colors

8	--> reserved <--

Please note that not all resolution modes may be available.

MicroStation PC Driver Installation:

1.	Insert the S3 86C801/805 Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains the

MicroStation Drivers.

3.	Copy the following files into the MicroStation Drivers subdirectory

from the \USTATION sub-directory on the floppy:

MGLS3.EXE

MGLS3.MGL

4.	Change to the \USTATION sub-directory.

5.	Copy the REMOVE utility from the \USTATION sub-directory on the floppy

into the \USTATION sub-directory on the hard disk.

6.	Run  USCONFIG, selecting the "ATC Graphics S3 86C911" driver.

7.	Save the new settings and exit USCONFIG.

8.	Before using MicroStation, execute the TSR driver first:

9.	To reconfigure the driver, remove the previous driver by using the

command "Remove".  Enter a new MGLS3 in the Drivers directory.

10.	Set the mode number by entering the command "Set S31_911 = <mode>".

11.	Run MicroStation again.

12.	Another way to reconfigure the driver is to reboot the system.

Examples:

MGLS3	Will install the driver in 16 color mode. If enough VRAM is

installed, swap screen will be enabled.

MGLS3 /noswap	Will install the driver, telling it to display the maximum

number

of colors possible

VERSACAD 7.0 DRIVER

This driver is a TSR type (Terminate and Stay Resident) program.  You only

need to install it once in memory each time your machine is powered up.

For this reason, it is recommended that a call to this driver be placed in

your AUTOEXEC.BAT file.  The drivers will support VersaCAD Designer 7.0 and

VersaCAD 386 7.0.  If you are using the real-mode application, the driver

used is VC2_911.COM.  If you are using VersaCAD 386, the driver used is

VC3_911.COM.

The following resolutions are supported:

640 x 480	256 colors	0.5M

800 x 600	16 colors	0.5M

800 x 600	256 colors	1M

1024 x 768	16 colors	0.5M

1024 x 768	256 colors	1M

1280 x 1024	16 colors	1M

VersaCAD/386 will run faster (redraws 50% faster) if you have a VCPI memory

manager (QEMM or 386MAX) present and correctly configured.  To optimize

your VCPI memory for use with VCAD/386, all memory outside the DOS 640K

range should be configured as EMS memory.  This means that all the memory

that would normally be allocated to "DOS high memory" should be reserved

for use as EMS.

EMS Memory Usage

As a default, this driver will reserve 512K of EMS memory when invoked.

This will be used to store the display lists needed for fast redraws.

While this should be enough memory to handle most drawings, you can tell

the driver to reserve more EMS space when the driver is loaded.  This is

done by setting the environment variable VCAD_EMS= to the number of EMS

pages to be reserved.  Each EMS page is 16K in size.

Examples:

SET VCAD_EMS=64	Will cause the driver to reserve 1024K (64 pages * 16K)

of memory when the driver is loaded.

SET VCAD_EMS=0	Will tell the driver to reserve no pages of EMS for

use.  This special case tells the driver not to reserve

any EMS.  In this case, display list processing will be

disabled.

Display Resolution

This driver will use the resolution selected by the ENVIRO program which

ships with VersaCAD.  If that mode is unavailable, VersaCAD will not run.

Display List

The main feature of this driver is the built-in display list capability.

To take advantage of this feature, VersaCAD must be told that the driver

can support display listing, and that you wish to use it.  To do this, go

into the screen options in ENVIRO, and select the Built-In option under

Display List.

To make the most of the display list, use the display list options menu

inside VersaCAD by pressing Ctrl-F5.

To minimize the number of "regens", (where VersaCAD sends down each

individual line) you should zoom the display all the way in, then back out

to where you were in the drawing.  After doing this, VCAD will redraw

everything from the local display list, (sketches) and zooming and panning

(with the arrow keys) will be much faster.  This is done by using the

"Full" function from the displist list (LIST) menu, then using the "Out"

function from the same menu.

To speed up the panning operations, (scrolling throughout the drawing can

be done with the arrow keys) you should use a keyboard utility to increase

the "Typematic" rate of your keyboard.  This will increase the rate at

which the scrolling occurs.

Driver Installation for VersaCAD 7.0 Real mode:

1.	Insert the S3 driver diskette in drive A:

2.	Change the current drive/directory to the one which contains the

VersaCAD application.

3.	Copy and execute file VC2_911.COM from the \VERSACAD sub-directory on

the diskette into the current drive/directory.

4.	Also, copy the file VCAD386.CFG into the current drive/directory.

5.	Run ENVIRO program, and select the "S3 86C911" driver.  Reboot the

system if the installed driver is replacing a previously run driver.

Driver Installation for VersaCAD 386 7.0:

1.	Insert the S3 86C801/805  Driver Diskette in drive A:

2.	Change the current drive/directory to the one which contains the

VersaCAD application.

3.	Copy and execute the file VC3_911.COM from the \VERSACAD sub-directory

on the diskette into the current drive/directory.

4.	Copy the file VCAD386.CFG from the \VERSACAD sub-directory on the

diskette into the current drive/directory.

5.	Run ENVIRO program, selecting the "S3 96C911" screen driver.  You will

be able to choose between "Single Screen" and "Dual Screen"

configurations, along with the various resolutions available.  Reboot

the system if the installed driver is replacing a previously run

driver.

LOTUS 1-2-3 2.3 DRIVER

Lotus 1-2-3 2.3 Driver Installation:

1.	Insert the S3 86C801/805 Driver Diskette in drive A:, and change the

current drive/directory path to the one which contains Lotus 1-2-3.

2.	Copy the following files from the \LOTUS sub-directory on the diskette

to the directory which contains Lotus 1-2-3:

123txt25.drv

123txt43.drv

123d768.drv

123s768.drv

123d600.drv

123s600.drv

4.	Run the Lotus 1-2-3 install program.

A.	You will be presented with several menu choices.  Follow the

directions on the screen and perform the following steps:

-	Press <Enter> to begin.

-	Choose the "Advanced Options" item.

-	Choose the "Add New Drivers to the Library" item.

-	Press <Enter>  twice to continue and to return to the menu.

-	Choose the "Modify Current Driver Set" item.

B.	To change an item, highlight it and you will be given a list of

choices for that entry.  Six new drivers have been added to

support the 86C911, four graphics drivers and two text drivers.

There are two graphics drivers (dual and single screen) for each

resolution supported by the 86C911 (1024x768, and 800x600).

Select "Graphics Driver" and choose either dual or single screen

86C911 for the desired resolution.

If single screen is chosen, select "Text Driver" and choose from

the 86C911 text drivers.  If dual screen is selected, the text

must be based on the desired mono (i.e. IBM/Hercules) and not the

S3 drivers.

When the configuration is complete, press <ESC> .

C.	Exit the installation program by doing the following:

-	Choose the "Save Changes" item.

-	Press <Enter> to accept the current set name.

-	Press <Enter> again to return to the menu.

-	Choose the "Yes" item to leave the install program.

MICROSOFT WORD 5.0 DRIVER

Microsoft Word 5.0 Driver Installation:

1.	Insert the S3 86C801/805  Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains

Microsoft Word.

3.	Save a copy of SCREEN.VID by renaming it.  Copy the SCREEN.VID file

from the \MSWORD sub-directory on the diskette to the directory which

contains Microsoft Word.

4.	Run Microsoft Word.

5.	Word uses several different display resolutions and character sizes to

offer a wide variety of visible characters.  Several new modes are now

available for use, and the user may choose from a list by pressing the

ESC key, the O key, then moving to the DISPLAY MODE entry with the

arrow keys.  Press the F1 key to display all of the available

resolution choices.  If necessary, press the Down key to display more

resolutions.  The new graphics modes are the entries for 37, 38, 42,

48, 54, 75 and 96 lines.  The new text modes are the 25 and 43 lines

(132 columns) .  Select the resolution by moving the highlight and

press <Enter>.

6.	After display option is selected, Word will use the new resolution.

WORDPERFECT 5.1 DRIVER

Word Perfect v5.1 Driver Installation:

1.	Insert the S3 86C801/805 Driver Diskette in drive A:

2.	Change the current drive/directory path to the one which contains Word

Perfect v5.1.

3.	Copy the following files from the \WORDPERF sub-directory on the

floppy disk to the directory on the hard disk where Word Perfect is

installed:

WPGRAPH.VRS

WPTEXT.VRS

4.	Run Word Perfect .

5.	Word Perfect allows you to select from a list of graphics and/or text

drivers from the setup screen.  Press <shift><F1> to go to the setup

screen, choose Display Option, then choose either Text or Graphics

Display.  You will be presented with several adapters to choose from,

including S3 86C911 (Carrera) Text and Graphics entries.  Select

Graphics, and choices for 1024x768 and 800x600 will be presented.

Selecting Text will give you 132x25 and 132x43.  Choose the

appropriate resolution for Text and/or Graphics and continue to use

Word Perfect as before.

Trademark Acknowledgment

Copyright (C) 1992 S3 Incorporated.

Autodesk, AutoCAD, and AutoShade are registered trademarks of Autodesk,

Inc.  Autodesk 3D Studio, Autodesk RenderMan, and Autodesk Device Interface

(ADI) are trademarks of Autodesk, Inc.

RenderMan is a registered trademark of Pixar used by Autodesk, Inc. under

license from Pixar.

ATC Graphics, Aquila, Paging Alarm, and Band-Free are trademarks of ATC

Graphics Company.

21

doscad8.doc		80XDEP-13I2