SOTA Technology, Incorporated

Sota 386si Utility Programs

Version 2.2

There are 8 files in this diskette, including this README.DOC.

The other 7 files are:

386SI.SYS    -  Cache driver

386SI.COM    -  Command for controlling the cache driver

SOTADISK.SYS -  RAM disk driver

SOTACACH.COM -  Disk caching utility

SOTASPL.COM  -  Print spooling program

SCANCODE.COM -  Change the speed switching toggle key

README.BAT   -  A batch file which types this file

README.DOC   -  This file

To use the CACHE DRIVER, the config.sys should contain:

DEVICE=path\386SI.SYS *

* FOR ZENITH systems, please use DEVICE=386SI.SYS /NOTEST /NODMA

* For PMI Fast Card, please use DEVICE=386SI.SYS /NOEMS

Note:  Once the cache is turned on by the caching driver, it can be turned

off by pressing Ctrl & Alt and the keypad 5 Key.  Actually, this key

toggles the cache and can also be used to turn it back on.  When it

turns the cache on, it beeps with a high pitch.  A low pitch

indicates that the cache is off.

DEVICE=path\386SI.SYS /PRINT=ON *

* The /PRINT=ON option is for systems that experience problems with their

printer (no response from parallel port, etc.)

To use the RAM DISK, the CONFIG.SYS file should contain:

DEVICE=path\SOTADISK.SYS {options}

OPTIONS:  To alter the configuration of the SOTADISK, you may select

the parameters it uses by adding the following options to the

CONFIG.SYS:

/EMS=nnn  ---Set the amount of LIM Expanded Memory to use.

/DOS=nnn  ---Set the amount of DOS memory to use.

NOTE:  nnn specifies the amount of memory in kilobytes and will

rounded up to the nearest 16K block.

You should run SOTASPL.COM from the DOS command line or add it

to your AUTOEXEC.BAT file.  To get a short explanation of its

options, type:

SOTASPL /?

You should also run SOTACACH.COM from the DOS command line or

add it to your AUTOEXEC.BAT file.  For an explanation of its

options, type:

SOTACACH {options}

OPTIONS:  To alter the configuration of the SOTACACH, you may

select the parameters it uses by adding the following options

to the AUTOEXEC.BAT or just by typing them in when you enter

the command:

/EMM=nnn    ---Set the amount of expanded memory to allocate.

/DOS=nnn    ---Set the amount of dos memory to allocate.

/HARD=n n n n ON   ---

Activate caching on the specified harddisk.  The disk is

specified as a physical number, ie:1 is the first harddisk, 2

is the second harddisk.  If no disk is specified then all

harddisks are affected.

/HARD= n n n n OFF  ---Deactivate caching on the speciefied

harddisk.	If no disk is specified then all harddisks

are affected.

/FLOPPY=n n n n ON  ---

Activate caching on the specified floppy drive.  The drive

is specified as a physical number, ie:1 is the first floppy

drive, 2 is the second floppy drive.  If no drive is specified

then all floppy drives are affected.

/FLOPPY=n n n n OFF  ---Deactivate caching on the specified

floppy drive.  If no drive is specified then all floppy

drives are affected.

/FLUSH=n n n n ---Flush data from cache for the specified

harddisk. If no disk is specified then all harddisk are

affected.

NOTE:  nnn specifies the amount of memory in kilo bytes rounded

to 16K byte blocks.  The memory options can only be set upon

initial installation.  After SOTACACH is installed these options

will be ignored.

You should run 386SI.COM only if 386SI.SYS is installed in

the CONFIG.SYS. To get a short explanation of its options, type:

386SI /?

To include any options, type 386SI {options} .	By typing 386SI

again, it will show the current status of what is being cached.