%OP%DESystem Calls Reference

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,5,74%OS_MAL, allocate memory

RST 20H, DEFB $54

IN:

OUT, if call successful:

OUT, if call failed:

Registers changed after return:

Notes:

Bugs:

Related calls:

OS_BIX, Bind in extended address

OS_BOX, restore bindings after OS_BIX

OS_FC, select fast code (fast bank switching)

OS_MCL, close memory (free memory pool)

OS_MFR, free (previously allocated block) memory

OS_MGB, get current bank binding in segment

OS_MOP, open memory (allocate memory pool)

OS_MPB, set new bank binding in segment

%P0%

%CO:B,5,69%

A = 0, always

BC = requested size of reserved memory in bytes (2 to 256)

IX = memory handle (returned from OS_MOP)

Fc = 0

HL = address of memory allocated (H14 & H15 are memory are memory

mask)

B = bank number of allocated memory

C = segment specifier implied by HL (H14 & H15)

Fc = 1

A = error code:

....DE../IXIY same

AFBC..HL/.... different

Remember "No Room" means the pool is exhausted, and not the whole of

memory. If a subsequent OS_MOP cannot allocate a pool, then memory

is definitively exhausted.

The call WILL NOT automatically bind in the allocated memory into

the Z80 address space.

%CO:C,5,64%

RC_HAND ($08), IX is not a valid memory handle

RC_ROOM ($07), no room to allocate block in MEMORY POOL

RC_ERR ($0F), if BC > 256 bytes, fatal error

%CO:D,5,59%%CO:E,5,54%%CO:F,5,49%